3GPP TSG-SA WG6 Meeting #57	S6-232980

Xiamen, China, 9th – 13th October 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0387 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update clause 10.6.2.9.5.1 and 10.6.2.9.5.2. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2023-09-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently, the MCPTT utilizes the preconfigured regroup procedures in 23.379 while the MCVideo and MCData utilize the preconfigured regroup procedures in 23.280. However, the requirements of the preconfigured regroup for all the three services are identical. Now the two sets of procedures are not fully aligned and make it difficult to maintain the consistency between the 23.280 and 23.379. In order to resolve the issue, keeping a single preconfigured regroup procedures in 23.280 for all the services is a good choice. So the clause 10.6.2.9.5.1 and 10.6.2.9.5.2 should be updated to refer to the preconfigured regroup procedures in 23.280. |  |  |  |  |  |  |  |  |
| Summary of change: |  | update the 10.6.2.9.5.1 and 10.6.2.9.5.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency will be caused between the two preconfigured regroups in two individual specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6.2.9.5, 10.6.2.9.5.1, 10.6.2.9.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.280 CR 0441 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /************************ First change ******************************/

10.6.2.9a	Group call handling withpreconfigured regroup update procedures

10.6.2.9a.1	MCPTT client PTTs on MCPTT group during an in-progress preconfigured group regroup

Figure 10.6.2.9a.1-1 illustrates the procedure when a user attempts to setup an MCPTT group call on a group involved in an in-progress preconfigured MCPTT group regroup.

Pre-conditions:

-	The MCPTT client is an affiliated member of MCPTT group A that is part of an in-progress preconfigured group regroup with MCPTT groups B and C.

-	MCPTT group D is being used as the preconfigured regroup group.

-	The MCPTT client has missed the preconfigured regroup request message (e.g. poor signalling conditions, race condition).

MCPTT client

MCPTT server

2.Check group regroup status

5. Preconfigured regroup update procedures as described in TS 23.280

1.Group call request

3.Group call response

4. Notify user

Figure 10.6.2.9a.1-1: Procedure for MCPTT client PTTs on MCPTT group during an in-progress preconfigured group regroup

1.	The MCPTT client attempts to start a call on MCPTT group A. The MCPTT client sends a group call request message to the MCPTT server containing MCPTT group A as the target group.

2.	The MCPTT server checks to see whether MCPTT group A is currently part of a preconfigured group regroup. In this case the group is part of an active preconfigured group regroup.

3.	The MCPTT server sends a group call response to the MCPTT client indicating that the call setup is denied because the group is part of an in-progress group regroup.

4.	The MCPTT client notifies the user of the group call setup failure.

5.	The MCPTT server initiates the preconfigured regroup update procedures as described in clause 10.15.3.4 of 3GPP TS 23.280 [16]..

NOTE 1:	If there is a call currently in progress on the regroup group then this MCPTT client can be added to the call using the late entry procedure. If there is no call currently in progress, then the MCPTT user can retry the group call setup.

10.6.2.9a.2	MCPTT client PTTs on preconfigured regroup group after group regroup has been cancelled

Figure 10.6.2.9a.2-1 illustrates the procedure when a user attempts to setup a MCPTT group call on a preconfigured regroup group after the preconfigured MCPTT group regroup has been cancelled.

Pre-conditions:

-	The MCPTT client is a member of MCPTT group A that was part of an in-progress preconfigured group regroup with MCPTT groups B and C that has been cancelled. MCPTT group D was used as the MCPTT regroup group.

-	The MCPTT client has missed the preconfigured regroup cancel request message (e.g. poor signalling conditions, race condition).

MCPTT client

MCPTT server

2.Check group regroup status

4. Notify

user

1.Group call request

3.Group call response

5. Precofigured regroup update procedures as descibed in TS 23.380

Figure 10.6.2.9a.2-1: Procedure for MCPTT client PTTs on regroup group after the group regroup is cancelled

1.	The MCPTT client attempts to start a call on MCPTT group D, the MCPTT regroup group. The MCPTT client sends a group call request message to the MCPTT server containing MCPTT group D as the target group.

2.	The MCPTT server checks to see whether MCPTT group D is currently being used as part of a preconfigured group regroup. In this case the preconfigured group regroup is no longer active.

NOTE 1: The regroup group D can be a group in the MCPTT client's profile, and the MCPTT client can be a member of group D.

3.	The MCPTT server sends a group call response to the MCPTT client indicating that the call setup is denied because the group regroup is no longer active.

4.	The MCPTT client notifies the user of the group call setup failure。

5.	initiates the preconfigured regroup update procedures as described in clause 10.15.3.4 of 3GPP TS 23.280 [16].

NOTE 2:	If there is a call currently in progress on MCPTT group A that was previously part of the group regroup then this MCPTT client can be added to the call using the late entry procedure. If there is no call currently in progress then the MCPTT user can retry the group call setup on MCPTT group A.