3GPP TSG-SA WG6 Meeting #57	S6-233103

Xiamen, China 9th – 13rd October 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0025 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Complete information flow and API for transmission quality measurement procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD |  |  |  |  |  | Date: |  |  | 2023-10-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the CT3 incoming LS in S6-232799, CT3 finds some incomplete information flow and API defination for SEALDD enabled transmission quality measurement procedure. Therefore, the related information flow and API shoud be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper is proposed to complete the information flow and API for SEALDD enabled transmission quality measurement procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The information flows and APIs for transmission quality measurement are not completed, which will casue the misunderstanding for CT3 working group. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.7.3.1, 9.7.3.x (new), 9.7.3.y (new), 9.7.3.z (new), 9.7.3.a (new), 9.7.4.1, 9.7.4.x (new), 9.7.4.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.7.3.1	SEALDD enabled data transmission quality measurement subscription request

Table 9.7.3.1-1 describes the information flow from the VAL server to the SEALDD server for subscribing to the data transmission measurement service.

Table 9.7.3.1-1: SEALDD transmission quality measurement subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Application traffic identifiers | M | Identify of the application traffic (e.g. VAL server ID, VAL service ID) |
| VAL UE identity | O (See NOTE) | Identifier of the VAL UE for which measurements need to be provided, e.g. UE ID, UE address |
| VAL UE group ID | O (See NOTE) | Identifier of a specific VAL UE group, as defined in clause 7.5 of 3GPP TS 23.434 [4]. |
| VAL UE identity list | O (See NOTE) | Identifies a list of VAL UEs. e.g. the list of UE ID, the list of UE address. |
| All VAL UEs Indication | O (See NOTE) | Indicates all VAL UEs of the application identified by application traffic identifiers. |
| Measurement conditions | O | Indicates the temporal and/or spatial conditions. |
| Transmission quality measurement requirements list | M | The measurement requirement information |
| > Measurement ID | M | Measurement identifiers, e.g. latency, bitrate, packet loss rate, jitter |
| > Reporting frequency | O | The reporting frequency of measurement results (e.g. periodic reporting). If not present, it implies periodic reporting. |
| > Reporting periodicity | O | If the reporting frequency is periodic, the reporting periodicity shall be provided. For multiple UEs, it is recommended to give sufficient time to allow report aggregation. |
| > Reporting granularity | O | The reporting granularity indicates whether the measurement report is for individual VAL UE or for VAL UE group or for all VAL UEs, if VAL UE group or all VAL UEs is the measurement target. |
| > Measurement period window | O | Indicates the measurement period window for transmission quality measurements |
| > Measurement expiration time | O | Indicates the measurement expiration time |
| > Reporting criteria | O | Indicates the criteria for reporting measurement results, e.g. if the latency or bitrate reaches below or above a certain value. It also includes a unique identifier for each criteria of more than one criteria is specified. |
| NOTE: One of them shall be present as the measurement target UE. |  |  |

* * * Next Change * * * *

9.7.3.x	SEALDD enabled data transmission quality measurement subscription update request

Table 9.7.3.x-1 describes the information flow from the VAL server to the SEALDD server for updating the data transmission measurement subscription service.

Table 9.7.3.x-1: SEALDD transmission quality measurement subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the updated subscription. |
| Updated transmission quality measurement information | O (See NOTE) | The updated transmission quality measurement information, as described in Table 9.7.3.1-1. |
| NOTE: At least one of these IEs in Table 9.7.3.1-1 is present. |  |  |

9.7.3.y	SEALDD enabled data transmission quality measurement subscription update response

Table 9.7.3.y-1 describes the information flow from the SEALDD server to the VAL server for responding to the transmission quality measurement subscription update request.

Table 9.7.3.y-1: SEALDD transmission quality measurement subscription update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| Expiration time | O | Indicates the expiration time of the subscription. Applicable for successful result. |

9.7.3.z	SEALDD enabled data transmission quality measurement unsubscribe request

Table 9.7.3.z-1 describes the information flow from the VAL server to the SEALDD server for unsubscribing to the data transmission measurement service.

Table 9.7.3.z-1: SEALDD transmission quality measurement unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the unsubscription. |

9.7.3.a	SEALDD enabled data transmission quality measurement unsubscribe response

Table 9.7.3.a-1 describes the information flow from the SEALDD server to the VAL server for responding to the transmission quality measurement unsubscribe request.

Table 9.7.3.a-1: SEALDD transmission quality measurement unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |

* * * Next Change * * * *

9.7.4	APIs

9.7.4.1	General

Table 9.7.4.1-1 illustrates the APIs exposed by SEALDD server for data transmission quality measurement.

Table 9.7.4.1-1: List of SEALDD server APIs for transmission quality measurement

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Sdd_TransmissionQualityMeasurement_subscription | Request | Request/Response | VAL server |
| Sdd_TransmissionQualityMeasurement_subscription_update | Request | Request/Response | VAL server |
| Sdd_TransmissionQualityMeasurement_unsubscribe | Request | Request/Response | VAL server |
| Sdd_TransmissionQualityMeasurement_notification | Notify | Subscribe/notify | VAL server |
| Sdd_TransmissionQualityQuery | Request | Request/Response | Other consumers (e.g. SEALDD server, NSCE server, etc) |

* * * Next Change * * * *

9.7.4.x	Sdd_ TransmissionQualityMeasurement_subscription update operation

API operation name: Sdd_ TransmissionQualityMeasurement_subscription update

Description: The consumer requests for one time for updating transmission quality measurement service.

Inputs: See clause 9.7.3.x.

Outputs: See clause 9.7.3.y

See clause 9.7.2.1 for details of usage of this operation.

9.7.4.y	Sdd_ TransmissionQualityMeasurement_unsubscribe operation

API operation name: Sdd_ TransmissionQualityMeasurement_unsubscribe

Description: The consumer requests for one time for unsubscribing transmission quality measurement service.

Inputs: See clause 9.7.3.z.

Outputs: See clause 9.7.3.a.

See clause 9.7.2.1 for details of usage of this operation.