3GPP TSG-SA WG6 Meeting #57	S6-233128

Xiamen, China, 9th – 13th October 2023	(revision of S6-232971)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0439 | rev | 1 | Current version: | 19.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Preconfigured User Regroup formation in a single MC system in 10.15.3.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2023-09-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently, the MCPTT utilizes the preconfigured regroup procedures in 23.379 while the MCVideo and MCData utilize the preconfigured regroup procedures in 23.280. However, the requirements of the preconfigured regroup for all the three services are identical. Now the two sets of procedures are not fully aligned and make it difficult to maintain the consistency between the 23.280 and 23.379. In order to resolve the issue, keeping a single preconfigured regroup procedures in 23.280 for all the services is a good choice. So the descriptions in clause 10.15.3.2.2 should be updated to cover all the services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the descriptions in clause 10.15.3.2.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency will be caused between the two preconfigured regroups in two individual specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.15.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is one of the set of CRs {CR 0437~0442 to TS 23.280, CR 0379~0388 to TS 23.379, and CR 0197 to TS 23.281 } which is proposed to have the single preconfigured regroup procedures only in TS 23.280. The whole set of CRs should be implemented together. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /************************ First change ******************************/

10.15.3.2.2	Preconfigured User Regroup formation in a single MC system

Figure 10.15.3.2.2-1 illustrates the procedure to initiate a user regroup procedure using a preconfigured MC service group. The procedure takes place prior to the establishment of a group call to the MC service regroup group.

Pre-conditions:

-	MC service clients 2 and 3 are registered with the MC service server.

-	The preconfigured MC service group have been preconfigured in MC service clients 2 and 3, and MC service clients 2 and 3 have received the relevant security related information to allow them to communicate in the MC service user regroup group.

-	MC service client 1 is authorized to initiate a user regroup using the preconfigured user regroup procedure.

-	MC service client 1 is aware of a suitable preconfigured MC service group whose configuration has been preconfigured in the MC service UEs of the MC service users who will be regrouped.

MC service client 1

MC service server

MC service

client 2

MC service client 3

1. Initiate preconfigured user regroup

6a. Notify

user

7a. Preconfigured regroup response

2. Preconfigured regroup request

3. Check authorization

6b. Notify

user

9. Preconfigured regroup response

Home MCPTT service provider

5a. Preconfigured regroup request

5b. Preconfigured regroup request

7b. Preconfigured regroup response

8. Affiliate MC service clients to regroup group

4. Preconfigured regroup request return

Figure 10.15.3.2.2-1: User regroup procedure using preconfigured group in a single MC system

1.	The authorized user of MC service client 1 initiates the user regroup procedure, specifying the list of MC service users to be regrouped (MC service clients 2 and 3), the MC service group ID of the regroup group (if available), and the MC service group ID of the group from which configuration information for the regroup group is to be taken. Alternatively, the MC service client 1 can provide the criteria which allows the MC service server to determine the list of MC service users to be regrouped.

2.	MC service client 1 sends the preconfigured regroup request to the MC service server. The request indicates the list of users to be included in the regroup operation.

3.	The MC service server checks that MC service client 1 is authorized to initiate a preconfigured user regroup procedure. If the preconfigured regroup request is authorized, the MC service server assigns a MC service group ID for this regroup group call if:

i. one is not provided in the request or

ii. the one provided in the request is not accepted.

NOTE 1:	MC service clients can be involved in multiple user and group regroups simultaneously.

4.	The MC service server shall send the preconfigured regroup request return message to MC service client 1 containing the below:

i.	result of whether the preconfigured regroup is authorized or not; and

ii.	the MC service group ID if authorized

If the preconfigured regroup request is not authorized, MC service client 1 shall not proceed with the rest of the steps and this procedure terminates.

5.	The MC service server sends the preconfigured regroup requests to MC service clients 2 and 3 respectively.

NOTE 2:	When using multicast, the MC service server can periodically rebroadcast the preconfigured regroup request.

6.	MC service clients 2 and 3 notify their users of the regrouping respectively.

7.	MC service clients 2 and 3 may send the preconfigured regroup response to the MC service server to acknowledge the regrouping action. These acknowledgements are not sent in response to a multicast transmission of the preconfigured regroup request.

8.	The MC service server affiliates the regrouped MC service clients to the regroup group.

9.	The MC service server sends a preconfigured regroup response to MC service client 1.

NOTE 3:	After the user regrouping procedure, the regrouping remains in effect until explicitly cancelled by the procedure in 10.15.3.2.3.