3GPP TSG-SA WG6 Meeting #58	S6-233647

Chicago, USA, 13th - 17th November 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0111 | rev |  | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS service area update failure cause handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In SA2#160, the CR addressing the MBS service area update issue was agreed in S2-2311901(R17 CR) and S2-2311276 (R18 mirror CR) In the CR, SA2 concluded that: - Case #1- TMGI allocation: NEF reject the request if cannot satisfy the service area request. - Case #2 - MBS Session Creation without TMGI: NEF reject the request if cannot satisfy the service area request. - Case #3 - MBS Session Creation with TMGI: MB-SMF partial accept the request, and provide non-supported MBS service area and indication for this condition; - Case #4 - MBS Session Update: MB-SMF partial accept the request, and provide non-supported MBS service area and indication for this condition. So the behaviour of the MC service server to well handle the above response from the network needs to be defined in order to better make use of the MBS feature: Case #1, #2 – The MC service sever needs to modify the service area, i.e., providing smaller service area, or using unicast delivery. Case #3, #4 – The MC service server may try another new MBS session for the non-supported MBS service area. If still fails, use the unicast delivery. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the clause to describe how the MC service server behave when the MBS service area update with requested MBS service area is failed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MC service server can not handle the MBS service area update failure correctly, and the MBS may not be used as much as impossible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.3.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************** First Changes ********************/

7.3.3.2.x	MBS service area update failure handling

The MBS service area requested by the MC service server during the MBS session update procedure towards the 5G core network may be rejected or partially accepted, the MC service server may take different actions as follows:

-	If the MBS service area requested in the TMGI allocation or MBS Session Creation without TMGI procedure as specified in 3GPP TS 23.247 [15] is rejected, the MC service server may recheck the requested MBS service area and retry with a correct MBS service area. The MC service server may also determine to use the unicast delivery for the group communication in that area.

-	If the MBS service area requested in MBS Session Creation with TMGI or MBS Session Update procedure as specified 3GPP TS 23.247 [15] is partially accepted due to the requested MBS service area is beyond the service area of the serving MB-SMF, or the MBS is not deployed at the updated MBS service area, the MC service server, based on the non-supported MBS service area and the indication provided by the 5GC, may try another new MBS session with the non-supported MBS service area. If it still fails, the MC service server utilizes unicast PDU session for the group communication in that area.