3GPP TSG-SA WG6 Meeting #58	S6-233933

Chicago, USA, 13th – 17th November 2023	(revision of S6-233907)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0541 | rev | 2 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service continuity in ENS via leading ECSP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The service continuity scenarios via leading ECSP are not specified in ENS. The leading ECSP can have SLA for Edge Node Sharing with multiple partners. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add service continuity scenarios via leading ECSP in ENS. In these scenarios, the EEC is not aware of ECSP partners so that the leading ECSP is used to relay service continuity information between the EEC and EAS hosted by partner ECSP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing procedures in supporting service continuity for ENS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Existing clauses: 8.18.2.1 New clauses: 8.18.2.x, 8.18.2.x.1, 8.18.2.x.2, 8.18.2.x.3, 8.18.2.x.4, 8.18.2.x.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.18.2.1	General

Following procedures are defined to support ENS:

-	Application information sharing between ECSPs; and

-	EAS discovery for ENS.

NOTE:	Service continuity for ENS via leading ECSP is described in clause 8.18.2.x.

* * * Next Change * * * *

#### 8.18.2.x	Service continuity for ENS via leading ECSP

##### 8.18.2.x.1	General

The AC connected EAS may be changed when the UE moves within the same partner ECSP service coverage or to a different partner ECSP service coverage or back to leading ECSP service coverage.

S-EAS and T-EAS can be both provided by partner ECSP. Both source EDN and target EDN may be same or different.

Editor’s note: More high-level descriptions are FFS, IE tables and EDGE reference point description enhancement are also FFS.

Editor’s note: It is FFS whether all EDGE-9 interaction in clause 8.18.2.x needs to identify the requestor’s ECSP, e.g. for authorization purpose.

##### 8.18.2.x.2	ACR launching procedure

The S-EES uses the procedure as defined in clause 8.8.3.4 to forward the ACR request with ACR action indicating ACR initiation from EEC towards partner EES over EDGE-9 reference point, along with leading ECSP ID.

##### 8.18.2.x.3	Selected T-EAS declaration

Upon receiving the Selected target EAS declaration request message, the EES ueses the procedure as defined in clause 8.8.3.7 to send the request to the EES of the lead ECSP over EDGE-9 reference point.

##### 8.18.2.x.4	ACR status update procedure

Upon receving ACR status update request from the EAS, if the EAS is serving the UE of the lead ECSP (i.e. ENS case), the EES uses the procedure as defined in clause 8.8.3.8 to send the request to the EES of the lead ECSP. Upon receiving the response from EES of the lead ECSP, the EES sends the response back to the EAS.

##### 8.18.2.x.5	EAS Information provisioning

Upon receving EAS Information provisioning request from the EEC, if the request contains selected EAS ID and selected EAS Endpoint and if the selected EAS is registered to the EES of the partner ECSP (i.e. ENS case), then the EES uses the procedure as defined in clause 8.15.2.2 to send the EAS Information provisioning request to the partner EES over EDGE-9 reference point to indicate about selection of the EAS for the service consumption. The request includes EES ID, EES endpoint address and leading ECSP ID along with the information sent by the EEC.

* * * END of Change * * * *