3GPP TSG-SA WG6 Meeting #59	S6-240287

Athens, Greece 26th February – 1st March 2024

Source:	CATT

Title:	Revised SID on study on enhanced application layer support for location services

Document for:	Approval

Agenda Item:	10

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Study on enhanced application layer support for location services

Acronym:	FS_eLSAPP

Unique identifier:	1000035

Potential target Release:	Rel-19

1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No |  |  | X |  |  |
| Don't know | X |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| x | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 830102 | Enhancement to the 5GC LoCation Services | Stage 2 Normative work for 5GC LoCation Services in Rel-16 |
| 910052 | Enhancement to the 5GC LoCation Services-Phase 2 | Stage 2 Normative work for 5GC LoCation Services in Rel-17 |
| 980111 | 5GC LoCation Services - Phase 3 | Stage 2 Normative work for 5GC LoCation Services in Rel-18 |
| 980003 | CT aspects of 5G_eLCS_Ph3 | Stage 3 Normative work for 5GC LoCation Services in Rel-18 |
| 830077 | NR Positioning Support | Stage 2 Normative work for NR positioning in Rel-16 |
| 900060 | NR Positioning Enhancements | Stage 2 Normative work for NR positioning in Rel-17 |
| 981038 | Expanded and improved NR positioning | Stage 2 Normative work for NR positioning in Rel-18 |
| 960018 | Study on Upper layer traffic steering, switching and split over dual 3GPP access | Stage 1 study work for dual-access based Services in Rel-19 |
| 32079 | Location Services (LCS); Service description; Stage 1 | Stage 1 study work for location Services |
| 960018 | Study on upper layer traffic steering, switching and split over dual 3GPP access | Stage 1 study for DualSteer in Rel-19 |
| 1020031 | Upper layer traffic steering and switching over dual 3GPP access | Stage 1 work item for DualSteer in Rel-19 |
| 1020070 | Study on Multi-Access (DualSteer and ATSSS_Ph4) | Stage 2 study for DualSteer in Rel-19 |

Dependency on non-3GPP (draft) specification:

3	Justification

In Rel-16 and Rel-17, the SEAL (Service Enabler Architecture Layer for Verticals) work item in SA6 has introduced the basic location management architecture serving the different vertical applications, including the generic on-network/off-network function model, procedure, information flows, reference points, and APIs for location management.

In Rel-18, the 5G-enabled Fused Location Service (5GFLS) is studied as part of SEAL-LM(Location Management) architecture and agreed to obtain the UE location information from more sources to further improve the LCS QoS (e.g. accuracy, latency) to meet the requirements from vertical applications.

However, there are still many value-added location service requirements from vertical applications that cannot be fulfilled based on the current SEAL-LM architecture. E.g. Location Alerting, History Trace request or playback, Location format mapping, Location information analysis, etc. And such requests have been identified as a key issue in the 5GFLS study item, but postponed to be studied in future releases due to time limitations.

Besides, lots of AF (Application Function) requests/functionalities related to location services which is specified in SA2 (e.g. TS 23.273, TS 23.586) should be utilized and performed by SEAL-LM architecture in SA6. The functionalities of the Location Management Server or VAL (Vertical Application Layer) server should consider aligning with the AF requests/functionalities as defined in TS 23.273 and TS 23.586. Such as the Location service exposure service requests from AF, GNSS assistance data collection from AF, Ranging/Sidelink Positioning service requests from AF and so on.

Furthermore, as identified in the SA1 and SA2 (see TS 22.261 and TR 23.700-54)  for DualSteer, it is beneficial for a DualSteer Device to apply traffic steering and/or switching between two 3GPP access networks connected to the same or different PLMN networks. Based on the definition in SA1, a DualSteer can be (a) a single UE, in case of non-simultaneous data transmission over the two networks, or (b) two separate UEs in case of simultaneous data transmission over the two networks. The subscriber of the DualSteer Device has two subscriptions/SUPIs, sharing one subscription profile from the same operator. And all of polices are controlled by HPLMN.

If the DualSteer Device includes two separate UEs which could transmit location reports to the application enabled layer (e.g., SEAL-LM) over two access networks simultaneously, the application enabler could get more accurate location information for DualSteer Device via estimating two UE location reports. Or the application enabler may only obtain one of UE location report instead of two to retrieve the location information for DualSteer Device. Thus how the DualSteer Device applying steering and/or switching for location services in the application enabled layer should also be considered and studied.

Therefore, it’s very necessary to further study how to support the above location services based on the SEAL-LM application layer.

4	Objective

Based on the above analysis, the objectives of this study will include:

- Identify the gaps between the service requirements for AF functionalities specified by SA2, new SA1 requirements, an enabler for the value-added location service, and the existing SEAL-LM architecture in the application layer.

- Study SEAL-LM architectural enhancements mainly to support the following items based on the above gap analysis:

  - Further improve the LCS QoS, especially for location accuracy and latency, to meet the strict location requirements for different vertical applications.

  - Enabler for Value-Added Location Services, includes(not limited to):

    - Location information analysis

    - Location Alerting

    - History Trace request or playback

    - Location format mapping

  - Support location services for DualSteer Device with two SUPIs in the application enabled layer.

- Identify key issues based on objective 2 and develop the corresponding functional model and potential solutions.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.700-72 | Study on enhanced application layer support for location services | SA#104 (Jun. 2024) | SA#105 (Sep. 2024) | Liping Wu, CATT, wuliping@cictmobile.com |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

6	Work item Rapporteur(s)

Liping Wu, CATT, wuliping@cictmobile.com

7	Work item leadership

SA6

8	Aspects that involve other WGs

SA2 for core network architecture aspects, SA3 for security aspects, SA5 for charging aspects.

9	Supporting Individual Members

Supporting IM name

CATT

Huawei

ZTE

CMCC

Lenovo

China Telecom

Vivo