3GPP TSG-SA WG6 Meeting #59	S6-240379

Athens, Greece 26th February – 1st March 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0158 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction cardinality IE service API unpublish for CAPIF interconnection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, eCAPIF |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During SA6#58 meeting it was introduced the procedure Service API unpublish for CAPIF interconnection for the sake of alignment between stage 2 and stage 3 specifications (in S6-233911). In stage 3 specification, in order to execute a DELETE operation, it is mandatory to know the serviceApiId URI variable (which identifies an individual service API published in a CCF). However, in S6-233911, the reference to this IE was kept Optional in the Interconnection API unpublish request. Since in the interconnection scenario a CCF could publish several service APIs to another CCF, it is needed to state which API is the one to be unpublished. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the status of the IE “Service API published information reference” to Mandatory (instead of Optional). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missalignment with Stage 3 Specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.25.2.5, 8.25.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

#### 8.25.2.5	Interconnection API unpublish request

Table 8.25.2.5-1 describes the information flow interconnection API unpublish request from a CAPIF core function to another CAPIF core function.

Table 8.25.2.5-1: Interconnection API unpublish request

| Information element | Status | Description |
| --- | --- | --- |
| CCF information | M | The information of the CAPIF core function which unpublishes service APIs, may include identity, authentication and authorization information |
| Service API published information reference (see NOTE) | M | The information which can be used for referencing the information (set) about the published service API by the CCF which publishes service APIs |
| NOTE: Obtained during the interconnection API publish procedure in clause 8.25.3.1. |  |  |

* * * * Next Change * * * *

#### 8.25.3.4	Service API unpublish for CAPIF interconnection

This clause describes the procedure for service API unpublish for CAPIF interconnection.

Pre-condition:

1.	CCF-A and CCF-B connect to each other, and either belong to the single trust domain of the same CAPIF provider or trust domains of different CAPIF providers. When CCF-A and CCF-B belong to trust domains of different CAPIF providers, the two CAPIF providers have business agreement for service API sharing.

2.	CCF-B is configured as the designated CAPIF core function in the trust domain of CAPIF provider A.

3.	CCF-A has successfully published service APIs to the designated CCF (i.e., CCF-B) using the procedure in clause 8.25.3.1.

Figure 8.25.3.4-1: Interconnection API unpublish

1.	CCF-A sends an interconnection API unpublish request to CCF-B. The request includes a set of (previously published) service API information. If the shareable information IE was present in the interconnection API publish request (see clause 8.25.2.1), the API unpublish request should be forwarded to the provided list of CAPIF provider domain information during the publish procedure.

2.	Upon receiving the interconnection API unpublish request, the CCF-B checks whether the CCF-A is authorized to unpublish service APIs. If the check is successful, CCF-B will remove the requested service API information from the API registry.

3.	CCF-B sends an interconnection API unpublish response to CCF-A containing the operation result and triggers notifications to subscribed API invokers (if any) as described in subclause 8.8.4.

* * * * End of Changes * * * *