3GPP TSG-SA WG6 Meeting #59	S6-240786

Athens, Greece 26th February – 1st March 2024	(revision of S6-240752, S6-240625,

S6-240107)

Source:	Samsung, Convida Wireless LLC

Title:	Pseudo-CR on Solution for spatial anchors management

Spec:	3GPP TR 23.700-21 v0.1.0

Agenda item:	8.4

Document for:	Approval

Contact:	Sapan Shah (sapan.shah@samsung.com)

1. Introduction

This pCR provides solution to KI#1 (Enabler support for managing spatial anchors)

2. Reason for Change

Spatial anchor is an association between a location in space (three dimensions) and service information that can be used to identify and access services, e.g. information to access AR media content.

SA6 agreed to study KI#1 (Enabler support for managing spatial anchors). This pCR provides solution for following open issues:

1)	How to enhance existing appliction enabler (either adding new capability in existing server or defining new capability server) to support CRUD operations for spatial anchors (association between location and service information)?

2)	How to discover spatial anchors by the consumer (e.g. UE, VAL server)?

Further, as discussed in the key issue, this pCR proposes to enhance SEAL location management server to provide spatial anchors service.

3. Proposal

It is proposed to agree the following changes to TR 23.700-21 v0.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.156: "Mobile Metaverse Services; Stage 1".

[3]	3GPP TR 22.856: "Feasibility Study on Localized Mobile Metaverse Services".

[4]	3GPP TR 23.700-77: "Study on system architecture for next generation real time communication services; Phase 2".

[5]	3GPP TR 23.700-70: "Study on architecture enhancement for Extended Reality and Media service (XRM); Phase 2".

[6]	3GPP TR 26.813: "Study of Avatars in Real-Time Communication Services".

[r23434]	3GPP TR 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows;".

[r26119]	3GPP TR 26.119: "Media Capabilities for Augmented Reality".

* * * Next Change * * * *

## 7.x	Solution #x: Support for spatial anchor management

### 7.x.1	Solution description

This solution maps to KI# 1. This solution enhances Service Enabler Architecture Layer (SEAL) Location Management (LM) service to provide spatial anchor management service to the VAL server or the UE. The architecture impacts are captured in clause 7.x.2 while procedures to manage spatial anchors are captured in clause 7.x.3.

### 7.x.2	Architecture Impacts

This clause provided architecture enhancements to the SEAL LM architecture as defined in 3GPP TS 23.434 [r23434]. The SEAL LM client is enhanced to provide client side functionalities for managing (i.e. create, read, update and delete) spatial anchors for metaverse application. The SEAL LM server is enhanced to provide server side functionalities for managing spatial anchors for the metaverse applications.

VAL client uses the SEAL LM client to manage spatial anchors for the metaverse applications over LM-C reference point. Similarly, VAL server uses SEAL LM server to manage spatial anchors for the metaverse applications over LM-S reference point. The LM-UU interface is enhanced to provide management of spatial anchors between SEAL LM client and SEAL LM server.

Editor’s note: Whether to use a new application enablement server or a new SEAL server for this solution is FFS.

### 7.x.3	Procedures

#### 7.x.3.1	Creating spatial anchor

Figure 7.x.3.1-1 depicts the procedure for creating association of spatial anchor with location for metaverse applications. The service is provided by SEAL LM server and consumed by VAL server or SEAL LM client.

Figure 7.x.3.1-1: Create spatial anchor

1)	The VAL server (or SEAL LM client) sends a request message to the SEAL LM server to create association of the spatial anchor with the location. The request includes VAL server identiy, position of the anchor and application service identifier, security credentials, spatial anchor service area. The request may include service information of the product to associate it with the spatial anchor, access control rules defining which entities are permitted to discover and access the spatial anchor.

2)	The SEAL LM server authorizes VAL server (or SEAL LM client). If the requestor is authorized, then the SEAL LM server creates the association between spatial anchor and the location, and stores the information as received in the request message. The server creates identity for the spatial anchor. The SEAL LM server sends the response back to the VAL server (or SEAL LM client) including identity for the spatial anchor for a successful case.

#### 7.x.3.2	Updating spatial anchor

Figure 7.x.3.2-1 depicts the procedure for updating spatial anchor association for metaverse applications. The update operation includes modifying existing spatial anchor or deleting the spatial anchor. The service is provided by SEAL LM server and consumed by VAL server or SEAL LM client.

Figure 7.x.3.2-1: Update spatial anchor

1)	The VAL server (or SEAL LM client) sends a request message to the SEAL LM server to update the existing spatial anchor association. The request is used to modify or delete the spatial anchor. The request message includes the identity of the spatial anchor and the application service identifier. To modify the spatial anchor, the request may include service information of the product to associate it with the spatial anchor.

2)	The SEAL LM server authorizes VAL server (or SEAL LM client). If the requestor is authorized, then the SEAL LM server checks whether the spatial anchor as identified by the identity of the spatial anchor exists or not. If the spatial anchor does not exists then the SEAL LM server sends failure response. Otherwise, the SEAL LM server updates the spatial anchor details (for update request) or deletes the spatial anchor details (for delete request). The SEAL LM server sends the response back to the VAL server (or SEAL LM client) including reason for failure for failure case.

#### 7.x.3.3	Get spatial anchor

Figure 7.x.3.3-1 depicts the procedure for getting spatial anchor details for metaverse applications. The service is provided by SEAL LM server and consumed by VAL server or SEAL LM client.

Figure 7.x.3.3-1: Get spatial anchor

1)	The VAL server (or SEAL LM client) sends a request message to the SEAL LM server to get the spatial anchor details. The request includes discovery filters like the position of the user and application service identifier. If the request to get specific spatial anchor, the request may include application specific information (e.g type of service).

2)	The SEAL LM server authorizes VAL server (or SEAL LM client). If the requestor is authorized, then the SEAL LM server determines all spatial anchors from the repository based on the discovery filters provided in the request message. The SEAL LM server sends the response back to the VAL server (or SEAL LM client) including list of spatial anchor details for successful case.

#### 7.x.3.4	Spatial anchor information subscription

Figure 7.x.3.3-1 depicts the procedure for subscribing to receive spatial anchor details for metaverse applications. The service is provided by SEAL LM server and consumed by VAL server or SEAL LM client.

Figure 7.x.3.4-1: Spatial anchor information subscription

1)	The VAL server (or SEAL LM client) sends a subscription request to the SEAL LM server to get notifications about the spatial anchor details. The request includes discovery filters like the position and application service identifier. If the request to subscribe to specific spatial anchor, the request may include application specific information (e.g type of service).

2)	The SEAL LM server authorizes VAL server (or SEAL LM client). If the requestor is authorized, then the SEAL LM server sends successful response along with subscription identity.

NOTE 1:	The SEAL LM server starts monitoring the location of the user if it is not already monitoring.

3-4)	Upon occurance of the event (e.g. spatial anchor added or removed or updated), the SEAL LM server determines all spatial anchors from the repository which are matched with the discovery filters provided in the request. The SEAL LM server sends the notification to the VAL server (or SEAL LM client) including list of spatial anchor details.

### 7.x.4	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution addressing KI#x.

* * * End of Change * * * *