3GPP TSG-SA WG6 Meeting #60	S6-241236

Changsha, P. R. China, 15th – 19th April 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0556 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MC gateway UE corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei,Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCGWUE |  |  |  |  |  | Date: |  |  | 2024-04-06 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the LS from SA3 (S6-240551/S3‑240828), the following clean ups to R18 MCGWUE related content are required - Cl.3 - Definitions related with the function entities designed for the conneciton authorization (MC gateway client, MC gateway UE server, MC gateway UE function) - Cl.8.1.7 - GW MC service ID should be voided and its coressponding usage in other clauses i.e., 11.4.3 and clause 11.5 - Cl.11 11.1 – functionalities clear up to remove the functions cannot be supported in R18. 11.2 – Functional model update to remove the MC gateway client, MC gateway UE server, MC gateway UE function, related reference points, connection authorization of the association b/t MC client on the non-3GPP device and the MC gateway UE, media plane aspect clean up (only unicast delivery from the MC server to the MC client in the non-3GPP device is supported) 11.5 – The whole 11.5 should be voided. - Annex A.7 – The whole A.7 should be voided. - Annex D – should be updated to clean the conneciton authorization related aspects. In addition, the following aspects needs to be considered to improve the TS. (1) The functional model for the two use cases (non-3GPP device can/cannot host the MC client) should be shown individually, as the functions of the MC gateway UE are different. For the case the non-3GPP device can host the MC client, all the CSC-n, MCX-n, SIP-n traffic are end-to-end and terminated at the MC clients within the non-3GPP device, the MC gateway UE only performs the transparently IP packet routing. For the case the non-3GPP device cannot host the MC client, all the CSC-n, MCX-n, SIP-n traffic are terminated at the MC clients at the MC client located within the MC gateway UE, the MC gateway UE delivery the content to the non-3GPP device (outside the scope of SA6) (2) The signalling plane and the application plane are mixed in a single functional model, which may cause confusion. The signalling plane should be introduce to further clarify the content forward function of the MC gateway UE. Moreover, in order to support the location and MBMS with the usage of gateway UE, a new application layer function entity has to be defined in the MC gateway UE. The connection authorization can be introduced as an optional procedure if the MC organization would like to stop the traffic at the MC gateway UE rather than wasting the transmission resource of the MC gateway UE and reject by the MC server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clean up as described in the reason for change part. - Introduce a sperate fuctional model for non-3GPP device can not have the MC client, and introduce the signalling plane for the functional models - Update the functional model to support the connection authorization, location and MBMS handlings at the gateway UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrection functions may be wrongly implemented by the downstream group as well as create confusions to the readers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 8.1.7, 11.1~11.3.3, 11.4.3, 11.5 (including all the subclauses), A.7, D.1, D.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The changes for R19 are different as follows: - Functional model as supporting the Location and MBMS requires a application layer function in the GW UE for the case non-3GPP device host the MC client - Changes to clause 11.5 is to fix the procedures to enabler the early traffic drop at the MC gateway UE rather than wasting the transmission resource between the gateway UE and the server. Also, fix the location and MBMS procedures. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /************************** 1st changes ****************************/

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1]. Not all definitions are used in this document.

Accuracy: Reflects the uncertainty of the location at the moment of location measurement, e.g. see 3GPP TS 25.305 [30] and 3GPP TS 23.032 [31].

Active MC service user profile: The MC service user profile that is currently used by an MC service client of an MC service user while receiving MC service.

Ad hoc Group Communication: The combining of a multiplicity of MC service users into a group for the duration of a communication. When the communication is released, the group no longer exists. If the communication is associated with an alert, then the group continues to exist until the alert is also canceled.

Ad hoc Group emergency alert: The combining of a multiplicity of MC service users into a group for sending an emergency alert. When the alert is cancelled, the group no longer exists. If the alert is associated with a communication, then the group continues to exist until the communication is also canceled.

Altitude: Third dimension for the geographical coordinates at the moment of location measurement, e.g. see 3GPP TS 25.305 [30] and 3GPP TS 23.032 [31].

Bearing: Direction at the moment of location measurement, e.g. see 3GPP TS 25.305 [30].

Chat group: An MC service group that is pre-defined with MC service group ID and member list in the group management server. Group members must join the pre-estabslihed group call to participate.

ECGI: E-UTRAN Cell Global Identifier, which is used to identify cells globally, where the ECGI is constructed from the Mobile Country Code (MCC), Mobile Network Code (MNC) and the E-UTRAN Cell Identifier (ECI).

Interconnection: A means of communication between MC systems whereby MC service users obtaining MC service from one MC system can communicate with MC service users who are obtaining MC service from one or more other MC systems.

Interconnection group: An MC service group that is configured to allow inclusion of MC service group members who are MC service users from partner MC system(s).

LCS network: The 3GPP network that provides location service as defined in 3GPP TS 23.271 [29].

Location: The current physical location of the MC service UE.

MBMS SAI: Multimedia Broadcast Multicast Service Area Identity which is mapped to the MBMS service area.

MC gateway server: A server providing topology hiding for MC service interconnection with a partner MC system, where that partner MC system is in a different trust domain.

MC service: A generic name for any one of the three mission critical services: either MCPTT, or MCVideo, or MCData.

MC service affiliated group member: An MC service user who has indicated an interest in a particular MC service group and has been accepted to participate in MC service group communication for that MC service group.

MC service client: A generic name for the client application function of a specific MC service. MC service client could be replaced by MCPTT client, or MCVideo client, or MCData client depending on the context.

MC service group: A defined set of MC service users with associated communication dispositions (e.g. media restrictions, default priority and commencement directions) configured for the use with one or more MC services.

MC service group affiliation: A mechanism by which an MC service user's MC service(s) communication interest in one or more MC service groups is determined.

MC service group call: A mechanism by which an MC service user can make a one-to-many MC service(s) transmission to other users that are members of MC service group(s).

MC service group de-affiliation: A mechanism by which an MC service user's MC service(s) communication interest in one or more MC service groups is removed.

MC service group home system: The MC system where the MC service group is defined.

MC service group host MC service server: The MC service server within an MC system which provides centralised support for a particular MC service of an MC service group defined in a MC service group home system.

MC service group member: An MC service user, whose MC service ID is listed in a particular MC service group.

MC service ID: A generic name for the user ID of a mission critical user within a specific MC service. MC service ID could be replaced by MCPTT ID, or MCVideo ID, or MCData ID depending on the context.

MC service server: A generic name for the server application function of a specific MC service. MC service server could be replaced by MCPTT server, MCVideo server, or MCData server depending on the context.

MC service user: An authorized user, who can use an MC service UE to participate in one or more MC services.

MC service user profile: The set of information associated to an MC service user that allows that user to employ one or more MC services in a given role and from a given MC service UE.

MC service UE: A UE that can be used to participate in one or more MC services.

MC service UE label: A generic name for identification of a specific MC service UE.

MC system: The collection of applications, services, and enabling capabilities required to provide a single mission critical service or multiple mission critical services to one or more mission critical organizations.

MC user: A user, identified by an MC ID, who, after authorization, obtains mission critical service(s).

Migration: A means for an MC Service user to obtain MC service directly from a partner MC system.

Partner MC system: Allied MC system that provides MC services to an MC service user based on the MC service user profiles that are defined in the primary MC system of that MC service user.

Preconfigured MC service group: an MC service group used only for regrouping that has been configured in advance of a group or user regrouping operation to serve as the source of regroup group configuration.

Pre-arranged group: An MC service group that is pre-defined with MC service group ID and member list in the group management server. Affiliated group members are invited when the group communication is setup.

Pre-selected MC service user profile: The MC service user profile that is to be selected as the active MC service user profile through configuration, and applicable for an authenticated MC service user upon MC service authorization.

Primary MC system: MC system where the MC service user profiles of an MC service user are defined.

Requested Priority: A value for use in a MC service group or MC private communication that, if accepted, is used by the MCX service server to temporarily replace the priority level that is predefined in the MC service group or MC service user profile. This value is used in combination with other factors to determine the application priority for the requested communication.

Selected MC service user profile: The MC service user profile that is to be selected as the active MC service user profile for an MC service upon request by an MC service user.

Serving MC service server: The MC service server which is providing MC service to an MC service client.

NOTE 1:	There is one serving MC service server for each MC service, which can be the primary MC service server of the MC service user of the MC service client, or can be a partner MC service server to which the MC service user has migrated.

Serving MC system: The MC system which is providing MC service to an MC user.

NOTE 2:	The MC system can be the primary MC system of the MC service user, or can be a partner MC system to which the MC service user has migrated.

Speed: Movement at the moment of location measurement, e.g. see 3GPP TS 25.305 [30] and 3GPP TS 23.032 [31].

Time of measurement: Date and time expressed with a certain precision to reflect the moment of the location measurement.

For the purposes of the present document, the following terms given in 3GPP TS 22.280 [3] apply

Mission Critical

Mission Critical Applications

Mission Critical Organization

Mission Critical Service

Functional alias

For the purposes of the present document, the following terms given in 3GPP TS 22.179 [2] apply

Multi-talker control

Group-broadcast group

For the purposes of the present document, the following terms related to a MC gateway UE function apply

MC gateway UE:  A functional entity that enables simultaneous access to the MC system for multiple MC clients.

MC client: Aggregates a set of clients (i.e. Group management client, Configuration management client, Identity management client, Key management client, Location management client and MC service client).

MC server: Aggregates a set of servers (i.e. Group management server, Configuration management server, Identity management server, Key management server, Location management server and MC service server) which serves the MC client accordingly.

Non-3GPP device: A device that enables connectivity towards an MC gateway UE using an access method not specified by 3GPP. A subset of these devices can host an MC client specified by 3GPP.

# /*********************** Next Changes ****************************/

8.1.7	Void

# /*********************** Next Changes ****************************/

11.1	General

An MC gateway UE enables MC service access for a MC service user residing on non-3GPP capable devices with the MC service clients and for devices which are unable to host MC service clients. In order to be able to participate in a MC service, the MC gateway UE provides the following necessary functions to enable the MC service access for such MC clients:

-	3GPP transport resources: Sharing of MC gateway UE 3GPP transport resources among the associated MC service clients.

-	MC client identification: Independent MC service client identification, MC service client authorisation and MC service profile association.

-	Location Management: Providing location management support for the associated MC clients to handle the location information and triggers related to 3GPP access network.

-	MC client communication integrity: Ensure that signalling, media content and traffic attributes, e.g. priority and QoS, of communications between the MC system and MC service client remains unaltered.

-	Transparently MC traffic routing:

The MC gateway UE provides the transparently routing without altering the content functions for the following traffic between the MC client at the Non-3GPP device and the MC servers for the case non-3GPP device hosts the MC client:

-	signalling between MC client on the non-3GPP device and MC service server/CSC server

-	signaling between the signaling user agent residing on the non-3GPP device and the SIP core

-	signaling between the HTTP client residing on the non-3GPP device and the HTTP proxy

-	Media between the MC service server and the MC clients on the non-3GPP device

For the case non-3GPP device without the MC client, the MC gateway UE provides the MC clients and the transparently routing without altering the content functions for the traffic exchanged between the non-3GPP device and the MC client at the MC gateway UE.

NOTE 1:	The traffic exchange between the non-3GPP device and the MC client at the MC gateway UE is outside the scope of the present document, and implementation specific.

- Multicast/Broadcast traffic forwarding:

The MC gateway UE provides support of the MBMS signaling handlings including the MBMS listening report and service announcement processing, and receives the MBMS traffic and forwards it to the MC clients in non-3GPP device.

NOTE 2:	Further information about MC gateway UE selection are summarized in Annex D.

11.2	Functional Model

11.2.0	General

The MC gateway UE offers access to the MC server for several MC clients as shown in Figure 11.2.0-1. The MC clients can be either located in the MC gateway UE or in non-3GPP devices connected to the MC gateway UE via non-3GPP access.

Figure 11.2.0-1: Functional architecture

For non-3GPP devices which can host an MC client, the MC client at non-3GPP device is used by the MC service user to access the MC service system.

For non-3GPP devices which cannot host MC clients, the MC gateway UE hosts the instantiation of the MC client which is used by the MC service user to access the MC system, i.e., the MC gateway UE instantiates an MC client, acting on behalf of the non-3GPP device, to request services (e.g. emergency call, group calls, short data messages services, etc.) towards the MC service system.

NOTE 1:	The communication interworking and the definition of associated procedures between the MC client (initiated at the MC gateway UE) and the non-3GPP devices is out of scope of this document.

NOTE 2:	MC clients residing on a non-3GPP device cannot use UICC credentials to perform authorisation with the 3GPP transport system.

If the MC service user on the non-3GPP device utilizes multiple MC services simultaneously, the MC service access may also be provided by one or multiple MC gateway UEs as shown in figure 11.2.0-2 while restricting each MC service to one MC gateway UE (e.g. MCPTT via MC gateway UE1, MCData via MC gateway UE2).

Figure 11.2.0-2: Simultaneous multiple MC gateway UE use by a single non-3GPP device

NOTE 2:	Even not shown in the above figure, the same principle of simultaneous use of multiple MC gateway UEs is applied for non-3GPP devices which cannot host an MC client.

11.2.1	Functional model

Before serving the MC service user using non-3GPP device behind the MC gateway UE, the MC gateway UE shall complete the UE authentication with the 3GPP network.

For both the non-3GPP device host the MC client and the non-3GPP device without hosting the MC client, the authentication and authorization of the MC client for access the MC service follows the procedure defined in clause 10.6.

Figure 11.2.1-1 illustrates the functional model for the application plane for the MC system to support a MC service user to participate in a MC service via the MC gateway UE using a non-3GPP device host MC clients.

CSC server(s)

3GPP network

MC service server

Common service core

MC gateway UE

Non-3GPP device

MC service client

CSC client

MCX-n

CSC-n

Connection control client function

Connection control GW function

Connection control server function

GW-local

GW-core

Figure 11.2.1-1: Functional model of MC gateway UE application plane (non-3GPP device host client)

The Connection control client function in the non-3GPP device, Connection control GW function in the MC gateway UE and the Connection control server function in the MC service server are specified to support the 3GPP location specific signalling handlings, MBMS delivery handlings and the connnection and traffic control at the MC gateway UE to more efficiently make use of the Uu resource of the MC gateway UE. The functionalities and the procedures to supported those capabilities are specified in clause 11.5.

Figure 11.2.1-2 illustrates the functional model for the signalling control plane corresponding to figure 11.2.1-1.

Common services core

MC service server

MC gateway UE

3GPP network

SIP database

Rx

SIP core

AAA-1

SIP-2

HTTP-1

To another SIP core

Registrar finder/

I-CSCF

Local proxy/P-CSCF

Registrar-

AS selection

/S-CSCF

HTTP client

SIP-3

HTTP server

HTTP Proxy

HTTP-3

HTTP-2

To other HTTP Proxy

MC gateway server

SIP AS

SIP AS

HTTP client

HTTP server

SIP AS

HTTP-1

HTTP-2

HTTP-3

SIP-3

Other MC system

SIP core

SIP-2

Diameter proxy

AAA-2

Non-3GPP device

Signalling user agent

HTTP client/

server

SIP-1

HTTP-1

HTTP  proxy

HTTP client/

server

HTTP-1

HTTP-2

Figure 11.2.1-2: Functional model of MC gateway UE signalling plane (N3GPP device host client)

Figure 11.2.1-3 shows the functional model for the application plane for the MC system to support a MC service user to participate in a MC service via the MC gateway UE using a non-3GPP device without MC clients.

MC service client

N3GPP device

CSC client

MC gateway UE

CSC server(s)

3GPP network

MC service server

Common service core

CSC--n

MCX-n

Connection control GW function

Connection control server function

GW-core

Figure 11.2.1-3: Functional model of MC gateway UE application plane (non-3GPP device not host MC client)

The functional model for the signalling control plane as described in figure 7.3.1-2 is applied to figure 11.2.1-3.

#### 11.2.2	Reference points

11.2.2.1	General

The reference points for the use of the MC gateway UE are described in the following subclauses.

11.2.2.2	Reference point GW-local (between the connection control client function and the connection control GW function)

The GW-local reference point between the connection control client function in the non-3GPP device and the connection control GW function in the MC gateway UE is used:

-	To authorize the use of the MC gateway UE for specific service by the MC service user.

-	To manage (request/release) forwarding of the signallingfrom the MC gateway UE to the connection control client function and its associated MC clients for unicast, multicast/broadcast communication using corresponding identifiers, e.g., TMGI.

-	To disconnect from the use of the MC gateway UE.

-	To notify about the connection status.

-	3GPP specific location provision and reporting handlings.

The GW-local reference point is based on HTTP-1 reference point.

11.2.2.3	Reference point GW-Core (between the connection control GW function and the connection control server function)

The GW-Core reference point between the connection control GW functionin the MC gateway UE and the connection control server function in the MC service server shall be used:

-	To authorise the use of the MC gateway UE by for specific service by the MC service user.

-	To manage (request/release) transport resources between the MC gateway UE and the MC service server signalling purposes are associated with the MC clients residing on a non-3GPP device,

-	To disconnect from the use of the MC gateway UE.

NOTE 1:	Signalling of the MC clients utilizes the allocated resources (default bearer for EPS or corresponding QoS flow in 5GS) and if necessary, MC gateway UE may request additional resources.

NOTE 2:	The authorized use of necessary connectivity between the non-3GPP device and the MC gateway UE is outside 3GPP specification.

The GW-Core reference point is based on HTTP-1 reference point.

11.2.2.4	Reference points MCX-n, CSC-n, SIP-1 and HTTP-1

The reference points MCX-n and CSC-n belonging to the application plane and the reference points SIP-1 and HTTP-1 belonging to the signaling control plane. The content over those reference points are transparently routed by the MC gateway UE between MC client on the non-3GPP device towards the MC gateway UE and the corresponding MC service server without altering the content.

MCX-n, CSC-n, SIP-1 and HTTP-1 reference points are specified in the present specification. The MCX-n reference points are specified in 3GPP TS 23.379 [16], 3GPP TS 23.281 [12] and 3GPP TS 23.282 [13].

#### 11.2.3	Media plane aspects

Only unicast media from the MC system to the MC client located in the non-3GPP device is supported.

NOTE 1:	Multicast/broadcast media (e.g., MCPTT-8/9, MCVideo-8/9, MCData-FD-5) from the MC system to the MC client in the non-3GPP device is supported in the present document.

For the case the non-3GPP device is able to host the MC client, all the CSC-n, MCX-n, SIP-n traffic are end-to-end and terminated at the MC clients at the non-3GPP device, the MC gateway UE only performs the transparently IP packet routing.

For the case the non-3GPP device is not able to host the MC client, all the CSC-n, MCX-n, SIP-n traffic are terminated at the MC clients at the MC client located within the MC gateway UE, the MC gateway UE exchange the content with the non-3GPP device.

11.3	Using identities behind the MC gateway UE

11.3.1	General

The MC gateway UE enables access to the MC system for MC clients either hosted on the MC gateway UE or hosted on non-3GPP devices using non-3GPP connectivity with the MC gateway UE.

An MC service user behind the MC gateway UE has no 3GPP transport access credentials (i.e., no UICC), whereas the MC system relies on private user identity associated with public user identities (i.e., IMPU/IMPI) needed for authentication purposes. Moreover, the one-to-one correspondence between MC service ID and IMPUs enables routing of signalling traffic between the MC service server and the MC service client.

The use of the various mission critical user identities is determined by clause 8 of the present document.

11.3.2	When using separate IMS subscriptions

If the MC clients are hosted by non-3GPP devices or the MC clients are hosted by the MC gateway UE, an MC service user corresponding to an MC client uses a dedicated IMS/SIP subscription without any relation to the MC gateway UE subscription.

Figure 11.3.2-1 depicts the use of the IMS Credential (IMC) application in accordance with 3GPP TS 23.228 [9].

Figure 11.3.2-1: IMC per MC service client

11.3.3	When sharing MC gateway UE's IMS subscription

The MC clients, instantiated at the MC gateway UE on behalf of the non-3GPP devices which cannot host an MC client, rely on MC service IDs provided by the MC gateway UE. In this case, the MC clients instantiated at the MC gateway UE share the credentials from the IMS subscription of the MC gateway UE, as shown in Figure 11.3.3-1.

Figure 11.3.3-1: Sharing MC gateway UE's IMC for non-3GPP devices which cannot host a client

# /*********************** Next Changes ****************************/

### 11.4.3	IP address association using the MC gateway UE

If the MC client relies on IP address provided by the MC gateway UE, the MC gateway UE shall store the correlation between the non-3GPP device and the IP address (MC gateway UE IP and the local IP) used by the MC client. Subsequent procedures initiated by the MC client, i.e. SIP registration, user authentication and service authorisation uses the MC gateway UE's IP address.

The MC gateway UE assigns the local IPs from address pool which it maintains to the individual MC clients. Managing the local IP addresses, re-using the local IP address, and how the traffic is routed between the MC clients and the network is left for implementation.

Whether and how the service continuity procedures are impacted

Whether and how the service continuity procedures are impacted

Hosted by a non-3GPP device

Whether and how the service continuity procedures are impacted

Whether and how the service continuity procedures are impacted

MC gateway UE

Whether and how the service continuity procedures are impacted

MC servers

MC client(s)

Hosted by a MC gateway UE

MC client(s)

Local IP

MC gateway UE IP

IP address mapping

MCX-n

CSC-n

SIP-n

MCX-n

CSC-n

SIP-n

Figure 11.4.3-1: non-3GPP device uses MC gateway UE's IP address

# /*********************** Next Changes ****************************/

11.5	Procedures and information flows

11.5.1.1	General

In order to better make use of the limited resource at the MC gateway UE, the use of connection provided by  an MC gateway UE by a MC service user for specific service(s) may require authorisation verification by the MC system. Two different types of non-3GPP devices are supported, those which can host MC service client and those which cannot host MC service clients.

Before serving the MC user who using the non-3GPP device, the MC gateway UE shall perform and complete the identity authenciation with the identity mangement server via the MC gateway UE.

NOTE 1:	For simplicity the identitfy management client and its CSC-1 is not shown in the funcitonal model.

Before the connection authorization, the MC service user using a non-3GPP device shall perform and complete the identity authenciation with the identity mangement server via the MC gateway UE. The MC service IDs are retrieved from the identity mangement server.

Before the connection authorization is successful, the MC gateway UE only handls the GW-local and GW-core signallings without routing other content between the MC client in the non-3GPP device or the MC gateway UE and the MC system. After the conneciton authorization is premitted, the MC gateway UE starts to route the authorized service traffic between the MC client and the MC system.

NOTE 2:	If any service traffic received from the MC client (in the non-3GPP device or the MC gateway UE), the MC gateway UE directly discards such packet to avoid Uu resource is wasted as such traffic is finally rejected by the MC service server.

11.5.1.1a	Information flows

11.5.1.1a.1	Connection request (connection control client function – connection control GW function)

Table 11.5.1.1a.1-1 describes the information flow connection request sent from the connection control client function, which resides on a non-3GPP device, to the connection control GW function in MC gateway UE.

Table 11.5.1.1a.1-1: Connection request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID(s) | M | The MC service ID(s) of the requesting MC service user. |
| NOTE: The MC service ID indicates for which MC service the connection is requested. |  |  |

11.5.1.1a.2	Connection response (connection control GW function – connection control client function)

Table 11.5.1.1a.2-1 describes the information flow connection response sent from the connection control GW function in the MC gateway UE to the connection control client function residing on a non-3GPP device.

Table 11.5.1.1a.2-1: Connection response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID(s) | M | The GW MC service ID of the requesting MC service user. |
| Response | M | Result of the connection authorization request, service feasibility, and connection evaluation. |

11.5.1.1a.3	Connection request (connection control GW function – connection control server function)

Table 11.5.1.1a.3-1 describes the information flow connection authorization request sent from the MC gateway client, which resides on a non-3GPP device, to the MC gateway UE, and from the MC gateway UE to the MC service server.

Table 11.5.1.1a.3-1: Connection request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID(s) | M | The MC service ID(s) of the requesting MC service user. |
| MC service ID | M | The MC service ID of the MC gateway UE |
| NOTE: The MC service ID indicates for which MC service the connection is to be authorised. |  |  |

11.5.1.1a.4	Connection response (connection control server function - connection control GW function)

Table 11.5.1.1a.4-1 describes the information flow connection authorization response sent from the MC server to the MC gateway UE.

Table 11.5.1.1a.4-1: Connection response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID(s) | M | The GW MC service ID(s) of the requesting MC service user. |
| MC service ID | M | The MC service ID of the MC gateway UE |
| Response | M | Result of the connection authorization request, service feasibility, and connection evaluation. |

11.5.1.2	Connection authorisation for non-3GPP devices that host an MC client

11.5.1.2.1	General

This clause is applied to non-3GPP devices which can host an MC client. The MC server performs authorization for the use of the MC gateway UE by the MC service user for one or more MC service(s). The MC clients is notified about the connection status.

After the successful connection authorization, the MC gateway UE is allowed to route the service traffic from/to the requesting MC service user.

11.5.1.2.2	Void

11.5.1.2.3	Connection authorisation procedure

The procedure for connection authorisation via an MC gateway UE towards an MC server is shown in figure 11.5.1.2.3-1.

Pre-conditions

-	The MC service user wishes to have access to MC services using a non-3GPP device.

-	The connection control client function has been configured with the necessary parameters needed for connectivity with the MC gateway UE.

-	The MC client (i.e., identity management client) has performed the identity authentication with the MC system and retrieved the MC service ID(s).

-	The MC gateway UE has performed service authorization identity authentication with the MC system and retrieved its MC service ID.

-	The MC gateway UE has been selected..

NOTE:	The selection of MC gateway UE is outside the scope of the present document.

Figure 11.5.1.2.3-1: Connection authorisation with an MC server via an MC gateway UE

1.	The connection control client function requests connection for one or more MC service(s)  via the MC gateway UE. The connection control client function provides the MC service ID(s) of the MC service user.

2.	The connection control GW function checks whether the requested MC service, as indicated by the MC service ID(s), is supported. If supported, the connection control GW function resolves the MC service server based on the requesting MC service ID(s). The connection control GW function may also check whether sufficient resources are available or if any other local criteria are met. If the MC service is supported, the procedure continues with step 3, otherwise the procedure proceeds with step 7 with failure response.

3.	For each requested MC service, the connection control GW function sends the connection request to the corresponding MC service server for each MC service. The MC gateway UE includes its own corresponding MC service ID in this connection request towards the MC service server.

4.	The MC server performs an authorization check, to verify that access via the MC gateway UE by the MC service user for this service is permitted. An MC service server shall reject the connection when the MC server receives connection from a connection control GW function for this MC service for which the connection already exists with the same or different MC gateway UE.

5.	The MC server sends the connection response to the MC gateway UE.

NOTE 2:	Step 3 to Step 5 performs for each MC service being requested.

6.	The connection control GW function marks the MC service user as authorized to exchange the MC service traffic with the MC service server via the MC gateway UE. The MC gateway UE now is allowed to route the service traffic between this MC service user and the MC service server.

7.	The connection control GW function sends the connection response to the MC gateway client.

After successful connection authorization, the MC service client is authorized to use this MC gateway UE to communicate with the MC service server e.g., service authorization, affiliation, call and other procedures. .

If the MC service user wishes to have access to another MC service which is not covered by the existing connection, the above procedure is may be used to update the authorization. The MC service user may select a different MC gateway UE for the new MC service, if multiple MC gateway UEs are available.

11.5.1.3	Connection authorisation for non-3GPP devices that do not host an MC client

11.5.1.3.1	General

The clause is applied to non-3GPP devices which cannot host an MC client. The MC service server performs authorization for the use of the MC gateway UE by the MC service user for one or more MC service(s).

11.5.1.3.2	Void

11.5.1.3.3	Connection authorisation procedure

The procedure for connection authorisation of an connection control GW function hosted by the MC gateway UE towards an MC server is shown in figure 11.5.1.3.3-1.

Pre-conditions

-	The MC service user wishes to have access to MC services using a non-3GPP device, where the connection control GW function and MC clients are hosted by the MC gateway UE.

-	The MC gateway UE has been selected.

NOTE:	How the MC gateway UE is selected is outside the scope of the present document.

-	An identity management client instantiated by the MC gateway UE for the MC service user, has performed the identity authentication with the MC system and retrieved the MC service ID(s).

-	The MC gateway UE has performed identity authentication with the MC system and retrieved the MC service ID.

Figure 11.5.1.3.3-1: Connection authorisation of an MC gateway client hosted by an MC gateway UE

1.	The connection control GW function checks whether the requested MC service, as indicated by the MC service ID(s), is supported. If supported, the connection control GW function resolves the MC service server based on the requesting MC service ID(s). The connection control GW function may also check whether sufficient resources are available or if any other local criteria are met. If the MC service is supported, the procedure continues with step 3, otherwise the procedure proceeds with step 7 with failure response.

2.	For each requested MC service, the connection control GW function sends the connection request to the corresponding MC service server for each MC service. The MC gateway UE includes its own corresponding MC service ID in this connection request towards the MC service server.

3.	The MC server sends the connection response to the connection control GW function residing on the MC gateway UE. The connection control GW function marks the MC service user as authorized to exchange the MC service traffic with the MC service server via the MC gateway UE. The MC gateway UE now is allowed to route the service traffic between this MC service user and the MC service server.

After successful connection authorization, the MC service client is authorized to use this MC gateway UE to communicate with the MC service server e.g., service authorization, affiliation, call and other procedures.

If the MC service user wishes to have access to another MC service which is not covered by the existing connection, the above procedure is may be used to update the authorization. The MC service user may select a different MC gateway UE for the new MC service, if multiple MC gateway UEs are available.

# /************************** Next changes ****************************/

11.5.2	3GPP access network related location information management

11.5.2.1	General

The MC clients residing on the non-3GPP devices may receive the location reporting configuration from the location management server containing the trigger criteria related to 3GPP access network related location parameters and the requested location information may contain the 3GPP access network related location information. The procedures defined in this subclause enables the MC clients residing on the non-3GPP devices to request the MC gateway UE to handle these triggers on its behalf and to request the required location information (3GPP access network related) from MC gateway UE.

11.5.2.2	Information flows

11.5.2.2.1	MC GW location reporting configuration

Table 11.5.2.2.1-1 describes the information flow from the connection control client function, which resides on a non-3GPP device, to the connection control GW function in the MC gateway UE for the location reporting configuration.

Table 11.5.2.2.1-1: MC GW location reporting configuration

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requesting MC service user |
| Requested location information | O (see NOTE 1) | Identifies what location information is requested |
| Triggering criteria | O (see NOTE 1) | Identifies when the location management client will send the location report (see NOTE 2) |
| Minimum time between consecutive reports | O (see NOTE 1) | Defaults to 0 if absent |
| NOTE 1: If none of the information elements is present, this represents a cancellation for location reporting, if configured. NOTE 2: The triggering criteria contains only the events related to the 3GPP access network. |  |  |

11.5.2.2.2	MC GW location information report

Table 11.5.2.2.2-1 describes the information flow from the connection control GW function in the MC gateway UE to the connection control client function residing on a non-3GPP device for the location information reporting.

Table 11.5.2.2.2-1: MC GW location information report

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requesting MC service user |
| Triggering event | O | Identity of the event that triggered the sending of the report |
| Location information (see NOTE) | O | Location information of the MC gateway UE |
| NOTE: The following location information elements which are related to 3GPP access network shall be present (configurable): Serving and neighbouring ECGI, MBMS SAIs, MBMSfnArea, PLMN ID. |  |  |

11.5.2.2.3	MC GW location information request

Table 11.5.2.2.3-1 describes the information flow from the connection control client function residing on a non-3GPP device to the connection control GW function in the MC gateway UE for requesting an immediate location information report.

Table 11.5.2.2.3-1: MC GW location information request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requesting MC service user |

11.5.2.3	Procedures

11.5.2.3.1	Event triggered location reporting procedure

The procedure for how the connection control client function residing on non-3GPP devices handling the location reporting configuration containing the trigger criteria related to the 3GPP access network related location information is shown in the figure 11.5.2.3.1-1.

Pre-conditions

-	The MC service user wishes to have access to MC services by using a non-3GPP device.

-	The MC client successfully completed service authorization via MC gateway UE.

Figure 11.5.2.3.1-1: Event-triggered location reporting procedure

1.	MC client receives the location reporting configuration request from LMS which contains the triggering criteria of 3GPP access network related location information changes.

2.	MC client forwards the MC GW location reporting configuration via the connection control client function to the connection control GW function in the MC gateway UE containing the 3GPP access network related location information triggers and the requested location information. MC gateway UE stores the location reporting configuration and starts monitoring for the triggers as received in the MC GW location reporting configuration.

3.	A location reporting event occurs, triggering step 4.

4.	MC gateway UE sends the MC GW Location information report containing the location information requested to the connection control client function via the connection control GW function.

5.	Connection control client function forwards the received information to MC client. MC client updates the locally available location information with the location information received from the MC gateway UE.

6.	The MC client sends a location information report to the location management server, containing location information identified by the location management server and available to the MC client.

11.5.2.3.2	On-demand location reporting procedure

The MC client may need to immediately send the location report to the location management sometimes and the requested location information may be related to the 3GPP access network. Under these circumstances the MC client can request the MC gateway UE to report its location information as described in the figure 11.5.2.3.2-1.

Pre-conditions

-	The MC service user wishes to have access to MC services by using a non-3GPP device.

-	The MC client successfully completed service authorization via MC Gateway UE.

Figure 11.5.2.3.2-1: On-demand location reporting procedure

1.	MC client receives the location information request from LMS to send the location information immediately or any other events where it has to send the location report to the location management server immediately like initial login, group call etc. Requested location information includes the location information related to 3GPP access network.

2.	MC service user is notified and asked for permission to share location information. MC service user can accept or deny the request.

3.	MC client sends the MC GW Location information request via the via the connection control client function to the connection control GW function in the MC gateway UE requesting for the location information related to the 3GPP access network of the MC gateway UE.

4.	MC gateway UE sends the MC GW Location information report containing the location information requested  to the connection control client function via the connection control GW function.

5.	Connection control client function forwards the received information to MC client. MC client updates the locally available location information with the location information received from the MC gateway UE.

6.	The MC client sends a location information report to the location management server, containing location information identified by the location management server and available to the MC client.

11.5.2.3.3	Location reporting cancel procedure

The location reporting cancel procedure reuses the information flow of location reporting configuration as defined in the subclause 11.5.2.3.1 as described in the figure 11.5.2.3.3-1

Pre-conditions

-	The MC service user wishes to have access to MC services by using a non-3GPP device.

-	The MC client successfully completed service authorization via MC gateway UE.

-	The MC client no longer needs the location information report from MC gateway UE.

Figure 11.5.2.3.3-1: On-demand location reporting procedure

1.	The location management client sends MC GW location reporting configuration without any information element to the connection control GW function in the MC gateway UE via the connection control client function to stop location reporting from the MC gateway UE.

2.	The MC gateway UE stops sending location information reports to the MC client via the connection control GW function and connection control client function.

# /************************** Next changes ****************************/

11.5.3	MBMS support for MC clients residing on non-3GPP devices

11.5.3.1	General

This subclause addresses the MBMS support by the MC gateway UE for the MC clients residing on the non-3GPP devices. The MC clients instruct the MC gateway UE about the corresponding MBMS bearer details to enable MC gateway listening on them. On demand, MC gateway UE forwards the traffic received over MBMS bearer to the MC clients residing on non-3GPP devices. With the procedure defined in this subclause MBMS bearer can be supported for the MC clients residing on non-3GPP devices. Changes required are confined to the reference point between MC gateway UE and the MC clients residing on non-3GPP devices. MC service server may consider the location of the corresponding MC clients as defined in subclause 11.5.2 while deciding to establish MBMS bearer.

11.5.3.2	Information flows

11.5.3.2.1	MC GW MBMS bearer announcement

Table 11.5.3.2.1-1 describes the information flow from the MC client which resides on a non-3GPP device to the MC gateway UE for sharing the details of MBMS bearer announcement received by the MC Client from the MC Service server.

Table 11.5.3.2.1-1: MC GW MBMS bearer announcement

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requesting MC client. |
| TMGI | M | TMGI information |
| List of service area identifier | M | A list of service area identifier for the applicable MBMS broadcast area. |
| Frequency | O | Identification of frequency if multi carrier support is provided |
| SDP information | M | SDP with media and floor control information applicable to groups that can use this bearer (e.g. codec, protocol id, FEC information) |
| Monitoring state | O | The monitoring state is used to control if the client is actively monitoring the MBMS bearer quality or not. |
| ROHC information | O | Indicate the usage of ROHC and provide the parameters of the ROHC channel to signal to the ROHC decoder. |

# /************************** Next changes ****************************/

11.5.3.2.3	Void

-

# /************************** Next changes ****************************/

11.5.3.2.7	Void

-

# /************************** Next changes ****************************/

11.5.3.3	Procedures

11.5.3.3.1	Procedure for handling MBMS bearer announcement

MC clients residing on non-3GPP devices receive MBMS bearer announcements from the MC system, MC clients share the details of the MBMS bearer received in MBMS bearer announcement to the MC gateway UE. This enables the MC gateway UE to start monitoring the corresponding MBMS bearer.

Figure 11.5.3.3.1-1 illustrates the procedure for handling the MBMS bearer announcement by the MC client and the MC gateway UE.

Pre-conditions:

1.	The MC client has been configured with the necessary parameters to enable the use of the MC gateway UE.

2.	The MC client successfully completed service authorization via MC gateway UE.

Figure 11.5.3.3.1-1: Handling of MBMS bearer announcement

1.	The MC service server establishes the MBMS bearer(s) according to the procedures defined in 3GPP TS 23.468 [18]. Service description associated with the MBMS bearer(s) is returned from the BM-SC.

2.	The MC service server provides service description information associated with the MBMS bearer to the MC client residing on non-3GPP devices via MC gateway UE.

3.	The MC client sends the MC GW MBMS bearer announcement to connection control GW function in the MC Gateway UE via the connection control client function . The messages containing the MBMS bearer related information received from the MC service server.

4.	The MC gateway UE stores the information associated with the TMGI(s). The MC gateway UE uses the TMGI and other MBMS bearer related information to activate the monitoring of the MBMS bearer.

5.	The MC gateway UE that enters or is in the service area of at least one announced TMGI notifies to the MC client that it can receive data over MBMS by sending the MC GW MBMS listening status report to the connection control client function via the connection control GW function. The MC GW MBMS listening status report also contains the details of the non-3GPP transport resources related parameters. The MC gateway UE may choose to send the details of existing transport resources information as part of non-3GPP transport resources establishment parameters IE if existing communication channel can be reused.

6.	The MC Gateway UE forwards the MC service data it received over the MBMS bearer from the MC service server to the MC client.

7.	The MC client sends the MBMS Listening Status Report to the MC service server indicating that it is able to receive the media over MBMS.

11.5.3.3.2	Void

-

.

11.5.3.3.3	Procedure for MBMS bearer suspension notification

The MC service server can choose to instruct some MC clients to send the MBMS bearer suspension report when notified by RAN. When the MC clients are residing on non 3GPP devices, MC gateway UE would be the one listening on the MBMS bearers. When RAN decides to suspend the MBMS bearer it indicates the MC gateway UE. MC gateway UE to notify the MC clients it is serving so that MC clients can report the same to the MC service server. This procedure is applicable only if the MC client is instructed to report the MBMS bearer suspension. Irrespective of whether the MC clients need to send the MBMS bearer suspension report to the MC service server, MC gateway can choose to notify the MC clients it is serving whenever RAN suspends the MBMS bearer. MC clients can then decide to send the MBMS bearer suspension report to the MC service server only if they are instructed by the MC service server.

Figure 11.5.3.3.3-1 illustrates the procedure for MC clients residing on non-3GPP devices reporting the MC service server about the MBMS bearer suspension.

Figure 11.5.3.3.3-1: MBMS bearer suspension notification

1.	The MC service server sends an MBMS suspension reporting instruction to the MC client residing on non-3GPP device.

NOTE:	This message may be included in the MBMS bearer announcement message and may be sent both on a unicast bearer and a multicast bearer.

2.	RAN decides to suspend the MBMS bearer, according to existing procedures in 3GPP TS 36.300 [21].

3.	An MBMS suspension indication is sent to the MC gateway UE in the MSI (MCH Scheduling Information), according to existing procedures in 3GPP TS 36.300 [21].

4.	The MC gateway UE detect the MBMS suspension and sends an MC GW MBMS suspension indication to the connection control client function in MC Client residing on non-3GPP device via the connection control GW function.

5.	Connection control client function forwards such information to the MC client. The MC client MBMS suspension report to the MC Service server via MC gateway UE.

11.5.3.3.2A	Void

11.5.3.3.4	Procedure for reporting MBMS bearer quality

The MC gateway UE listening on the MBMS bearer has to report the MBMS bearer quality to the MC clients so that MC clients can report the same to the MC service server. MC Gateway UE monitors an MBMS bearer to receive MC service media. Based on the received quality (e.g. radio level quality) the MC gateway UE needs to inform the MC Clients which requested the MC gateway UE to listen on MBMS bearer, whether it is able to receive the MC service media on the MBMS bearer with sufficient quality or not the MC Clients can inform the MC service server accordingly.

Figure 11.5.3.3.4-1 illustrates the procedure for MC clients residing on non-3GPP devices reporting the MC service server about the MBMS bearer quality.

Pre-conditions:

1.	There is an MBMS bearer activated and the MBMS bearer information is announced to the MC gateway UE.

2.	The MC gateway UE is located in the MBMS broadcasting area

3.	The MC gateway UE monitors SIB-13 (or SIB-20) and (SC-)MCCH to receive the modulation and coding scheme.

4.	The MC gateway UE monitors the cell specific reference signal and when MBSFN transmission is used, the MBSFN specific reference signals.

Figure 11.5.3.3.4-1: Reporting MBMS bearer quality

1.	The MC gateway UE follows the Step 1 of the procedure as described in clause 10.7.3.6.2 for the MC service UE. The corresponding MBMS bearer quality information is forwarded to all the MC clients which have asked the MC gateway UE to listen on the particular MBMS bearer.

2.	If the MBMS bearer quality reaches a certain threshold, the MC gateway UE sends an MC GW MBMS bearer quality report to the connection control client function in the non-3GPP device via the connection control GW function . The threshold is used to define the MBMS listening status, which indicates if the MBMS bearer quality has been acceptable or not to receive a specific MC service media. If the MBMS bearer quality is mapped to a different MBMS reception quality level, the MC gateway UE may send an MBMS Bearer Quality report including the MBMS reception quality level to the MC Client.

NOTE:	The threshold used to indicate MBMS bearer quality depends on service type (i.e. MCPTT, MCVideo or MCData) and the metrics used. The metrics used and the associated thresholds are out of scope of this specification.

3.	Connection control client function forwards such information to the MC client. The MC Client sends the MBMS listening status report to the MC Service server via MC gateway UE containing the information received in the MC GW MBMS bearer quality report.

4.	The MC service server may send an additional proposal for measurements e.g. information about neighbouring MBMS bearers. This message may be an MBMS bearer announcement message.

# /************************** Next changes ****************************/

11.5.4	Disconnection mechanism

11.5.4.1	General

A connection using an MC gateway UE for one or more service(s) can be cancelled over time or re-established using same or another MC gateway UE. The disconnection mechanism allows the MC service user to disconnect the use of the corresponding MC gateway UE considering the various MC client hosting scenarios.

Under certain circumstances, the connection with the corresponding MC gateway UE can change or has to be adjusted. The various reasons are detailed in the informative Annex D. For this purpose, the MC gateway UE can send a notification to the corresponding connection control client function hosted on a non-3GPP device.

11.5.4.2	Disconnection for non-3GPP devices that host an MC client

11.5.4.2.1	General

The clause is applied to non-3GPP devices which can host an MC client. The MC gateway UE forwards the disconnection request to the corresponding MC service server to disconnect the MC gateway UE connection of the MC service user for certain service(s).

11.5.4.2.2	Information flows

11.5.4.2.2.1	Disconnection request

Table 11.5.4.2.2.1-1 describes the information flow disconnection request sent from the connection control client function, which resides on a non-3GPP device, to the connection control GW function in the MC gateway UE and from the connection control GW function in the MC gateway UE to the corresponding MC service server.

Table 11.5.4.2.2.1-1: Disconnection request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID(s) | M | The MC service ID(s) of the requesting MC service user. |
| MC service ID | M (NOTE) | The MC service ID of the MC gateway UE |
| NOTE: Only present on the GW-core reference point between the MC gateway UE and the MC service server. |  |  |

11.5.4.2.2.2	Disconnection response

Table 11.5.4.2.2.2-1 describes the information flow disconnection response sent from the MC service server to the connection control GW function in MC gateway UE, and from the connection control GW function in the MC gateway UE to the connection control client function residing on a non-3GPP device.

Table 11.5.1.2.2.2-1: Disconnection response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requesting MC service user. |
| Response | M | Result of the disconnection request. |
| MC service ID | M (NOTE) | The MC service ID of the MC gateway UE |
| NOTE: Only present on the GW-core reference point between the MC gateway UE and the MC service server. |  |  |

11.5.4.2.2.3	Connection status notification

Table 11.5.4.2.2.3-1 describes the information flow connection status notification sent from the connection control GW function in the MC gateway UE to the connection control client function , which resides on a non-3GPP device.

Table 11.5.4.2.2.3-1: Connection status notification

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID(s) | M | The MC service ID(s) of the associated MC client. (see NOTE 1) |
| Status information | M | This information element provides connection status. (see NOTE 2). |
| NOTE 1: The GW MC service ID indicates for which MC service the connection is to be disconnected. NOTE 2: Information about the connection status are further detailed in Annex D. |  |  |

11.5.4.2.3	Disconnection procedure

The procedure for disconnection via an MC gateway UE towards an MC server is shown in figure 11.5.4.2.3-1.

Pre-conditions

-	The MC service user has an authorized connection via an MC gateway UE to an MC server.

-	The MC clients have no communication ongoing, e.g. group communication.

-	The MC service user on a non-3GPP device wishes to disconnect the authorized connection.

Figure 11.5.4.2.3-1: Disconnection with an MC server via an MC gateway UE

1.	The connection control client function requests disconnection via the MC gateway UE with an MC service server. The connection control client function provides the MC service ID(s) to be disconnected.

2.	The connection control GW function in the MC gateway UE sends the disconnection request to the corresponding MC service server to disconnect the authorized connection of the MC service user for this MC service.

3.	The MC service server verifies if the connection is active and updates the connection status as disconnected.

4.	The MC service server sends the disconnection response to the connection control GW function in the MC gateway UE.

5.	The connection control GW function in the MC gateway UE updates connection status of the MC service user for this MC service as disconnected. After this, the MC gateway UE stop routing service traffic between this user and the MC service server.

6.	The connection control GW function in the MC gateway UE sends the disconnection response to the connection control client function.

11.5.4.2.4	Connection status notification

The procedure for connection status notification initiated by an MC gateway UE towards a connection control client function is shown in figure 11.5.4.2.4-1 informs about the status of connection status that may result into a disconnection.

Pre-conditions

-	The MC service user has an authorized connection via an MC gateway UE to an MC service server.

-	The MC gateway UE is no longer able to provide the requested service depending on reasons further detailed in Annex D.

Figure 11.5.4.2.4-1: Connection status notification to an authorized connection control client function

1.	The connection control GW function in the MC gateway UE wants to disconnect the connection with an MC service server for the corresponding connection control client function. The MC gateway UE sends connection status notification to the connection control client function using the corresponding MC gateway ID(s).

2.	The connection status may result that the connection control client function wants to disconnect the connection with the MC service server (see disconnection in clause 11.5.4.2.3).

# /************************** Next changes ****************************/

11.5.4.3.2.1	Disconnection request

Table 11.5.4.3.2.1-1 describes the information flow disconnection request sent from the MC client, which resides on a MC gateway UE, to the MC server.

Table 11.5.4.3.2.1-1: Disconnection request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID(s) | M | The MC service ID(s) of the requesting MC service user. |
| MC service ID | M | The MC service ID of the MC gateway UE |
| NOTE: The GW MC service ID indicates for which MC service the connection is to be disconnected. |  |  |

11.5.4.3.2.2	Disconnection response

Table 11.5.4.3.2.2-1 describes the information flow disconnection response sent from the MC server to the MC client residing on the MC gateway UE.

Table 11.5.4.3.2.2-1: Disconnection response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID(s) | M | The MC service ID(s) of the requesting MC service user. |
| MC service ID | M | The MC service ID of the MC gateway UE |
| Result | M | Success or failure of the disconnection request (successful/failed; not permitted). |

11.5.4.3.3	Disconnection procedure

The procedure for disconnection of MC service user for a certain MC service(s)is shown in figure 11.5.4.3.3-1.

Pre-conditions

-	The MC service user has an authorized connection using an MC gateway UE to an MC server.

-	The MC clients have no communication ongoing, e.g. group communication.

-	The connection control client function hosted on a MC gateway UE wishes to disconnect the authorized connection.

Figure 11.5.4.3.3-1: Disconnection of an MC client hosted by an MC gateway UE

1.	The connection control GW function, hosted by the MC gateway UE, sends a disconnection request to the corresponding MC server encompassing the MC service ID(s) of the requesting MC service user and the MC service ID of the MC gateway UE.

2.	The MC server verifies if the connection is active and updates the connection status as disconnected.

3.	The MC server sends the disconnection response to the connection control GW function residing on the MC gateway UE.

4.	The connection control client function updates MC service user connection status to disconnected. After this, the MC gateway UE stop routing service traffic between this user and the MC service server.

# /************************** Next changes ****************************/

A.7	Void

# /*********************** Next Changes ****************************/

Annex D (informative):
Consideration MC gateway UE selection

D.1		General

Operating conditions can influence the selection and the use of the MC gateway UE and its response/decision on providing an MC client connection to the MC server.

Such status information can be considered by the MC client either to connect to another MC gateway UE (e.g. with better operating conditions for the requested service), or to re-connect to the same MC gateway UE (e.g. reattempt connection after a certain time).

D.2		Potential Conditions

Possible operating conditions information could belong to one of the following categories:

MC gateway UE capacity limits

The MC gateway UE might not be able to allow MC clients to connect when the limit of connected MC clients has been reached.

3GPP access congestion

The MC gateway UE can make use of to the special Categories/Access Identity, the preferential access applies based on its assigned Access Control Class/Access Identity during periods of congestion. The MC gateway UE subscription allows to obtain priority treatment for the required communication.

Nevertheless, the serving access network can indicate to the UEs (including MC gateway UE) that Access Class Barring applies and this can be passed by the MC gateway UE to the MC client located in the non-3GPP device.

PLMN reselection and migration

Depending on operator policy, roaming agreements, and on national/regional regulatory requirements MC gateway UE connection and reconnection with the corresponding MC server are necessary (e.g. to handle the IP connectivity, QoS, etc.).

Network status information

If the network status information of an EPS/5GS capability available at MC gateway UE, it can be further propagated to the corresponding MC clients located in the non-3GPP device for further processing (e.g. if the requested QoS for service can be provided for MC client or not).