3GPP TSG-SA WG6 Meeting #60	S6-241595

Changsha, China 15th – 19th April 2024	(revision of S6-241416)

Source:	Ericsson

Title:	Satellite edge computing

Spec:	3GPP TR 23.700-01 v0.2.0

Agenda item:	8.6

Document for:	Approval

Contact:	Wenliang Xu (wenliang.xu@ericsson.com)

1. Introduction

This pCR provides solution update for sol#1.

2. Reason for Change

TR 23.700-01 v0.2.0 has a KI#2 related to satellite EDN which has the following open issues for GEO satellite:

Deployment of Edge computing on board GEO satellites might impact the latency of application control and data information exchange. To assess the impacts of using Edge on board GEO satellites it would be benefitial to study the following aspects with the aim to reduce latency and data exchange over satellite feeder link:

- Investigate different deployment options for EDGEAPP when EASs are deployed on board GEO satellites?

- When/how could EAS discovery by the EEC in EDGEAPP be optimized?

TR 23.700-01 v0.2.0 has a KI#5 related to satellite EDN which has the following open issues for NGSO satellite:

-	How the EES(s) are placed on board the Satellite.

-	How the EAS(s) are placed on board the Satellite.

-	Whether and how the discovery and the service provisioning are impacted.​

-	Whether and how the service continuity procedures are impacted while the UE is only connected with NTN.

Therefore, it is needed to study solution to address this key issue.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-01.

* * * First Change * * * *

### 7.2.1	Solution #1: Satellite edge computing

#### 7.2.1.1	Solution description

Editor's Note:	This clause will describe the solution. Each solution should clearly describe which of the key issues it covers and how.

#### 7.2.1.1.1	General

Figure 7.2.1.1.1-1: satellite EDN deployment

In this deployment option, ECS is deployed on ground. The UE can be on the ground/sea or in the air (e.g. drone). The  UPF to access satellite EDN is deployed on satellite, EAS and EES are deployed in one or more satellites. RAN (e.g. gNodeB) can either be deployed on ground/sea (e.g. in a ship) and connected to satellite UPF or be deployed on regenerative satellite (see Annex A and key issue #1 in 3GPP TR 23.700-29 [TR2370029]). The 5GS control plane functions (e.g. AMF, SMF) are deployed on the ground, which is not depicted in the figure for simplicity. EASs and their registered EES are deployed on one or more satellites which constitutes an EDN and these satellites’ coverage areas may correspond to an EDN service area. UPF can be deployed on ground to access the ECS, the EEC (in UE) can reach the ECS by EDGE-4 (on ground or via space).

If there are multiple EESs in a satellite EDN, EEC may trigger EAS discovery towards each EES which increases delay due to EDGE-1 interactions. It is recommended to deploy a single EES per EDN to reduce complexity in satellite.

NOTE:	The UPF and RAN deployment are described above for the sake of clarity which is related to EDN on-board satellite (e.g. EDGE-4 path), see also TS 23.501 [23501] clause 5.43.2 and TR 23.700-29 [TR2370029]).

During initial service discovery, the EEC (in UE) contacts the ECS via EDGE-4 (on ground or via space) to find an appropriate EES, then an appropriate EAS instance is selected during EAS discovery via EDGE-1 interaction. Finally, AC (in UE) communicates with the selected EAS.

Editor's Note:	Other deployment options and associated latency aspects are FFS.

##### 7.2.1.1.2	Further consideration for service provisioning

an external server responsible for providing satellite availability information for each satellite based on real-time satellite orbit information (e.g. eccentricity, inclination, true anomaly) may exist (e.g. see TS 23.501 [TS23501], Annex Q). During service provisioning, the EEC can provide needed satellite capability information (e.g. satellite frequency bands) to the ECS and the ECS will be responsible to monitor UE location and UE moving prediction, and query the external server(s) to get a list of satellites (each identified by satellite ID) that can service the list of waypoints of the UE, then determine a suitable EES with a matching satellite ID to serve the UE with longest service time in service provisioning. Once the ECS identifies the suitable EES, it responds/notifies the EEC with the EES information. In addition, the corresponding ECS selected satellite ID is sent to the EEC.

#### 7.2.1.2	Architecture Impacts

This solution has no architecture impacts to EDGEAPP. Under the existing EDGEAPP architecture, the service provisioning procedure is enhanced to support different cases in service continuity.

#### 7.2.1.3	Corresponding APIs

Editor's note:	This clause provides the corresponding APIs for supporting the solution.

#### 7.2.1.4	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End of Changes * * * *