3GPP TSG-SA WG6 Meeting #60	S6-241255

Changsha, China 15th April – 19th April 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0059 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the identifier of stored data from multiple SEALDD servers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2024-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the mobility scenario, the consumer (e.g. mobile VAL client or mobile SEALDD client) has the requirements to retrieve the same application data from the different SEALDD servers by using the same identifier (e.g. file name or video name). To satifsfy this requirements, for the same application data originated from VAL server, the identifier of the stored data allocated by different SEALDD servers may be same (e.g. using the URI with data name) to support to retrieve application data in mobility scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper proposes to add the description to clarify the identifier of stored data allocated by multiple SEALDD server for the same application data from VAL server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the related clarifications are not added, the mobile VAL/SEALDD client may need to retrieve the same application data from multiple SEALDD servers but using different identifiers, which will increase the complexity in VAL/SEALDD client. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.5.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.5.2.1	Data storage creation

Pre-conditions:

1.	The VAL server has discovered and selected the SEALDD server by CAPIF functions.

2.	The SEALDD client has discovered and selected the SEALDD server as specified in clause 9.4.3.

Figure 9.5.2.1-1: Storage service creation

1.	The VAL client/server determines to use SEALDD storage service, which could store/host data on behalf of the VAL client/server. The consumer (e.g. SEALDD client, VAL server) send a Sdd_DataStorage_Creation request to the SEALDD server. The request includes the data to be stored and information associated with the data, such as access control policy, expiration time of the storage, etc. The consumer may also specify in the request the management or status information of the data storage that is required (e.g., information about how often the stored data is accessed or managed).

NOTE 1:	If the VAL client determines to use SEALDD storage service, the request is first sent to the SEALDD client hosted on the same UE, and then the SEALDD client send the Sdd_DataStorage_Creation request to the SEALDD server.

NOTE 2:	The detailed request between VAL client and SEALDD client is out of scope of this release of this specification.

2.	The SEALDD server checks for the authorization of the storage service creation request. If the request is successfully authorized, then the SEALDD server configures the storage service based on the request and stores the data at the storage server. If status information of the stored data is requested, the SEALDD server will start to monitor the status of the stored data, such as to track the accesses to the data for the data access status.

3.	The SEALDD server sends a response to the requesting consumer (e.g. SEALDD client, VAL server). The response indicates if the request is accepted and the identifier of the stored data (if applicable). Upon receiving the response, the requesting consumer may create a record for the stored data.

NOTE 3:	If the same application data originated from VAL server is stored in multiple SEALDD servers, the identifier of the stored data allocated by different SEALDD servers may be same (e.g. using the URI with data name). The consumer (e.g. mobile VAL/SEALDD client) can retrieve the same application data from different SEALDD servers by using the same identifier of stored data, as specified in clause 9.5.2.3.