3GPP TSG-SA WG6 Meeting #60	S6-241636

Changsha, P. R. China, 15th – 19th April 2024	(revision of S6-241228, S6-241442)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0055 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Functional model update to introduce the SEALDD UU-U and SEALDD S-U |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2024-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently SEALDD-S and SEALDD-Uu contains both the signalling plane and the user plane which have different functions and protocols. In addition, user plane is the essential part of data delivery, it has to be well reflected in the functional model and reference point. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the SEALDD server and SEALDD client descriptions - Update the reference point description to align with (1) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The missing of user plane from the functional model is not well reflect the data delivery concept designed for VAL service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2, 7.3.3, 7.4.2, 7.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************** First Changes ********************/

7.3.2	SEAL Data Delivery server

The SEAL data delivery server functional entity acts as the application server for the data delivery enablement. The SEALDD server supports the following capabilities:

a)	Suppport the signalling interaction with VAL server to negotiation the data delivery aspects including QoS requirement, protocols information, bandwidth settings, delivery policy provisioning, transmission quality measurement.

b)	Support the signalling interaction with the SEALDD client to managemant the data delivery between the SEALDD server and the SEALDD client, including the establish/update/release of signalling plane and user plane of SEALDD-UU, bandwidth control, transmission quality measurement.

c)	Providing the application data/media storage.

d)	Interacting with 5GC via N33/N5 (i.e. send control plane requirements or receive control plane notification) with usage of capability exposed by 3GPP network.

e)	Support user plane handlings including obtaining the application media data and/or application signalling data from VAL server via SEALDD-S, delivering it to the SEALDD client via the SEALDD-UU, and receiving the application media data and/or application signalling data from SEALDD client and providing it to the VAL server via SEALDD-S, enforce the rate control.

7.3.3	SEAL Data Delivery client

The SEAL data delivery client functional entity acts as the application client for the data delivery enablement. The SEALDD client supports the following capabilities:

a)	Interact with the SEALDD server to managemant the data delivery between the SEALDD server and the SEALDD client, including the establish/update/release of signalling and user plane of SEALDD-UU, bandwidth control, transmission quality measurement.

b)	Support user plane handlings including obtaining the application media data and/or application signalling data from VAL client, delivering it to the SEALDD server via the SEALDD -UU, and receiving the application media data and/or application signalling data from SEALDD client, providing it to the VAL server via the SEALDD-S.

# /******************** Next Changes ********************/

7.4.2	SEALDD-UU

SEALDD-UU reference point is between the SEALDD client and the SEALDD server. It supports the user plane functionality of transfering data content and the signalling plane functionality of exchanging information for SEALDD service provisioning, control, reporting etc.

# /******************** Next Changes ********************/

7.4.4	SEALDD-S

SEALDD-S reference point is between the SEALDD server and the VAL server. It supports the user plane functionality of data content delivery and the signalling plane functionality SEALDD service provisioning, control, reporting etc.