3GPP TSG-SA WG6 Meeting #61	S6-242086

Jeju Island, South Korea, 20th – 24th May 2024	(revision of S6-242xxx)

Source:	Huawei, HiSilicon

Title:	pCR on Evaluation and Conclusion of key issue#2

Spec:	3GPP TR 23.700-92 v0.5.0

Agenda item:	8.2

Document for:	Approval

Contact:	wanghan28@huawei.com

1. Introduction

This pCR proposes evaluation and conclusion of key issue#2.

2. Reason for Change

Contribution with new text.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-92 v0.5.0.

* * * First Change * * * *

## 10.3	Solution evaluation

Editor's note:	This clause will provide evaluation of different solutions.

## 10.3.x	Key issue #2: Download data channel application to UE

The open issues of this KI are as follows:

1.	The application layer should support to provide the capability of indicating how to handle the downloading and processing of a specific data channel application on UE based on the service requirement of the specific data channel application and MNO.

2.	The application layer should support to provide and control the capability to data channel application provider to set this indicator.

To address the open issues the following solutions has been proposed:

-	Solution #3: downloading of data channel application profiles to UE

-	Used to provision data channel application profiles to UE via the eMMTel enabler Server from verticals/enterprises to UE. In the procedure, UE needs to request the root application and the profile of the application, then the eMMTel enabler Server sends the root application and the profile of the application to UE. The eMMTel enabler Server cannot actively push root application and the profile of the application to the UE when necessary.

-	Solution #4: configuration of DC application profile

-	The eMMTel Enabler Server provides APIs to the Controlling Application Server to manage the necessary information needed in eMMTel service. in the procedure, the Controlling Application Server upload the DC Application Profile to the eMMTel Enabler Server before provision it to UE, this solution is part of provision data channel application profiles to UE procedure.

-	Solution #5: updating of data channel application profiles to UE

-	Used to provision data channel application profiles to UE via the eMMTel enabler layer from verticals/enterprises to UE. The eMMTel enabler layer can actively push root application and the profile of the application to the UE when necessary, this solution is a supplement to solution#3.

Solution #3 and Solution #5 covers open issues#1 of KI#2, and Solution#4 covers open issues#2 of KI#2. All three solutions are complementary, Solution #5 is a supplement to Solution #3, Solution #4 is a pre-condition for a complete solution for Solution #3 and Solution #5, Both solutions can be considered in the normative work.

* * * Next Change * * * *

# 11	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the study item activities.

The study concludes with the following solution considerations for the normative work:

1.	Following individual solutions, corresponding to the key issues, will be considered as candidate solutions:

For key issue#2 (downloading of data channel application profiles to UE)

The solution#3, solution#4 and solution#5 will be considered in the normative phase and used as baseline.

* * * End of Changes * * * *