3GPP TSG-SA WG6 Meeting #61	S6-242328

Jeju Island, South Korea, 20th - 24th May 2024	(revision of S6-2422239)

Source:	Apple, AT&T

Title:	Backend For Frontend reference points

Spec:	3GPP TR 23.700-22 v0.2.0

Agenda item:	8.7

Document for:	Approval

Contact:	Walter Featherstone, w_featherstone@apple.com

1. Introduction

This paper proposes to highlight the required capabilities of the CAPIF-8 & CAPIF-X reference point introduced through solution 1 of 23.700-22, clause 6.2 on Backend For Frontend (BFF) based on the discussion in contribution S6-242238.

2. Reason for Change

Address the editor’s notes in clause 6.2.1 regarding the required capabilities for CAPIF-8 & CAPIF-X.

3. Proposal

It is proposed to update solution 1 of TR 23.700-22 v0.2.0 to provide the required capabilities of CAPIF-8 & CAPIF-X.

* * * First Change * * *

## 6.2	Solution #1: Backend For Frontend

### 6.2.1	Solution description

This solution relates to KI#3 on RNAA architecture enhancements. Specifically, it proposes the Backend For Frontend (BFF) pattern (as described, for example, in clause 6.1 of IETF draft-ietf-oauth-browser-based-apps-17 [5]) to be included as an architectural option.

In 3GPP TS 23.222 [2] clause 7.5, the RNAA deployment option in which the API invoker is deployed on a UE is presented. A UE can be considered as hosting public clients according to the OAuth 2.0 Authorization Framework [6]. Such public clients lack the ability to maintain the confidentiality of secrets such as their client credentials or even access tokens provided to them. Regarding this vulnerability, as an example, clause 5.1 of IETF draft-ietf-oauth-browser-based-apps-17 [5] highlights the danger of malicious JavaScript in relation to browser-based applications (highlighted as an example of a user-agent-based application public client in the OAuth 2.0 Authorization Framework [6]). This IETF draft presents the BFF application architecture pattern as an approach in which applications are built to rely on a backend component for handling OAuth responsibilities and then all requests are proxied through this backend component. Such an architectural approach is applicable to CAPIF as described in the next sub-clause.

NOTE:	The security aspects of the CAPIF architectural variant described in this solution are the responsibility of SA3.

### 6.2.2	Architecture Impacts

In relation to the OAuth 2.0 Authorization Framework [6] the CAPIF resource owner function (responsible for managing authorization related procedures towards the CAPIF authorization function hosted by the CAPIF core function) can be considered as the user-agent (aligned with clause 6.3.8 of 3GPP TS 23.222 [2]), which in turn manages procedures towards the authorization server in OAuth. Furthermore, the CAPIF resource owner function is responsible for interactions with the resource owner, which is anticipated to be in a similar manner to the way in which interactions between the resource owner’s user agent are described by the OAuth framework.

With OAuth there is also a client (or client application) that interacts with the user-agent and authorization server in order to gain access to information exposed by a resource server.

The OAuth resource server is considered as the CAPIF API exposing function, aligned with clause 6.5.3.1 of 3GPP TS 33.122 [3].

The OAuth client is considered as the CAPIF API invoker (aligned with clause 6.5.3.1 of 3GPP TS 33.122 [3]), where that API invoker can be UE hosted as previously highlighted.

Then in this solution (as presented in Figure 6.2.2-1) it is proposed to support an option in which the API invoker hosted by a UE (API invoker frontend) interacts via a new reference point (CAPIF-X) with a “helper” API invoker (API invoker backend) that is placed server-side (network-side) and is considered alongside the Backend For Frontend component in the Application Architecture presented in clause 6.1.1 of IETF draft-ietf-oauth-browser-based-apps-17 [5].

According to that IETF draft, the BFF:

1.	Interacts with the authorization server as a confidential client

2.	Manages access and refresh tokens and does not share those with the client, i.e., the tokens are inaccessible to the client

3.	Proxies all requests from the client to the resource server, augmenting them with the correct access token before forwarding them to the resource server

In the IETF draft, an OAuth user-agent-based application is considered in which the client application runs within the user-agent. On the other hand, for an OAuth native application, the user-agent and client application are presented as separate entities, which is also the case the architectural solution proposal in Figure 6.2.2-1.

Figure 6.2.2-1: High level functional architecture for CAPIF supporting BFF

NOTE:	Impacts to the reference points with their assocaited procedures between the API provider domain functions and the CAPIF core function are not anticipated (and hence have not been depicted in Figure 6.2.2-1).

Editor's note:	Whether the reference point and procedures between the resource owner function and API invoker are in scope of 3GPP is TBD.

Figure 6.2.2-2: BFF based procedure for service API access

Figure 6.2.2-2 illustrates the BFF based procedure. Firstly, in step 1, the UE hosted API invoker (API invoker frontend) initiates the authorization flow, where the API invoker’s objective is to gain access to particular resources offered through the service APIs exposed by the API provider domain.

In step 2, the API invoker backend, in response to the trigger from the API invoker frontend to initiate the authorization flow, provides a response to the API invoker frontend redirecting its request towards the CCF authorization function (CCF(AuthF) step 3).

Once the response is obtained indicating granted authorization (step 4), the resource owner function provides indication of that authorization to the API invoker backend (step 5).

Next in step 6, the API invoker backend requests access to the API exposing function’s service APIs from the authorization function using the obtained authorization.

The API invoker backend maintains an association between its session with API invoker frontend and access information it has obtained in step 7 from the authorization function.

The API invoker backend then provides indication to the API invoker frontend that service API access is permitted, e.g., through cookies using a secure HTTP session, in step 8. The indication provided by the API invoker backend does not include the access information necessary to enable the API invoker frontend to access the service APIs directly.

When the API invoker frontend subsequently initiates a service API request including the access indication information previously by the API invoker backend (step 9), the API invoker backend uses its granted service API access that it received from the authorization function (that is associated with the API invoker frontend’s resource owner function to API invoker backend session) to make the request towards the service API (step 10).

The response from the service API (step 11) is then forwarded towards the API invoker frontend by the API invoker backend (step 12).

### 6.2.3	Corresponding APIs

CAPIF-8 (ROF – CCF(AuthF))

- Authorization request, from ROF to CCF(AuthF) (step 3, clause 6.2.2)

- Granting authorization, from CCF(AuthF) to ROF (step 4, clause 6.2.2)

CAPIF-Y (ROF – API invoker backend)

- Initiate authorization request, from API invoker frontend to API invoker backend via ROF (step 1, clause 6.2.2)

- Initiate authorization response from API invoker backend to ROF redirecting ROF to CCF(AuthF) (step 2, clause 6.2.2)

- Forward authorization, from ROF to API invoker backend (step 5, clause 6.2.2)

CAPIF-X (API invoker frontend – API invoker backend)

- Access granted indication, from API invoker backend to API invoker frontend (step 8, clause 6.2.2)

- API service request, from API invoker frontend to API invoker backend (step 9, clause 6.2.2)

- API service response, from API invoker backend to API invoker frontend (step 12, clause 6.2.2)

Editor's note:	The CAPIX-8, CAPIF-X and CAPIF-Y related procedures are FFS

, although those may leverage OAuth 2.0 Authorization Framework [6].

* * * End of Changes * * *