3GPP TSG-SA WG6 Meeting #61	S6-242353

Jeju Island, South Korea, 20th – 24th May 2024	(revision of S6-242161)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0359 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IP address(es) of target data host(s) or server(s) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Kontron Transportation France, UIC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh4MCPTT |  |  |  |  |  | Date: |  |  | 2024-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The session charactaristics of the IP connection are not known by both the origination and terminating data hosts. Figure 5.11-1: IP connectivity model |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding SDP offer and SDP answer to the MCData IPcon point-to-point request/response. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Originating data host cannot route IP packets to target data host(s) or server(s) using MCData transport service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.14.2.1.1, 7.14.2.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of change * * * *

##### 7.14.2.1.1	MCData IPcon point-to-point request

Table 7.14.2.1.1-1 describes the information flow of the MCData IPcon point-to-point request sent from the MCData client to the MCData server.

Table 7.14.2.1.1-1: MCData IPcon point-to-point request (MCData client to MCData server)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData identity of the originator MCData user; |
| Functional alias | O | The associated functional alias of the originator MCData user; |
| MCData ID | O (NOTE 2) | The MCData identity of the target MCData client IP connectivity is requested. |
| Functional alias | O (NOTE 2) | The functional alias of the target MCData client. |
| SDP offer | M | Offered media parameters describing the requested characteristics of the IP tunnel between the MCData UEs. |
| Requested Priority (NOTE 3) | O | Application priority level requested for this communication. |
| Location Information | O (NOTE 1) | Actual location information of the originating MCData user; |
| Time Limit | O | Proposed time limit of the requested IP connectivity (1min- infinite); |
| Establishment reason | O | IP connectivity establishment reason |
| NOTE 1: This information contains the latest available location information of the requesting MCData user that may be different to the latest available location information in the MC system. NOTE 2: Either the MCData ID or the functional alias of the target MCData user must be present. NOTE 3: The predefined priority of the MC service user is applied by the MCData server if the requested priority is not present or not accepted by the MCData server. |  |  |

Table 7.14.2.1.1-2: MCData IPcon point-to-point request (MCData server to MCData client)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData identity of the originator MCData user; |
| MCData ID | M | The MCData identity of the target MCData client IP connectivity is requested. |
| SDP offer | M | Offered media parameters describing the requested characteristics of the IP tunnel between the MCData UEs. |
| Location Information | O (NOTE 1) | Actual location information of the originating MCData user; |
| Time Limit | O | Proposed time limit of the requested IP connectivity (1min- infinite); |
| Establishment reason | O | IP connectivity establishment reason |
| NOTE 1: This information contains the latest available location information of the requesting MCData user. |  |  |

##### 7.14.2.1.2	MCData IPcon point-to-point response

Table 7.14.2.1.2-1 describes the information content of the MCData IPcon point-to-point response as answer to MCData IPcon point-to-point request.

Table 7.14.2.1.2-1: MCData IPcon point-to-point response

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData identity of the targeted MCData user. |
| MCData ID | M | The MCData identity of the requesting MCData user. |
| SDP answer | O | Media parameters selected. This shall be present if the IP connectivity establishment result is successful. |
| Time Limit | O | Negotiated time (1 min – infinite) |
| IP connectivity status | M | IP connectivity establishment result |

# * * * * End of change * * * *