3GPP TSG-SA WG6 Meeting #61	S6-242624

Jeju, KR, 20th – 24th May 2024

Source:	Ericsson, KPN N.V., Convida Wireless LLC

Title:	Key Issue#7. Overall Evaluation

Spec:	3GPP TS 23.700-82 V0.4.0

Agenda item:	8.3

Document for:	Approval

1. Introduction

This pCR provides the overall evaluation of the KI#7.

2. Reason for Change

The overall evaluation of the KI#7 is required to complete the evaluation stage.

3. Conclusions

This pCR proposes the overall evaluation of the KI#7.

4. Proposal

It is proposed to agree the following changes to 3GPP 23.700-82 V0.4.0.

* * * First Change * * * *

### 11.X7	Key issue#7: Discovery or Support of Member Selection and Maintenance for Application Layer AIML Service

The open issues studied in the Key Issue#7 are as follows:

1.	How to support the AI/ML for member selection and re-selection (e.g., policies).

2.	How to support the AI/ML member participation configurations.

3.	How to support the AI/ML maintaining the AI/ML process.

4.	How to utilize the AI/ML policies and configurations in the AI/ML Application layer Services.

This clause provides an overall evaluation of the key issue #7. The Solution#2, Solution#3, Solution #6, Solution #7,Solution #10, Solution#14, and Solution#30 cover different aspects for the open issues in the KI#7.

Solution#6 and Solution#14 address the open issues 1, 3 and 4 by introducing the AI/ML policies provisioning and management (solution#14) with the AIML policies enforcement for AIMLE Clients selection and re-selection (solution#6). Solution#6 and Solution#14 are complementary, i.e., Solution#14 provisions the AI/ML policies and Solution#6 utilises the provisioned AIML policies for the AIMLE clients (re-)selection. Thus, the Solution#6 and Solution#14 can be considered as candidates for normative work and recommended to be merged into one API (e.g, AIMLE Client Discovery and Selection) in the normative phase.

Solution#7 addresses open issues 1 and 2 via introduction of the AIMLE Client discovery for further selection. The Solution#7 can be considered as candidate for normative work and recommended be a part of the AIMLE Client Discovery and Selection API in the normative phase.

Solution#2 address the open issue 1 via introduction of the ML client information retrieval procedure that is useful for for member selection and re-selection. Steps 1, 2a and 2b of Solution#2 is recommended be merged to the procedure of Solution#7.

Solution#10 addresses open issue 2 via introduction the ML mode agnostic (i.e., applicable for registration and configuration provisioning for FL, DML, ML, Transfer and/or Split Learnings) configuration provisioning during AIMLE Client registration defined in Solution#8. Solution #3 addresses the open issue 2 via introduction of the member participation configurations for the FL operations. Thus, Solution#3 is covered by Solution#10, and Solution#10 can be merged with Solution#8 (e.g., the member participation configurations will be merged with AIML client profiles) and be considered as candidate for normative work. It is recommended to specify Solution#10 merged with Solution#8 in the AIMLE Client Registration API.

Solution#30 addresses open issue#3 by introducing the support of the intermediate AI/ML operation information transfer between AIMLE Clients in order to maintain the continues Transfer Learning process. The solution can be considered as a candidate for normative work.

# *** Next Change ***

## 12.Y7	Conclusions of key issue #7

Following solution considerations will be considered for the normative work for KI #7:

-	Solution#6 and Solution#14 are complementing each other and can be considered for the normative work. It is recommended to specify Solution#6 and Solution#14 as a part the AIMLE Client Discovery and Selection API.

-	Solution#2 can be merged into Solution#7. Solution#7 can be considered for the normative work. It is recommended to specify Solution#7 as a part of AIMLE Client Discovery and Selection API.

-	Solution#3 covered by Solution#10, and Solution#10 can be merged with Solution#8. Solution#8 merged with Solution#10 can be considered for the normative work. It is recommended to specify Solution#8 merged with Solution#10 in the AIMLE Client Registration API.

-	Solution#30 can be considered for the normative work.

# *** End of Changes ***