3GPP TSG-SA6 Meeting #61	S6-242767

# Jeju, Korea (Republic Of), 20th May 2024 - 24th May 2024	(revision of 2650, 2107)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0642 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | API requester identifier |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Trading |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2024-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the UE Identifier API request an EAS can request for the AF specific UE identifier for a list of EASs, however which is its own EAS identifier is not explicitly identified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to include the requesting EASID if the request originates from an EAS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This enhancement enables the EES to easily verify if the EAS is requesting a UE identifier on behalf of itself. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Originally it had been proposed requester identifier should be required for UE location requests, however the EES is able to use its own identifier when interacting with the core network when requesting UE location and therefore EASID is not strictly necessary. Impacts to the EELManagedACR service request are also reversed. |  |  |  |  |  |  |  |  |

Change 1

##### 8.6.5.3.2	UE Identifier API request

##### Table 8.6.5.3.2-1 describes information elements for the UE Identifier API request from the EAS or EEC to the EES.

Table 8.6.5.3.2-1: UE Identifier API request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identifier (NOTE 4) | O | Identifier of the requestor (i.e. EASID or EECID). |
| User information (NOTE 1) (NOTE 3) | O | Information about the User or UE available in the EAS or EEC, e.g. IP address. |
| UE ID (NOTE 2) (NOTE 3) | O | UE ID in the form of GPSI as per clause 7.2.6. |
| EAS ID list (NOTE 4) | O | Identifier of the EAS(s) for which the UE IDs are requested for by EAS or EEC given the User information (e.g. IP address). |
| EAS Provider ID | O | Identifier of the ASP that provides the EAS. |
| Application Port ID (NOTE 5) | O | Application Port ID, as defined in 3GPP TS 23.502 [3], associated with the EAS. |
| Security Credentials | M | Security credentials of the EAS or EEC. |
| NOTE 1: This IE is Mandatory when EAS invoke the UE ID API. When EEC invokes the API, if available, this IE contains both UE’s private IPv6 address (due to the existence of NAT66) and UE’s private IPv4 address. When EAS invokes the API, it may recognize the UE IP address is a public IP address different from the actual UE IP address (private IP address), i.e., the UE is behind a NAT, and should therefore include the Port Number and associated IP address as part of the User information. NOTE 2: This IE is used when invoked by the EEC and if the EEC have the UE ID already in a form not desired to be shared with the EAS. NOTE 3: At least one of them shall be present. NOTE 4: This IE is Mandatory when EAS invoke the UE ID API. NOTE 5: This IE may only be present when EAS invoke the UE ID API. |  |  |

Editor's note: [SA3] It is FFS whether the IP address (i.e. User information) provided in the request can be verified that is indeed the UE’s and whether the UE provided information (IP Address) can be trusted.