3GPP TSG-SA WG6 Meeting #65	S6-250473

Athens, Greece 17th – 21st February 2025	(revision of S6-250248)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0259 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Discover service APIs without onboarding |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Proposes solution as per conclusion of Key Issue#5 in TR 23.700-22. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In TR 23.700-22, conclusions of Key Issue #5 states: Solution#22 can be considered as candidate for normative work for enhancements related to discover service APIs for improving the information to be provided depending on the entity and scenario. To provide to the API Invoker an improved experience prior to on-boarding, a new API may be defined during normative phase. Solution#25 is an enhancement to solution#22, whose specific aspects can be considered together with solution#22 during normative phase. The CR proposed is using the approach proposed by solution#25 i.e. to introduce new service API operation and not modifying existing Discover Service API operation along with the details covered by solution#22. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support for necessary service API operation will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (All new) 8.x, 8.x.1, 8.x.2, 8.x.2.1, 8.x.2.2, 8.x.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /************************ First change ******************************/

## 8.x	Open Discover service APIs

### 8.x.1	General

The following procedure in this subclause corresponds to supporting open discover service APIs. Such open discovery operation allows API invokers to discover service API information about the available set of APIs offered by CCF before onboarding.

### 8.x.2	Information flows

#### 8.x.2.1	Open Service API discover request

Table 8.x.2.1-1 describes the information flow open service API discover request from the API invoker to the CAPIF core function.

Table 8.x.2.1-1: Open Service API discover request

| Information element | Status | Description |
| --- | --- | --- |
| API invoker information | O (NOTE 1) | API invoker information who is yet to be onboarded |
| Query information | O (NOTE 2) | Criteria for discovering matching service APIs. It may contain Service API information described in table 8.3.2.1-1, except service API interface details. |
| NOTE 1: API invoker information may be absent to indicate API invoker is not yet onboarded. NOTE 2: It should be possible to discover all the service APIs. |  |  |

#### 8.x.2.2	Open Service API discover response

Table 8.x.2.2-1 describes the information flow open service API discover response from the CAPIF core function to the API invoker.

Table 8.x.2.2-1: Open Service API discover response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the open discovery of the service API information |
| Service API information | O (NOTE 1) | Filtered service APIs information that can be shared to the API Invoker yet to be onboarded. It may contain Service API information described in table 8.3.2.1-1, except service API interface details. |
| NOTE 1: The service API information shall be present if the Result information element indicates that the open service API discover operation is successful. Otherwise, it shall not be present. |  |  |

### 8.x.3	Procedure

Figure 8.x.3-1 illustrates the procedure for open discover service APIs.

The open service API discovery mechanism is supported by the CAPIF core function.

Pre-conditions:

1.	The API invoker is yet to be onboarded and not a recognized user of the CAPIF.

Editor's note:	The access credentials the API provider domain provides to the API invoker to enable the discovery without onboarding is specified by SA3.

2.	The CAPIF core function is aware of the Service API information that can be disseminated according to the entity requesting e.g. API invoker information and the invocation of the API invoker discovery without onboarding.

Figure 8.x.3-1: Open Discover service APIs

1.	The API invoker sends an open service API discover request to the CAPIF core function. It may include the API invoker information who is yet to be onboarded and the query information.

2.	Upon receiving the open service API discover request, the CAPIF core function is aware that the API invoker is yet to be onboarded. The CAPIF core function retrieves the stored service API(s) information as per the query information in the open service API discover request. Further, since the API invoker is yet to be onboarded, the CAPIF core function considers the entity requesting to perform filtering of service APIs information.

3.	The CAPIF core function sends an open service API discover response to the API invoker with the filtered list of service API information.

# /************************ End of changes ******************************/