3GPP TSG-SA WG6 Meeting #67	S6-252496

Fukuoka, Japan 19th – 23rd May 2025	(revision of S6-252148, 2426)

Source:	Lenovo

Title:	Architectural Assumptions and Principles

Spec:	3GPP TR 23.700-83 v0.1.0

Agenda item:	9.4

Document for:	Approval

Contact:	Emmanouil Pateromichelakis <epateromiche@lenovo.com>

1. Introduction

This paper proposes the introduction of asssumptions and principles for the AIML_App_ph2 study.

2. Reason for Change

This paper proposes the introduction of asssumptions and principles for the AIML_App_ph2 study.

3. Proposal

It is proposed to agree the new key issue for 3GPP TR 23.700-83 v 0.1.0.

* * * * First Change * * * *

# 4	Architectural Assumptions and Principles

## 4.1	Analysis of relation between AIMLE and ADAE services

In this clause an analysis of the relation of ADAE analytics and AIMLE potential service on ML model inference is discussed. Initially a definition is provided about the possible role of AIMLE vs ADAES in inference and analytics:

1.	Analytics as exposed by ADAE, does not necessarily rely on AI/ML models. The consumer simply requests analytics and receives insights—without needing to know or specify how those insights are derived. Thus, when requesting analytics a consumer does not provide any information in relation to the ML model even if such model is used as it is abstracted away from it.

2.	Inference, on the other hand, is fundamentally tied to the use of a specific ML model. The consumer must either provide a model, model information or define criteria for selecting a model. This makes the ML model an integral part of the inference service. While inference can be used to support analytics, it does not itself expose analytics as a service.

To capture this distinction, the following analysis is provided in Table 4.1-1.

Table 4.1-1 Analysis of ADAE vs AIMLE

|  | Analytics (ADAE) | Inference (AIMLE) |
| --- | --- | --- |
| Definition | Derivation of statistical or predictive insights from collected data. | Execution of a trained ML model to produce predictions or classifications. |
| Purpose | Provide application-layer insights (e.g., performance, usage, QoS) to VAL servers and clients. | Provides AI/ML operations such as model training, inference, and task transfer. |
| Processing | May involve statistical methods or ML-based analytics. | Uses trained models to infer outcomes from input data. |
| Model Awareness | Abstracted from the VAL server. | VAL server must specify or select the model. |
| Information Elements | Analytics ID, Analytics Type, Area of Interest, Time Validity, Confidence Level, Reporting Requirements. | Either ML model, ML model information or ML model requirement information is mandatory, along with AI/ML Task Type, QoS, Target Clients, Time Window. |

## 4.2	Architectural Assumptions

The following high-level assumptions and principles will be considered in this study for ADAES:

1.	ADAES is the entity providing analytics which can be either ML-enabled or not

2.	If ADAES analytics is ML-enabled, the ML training capability is provided by AIMLE (Rel-19 AIMLAPP),

3.	If ADAES analytics is ML-enabled, the ML inference capability can be provided as part of the analytics functionality inside ADAES (internal to ADAES logic),

The following high-level assumptions and principles will be considered in this study for AIMLE:

1.	AIMLE provides ML support capabilities as of Rel-19 for both VAL and other SEAL servers (Rel-19 AIMLAPP)

2.	AIMLE provides ML model training capability for both VAL task and ADAE analytics (Rel-19 AIMLAPP)

3.	AIMLE is not expected to provide ML model inference for tasks related to ADAE analytics

4.	AIMLE can only be seen as a candidate entity for performing ML model inference for VAL/ASP automation tasks (needs to be discussed further within Rel-20 study)

The following high-level assumptions and principles will be considered in this study for ML repository:

1.	The ML repository is the unique repository / registry per SEAL provider for the application-layer ML models and the parameters required for the AIMLE operations (Rel-19 AIMLAPP).

- Different deployments and cardinalities of ML repository within the SEAL provider’s domain will be studied as part of Rel-20 work.

* * * * End of Change * * * *