3GPP TSG-SA WG6 Meeting #68	S6-253022

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253xxx)

Source:	Netherlands Police

Title:	Pseudo-CR on Scenario for Discreet monitoring of MCPTT and MCVideo group calls

Spec:	3GPP TR 23700-37-020

Agenda item:	9.1

Document for:	Approval

Contact:	keesdotverweijatpolitiedotnl

1. Introduction

A scenario detailing MCPTT and MCVideo group calls is added to clarify the usecase and identify techical impact. The text originates from TR 23.784, but has been enhanced as required.

2. Reason for Change

Scenario for MCPTT and MCVideo group calls is missing.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23700-37-020 v 0.2.0.

* * * First Change * * * *

## 4.3	Scenario 2:	Discreet monitoring of MCPTT and MCVideo group calls

This scenario describes the case where authorized MC user A requests discreet monitoring for communications involving MC user B where MC service user B is within the authority of authorized MC user A, where MC user A and MC user B have the same primary MC system, MC system A. MC user B is a member of MC service group X together with MC users C to N, and therefore the communications in MC service group X are to be provided to MC user A. The primary MC system of MC service group X is also MC system A. The scenario is illustrated in figure 4.3-1 below.

Figure 4.3-1: Discreet monitoring of MCPTT or MCVideo group call

The scenario consists of the following aspects:

-	Authorized MC user A identifies MC user B as the target for discreet monitoring. MC system A identifies that MC user A is authorized to perform discreet monitoring on MC user B. This aspect takes place before any communications involving MC user B can be subject to discreet monitoring.

-	MC system A identifies MC user B as a member of MC service group X

-	A group member in MC service group X (one of MC users B or C to N) makes an MCPTT or MCVideo group call.

-	MC system A provides MC service client A with information concerning the initiation of the call and provides the media exchanged throughout the group call, including the identity of the calling MC users.

-	When the group call completes, the call completion is conveyed to MC service client A.

NOTE 1:	The scenario does not determine whether authorized MC user A is a member of MC service group X or not.

NOTE 2:	The scenario also applies to other variants of group call, such as emergency group call, imminent peril group call, broadcast group call and calls to temporary groups where MC user B is a group member.

NOTE 3:	If any of MC users B or C to N is receiving MC service from a different MC system from authorized MC user A, or if the group host MC system is different from the MC system serving authorized MC user A, the scenario is similar to that described in subclause 4.2.2 for private call with different MC systems.

* * * End of Changes * * * *