3GPP TSG-SA WG6 Meeting #68	S6-253115

Gothenburg, Sweden 25th – 29th August 2025

Source:	Huawei, Hisilicon

Title:	Key issues: how to utilize sensing results for HD map in V2X

Spec:	3GPP TR 23.700-15 v0.0.0

Agenda item:	9.13

Document for:	Approval

Contact:	Lei Chen (chenlei79@huawei.com)

1. Introduction

This contribution proposes a key issue for how to utilize sensing results for HD map in V2X.

2. Reason for Change

This contribution is to disscuss potential key issues of utilizing sensing results for V2X.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS / TR <TS/TR number and version>.

* * * First Change * * * *

## 4.x Key issue #x: how to utilize sensing results for HD map in V2X

According to TS23.286, the V2X application specific server can be responsible for managing HD maps and providing the HD map information to the V2X application specific client on V2X UE. As per a proximity range set by the application layer, the VAE layer support providing the dynamic information (i.e. location information) required for HD maps management to the V2X application specific server. During this procedure, VAE needs to invoke location management service to get dynamic UEs list and further obtain their location. However, in curren transportation system, there are many traditional vehicles which may be not equiped with UE module, or GNSS module, so it is diffcult to localize their location by location management service provided by SEAL. Morover, the environment information around vehicle (e.g., pedestrian/animal objects) also is important for safe operation, but there is no an efficient method to obtain those informantion.

According to TS22.137, the 5G system shall be able to provide sensing service to detect, and/or track one or more objects and the environment around the object(s). This service could be used to enhance HD map service to provide the location information of traditional vehicles and environament information aroud host vehicle to improve system safety.

Hence, it would make sense to study how to utilize sensing results for HD map in V2X. For this, at least the following aspects should be considered:

- To enhance VAE layer to provide enhanced HD map with considering sensing results

- VAE layer how to obtain sensing results.

Editor’s note: VAE layer how to oabtain sensing results needs to wait  for SA2 progress

* * * END of CHANGE * * * *