3GPP TSG-SA WG6 Meeting #69	S6-254117

Wuhan, China 13th – 17th October 2025	(revision of S6-25xxxx)

Source:	Telefónica, Apple, Fogus, UMA

Title:	TR 23.947 Overview of Frameworks Introduction

Spec:	3GPP TR 23.947

Agenda item:	9.14

Document for:	Approval

Contact:	w_featherstone@apple.com

1. Introduction

This contribution provides the Overview of Frameworks Introduction text for the Guidelines for 3GPP Application Enablement usage (AppEnable_EXT) technical report.

2. Reason for Change

This contribution proposes text for the Overview of Frameworks Introduction of 3GPP TR 23.947.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.947, version 0.1.0.

* * * First Change * * * *

## 4.1	Introduction

Emerging 5G vertical services – such as connected vehicles (V2X), unmanned aerial systems (UAS), and smart manufacturing – demand low-latency, high-bandwidth, and context-aware processing at the network edge. To address these needs, 3GPP SA6 has developed several application enablement frameworks, each targeting a specific aspect of the application layer architecture:

- Common API Framework (CAPIF) – provides a unified, secure way to expose and consume service and network APIs across multiple domains and stakeholders. CAPIF offers a harmonized approach for API development and exposure, allowing 3GPP network functions and even third parties to publish and discover APIs in a common environment.

- Edge Application Enablement (EDGEAPP) – specifies an architecture for deploying and orchestrating applications at the network edge, including support for local service discovery, application lifecycle management, and continuity during user mobility. EDGEAPP enables edge computing with minimal changes to existing applications, abstracting the edge infrastructure complexity behind standardized interfaces.

- Service Enabler Architecture Layer (SEAL) – defines a layer of reusable service enablers (e.g. location, group communications, messaging) available to vertical applications via standardized APIs. By providing common functions that many industries require, SEAL allows different vertical applications to leverage the same core capabilities (for example, a group communication service or location service) instead of each vertical implementing its own.

This Technical Report provides an overview of how these frameworks work together and guides application developers on using them to build advanced services. Clauses 4.2 to 4.4 describe the architectural role, key components, and interactions of CAPIF, EDGEAPP, and SEAL, respectively, including how they relate to one another. In later sections, illustrative use cases and flows will demonstrate how an application can simultaneously leverage CAPIF for API exposure, EDGEAPP for edge computing support, and SEAL for common vertical services.

* * * End of Changes * * * *