3GPP TSG-SA WG6 Meeting #69	S6-254204

Wuhan, China 13th – 17th October 2025	(revision of S6-25xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0703 | rev | - | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Provide User Info ID during MC service user authentication procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MultiHop-MC |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR introduces the possibility of the MC service client to share the User Info ID associated with a 5G ProSe UE-to-Network enabled MC service UE or an MC capable 5G ProSe relay. The User Info ID is utilized to refer to the identity of the 5G ProSe UE-to-Network relay and 5G ProSe intermediate UE-to-Network relay as defined in 3GPP TS 23.304. By sharing the User Inof ID, the MC service server is capable of demanding the MC service client corresponds to the User Info to report the quality of its connection as part of the overall quality of indirect connection of a remote MC service UE under consideration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a sentence in step 3 where the MC service client of a ProSe enabled UE shall include its User Info ID during third party registration towards the MC service server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement in clause 6.9.2.2 in TS 22.261 is not met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 23.289 CR XXX |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 10.6.1	Primary MC system

NOTE:	Figure 10.6.1-1 is a high level user authentication and authorization flow. 3GPP TS 33.180 [25] defines the specific user authentication and authorization architecture required by the MC services in order to realize the MC service user authentication and authorization requirements as defined in 3GPP TS 22.280 [3].

The user authentication process shown in figure 10.6.1-1 may take place in some scenarios as a separate step independently from a SIP registration phase, for example if the SIP core is outside the domain of the MC service server.

A procedure for user authentication is illustrated in figure 10.6.1-1. Other alternatives may be possible, such as authenticating the user within the SIP registration phase.

Figure 10.6.1-1: MC service user authentication and registration with Primary MC system, single domain

1.	In this step the identity management client begins the user authorization procedure. The MC service user supplies the user credentials (e.g. biometrics, secureID, username/password) for verification with the identity management server. This step may occur before or after step 3. In a MC system with multiple MC services, a single user authentication as in step 1 can be used for multiple MC service authorizations for the user.

2.	The signalling user agent establishes a secure connection to the SIP core for the purpose of SIP level authentication and registration.

3.	The signalling user agent completes the SIP level registration with the SIP core, and an optional third-party registration with the MC service server(s). If location information is obtained from the PLMN operator via the LCS, the MC service client shall share the MC service UE’s IMSI to the MC service server during the third party registration. For L4S capabilities defined in IETF RFC 9330 [37], the MC service client shall include the MC service UE’s capabilities related to L4S during the third-party registration with the MC service server. Furthermore, for ProSe enabled MC service UE, the MC service client shall share the User Info ID associated with the MC service UE during the third-party registration with the MC service server.

NOTE:	The MC service client(s) perform the corresponding MC service authorization for the user by utilizing the result of this procedure.

# * * * * End of changes * * * *