3GPP TSG-SA WG6 Meeting #69	S6-254222

Wuhan, P.R.China, 13th – 17th October 2025	revision of S6-25xxxx

Source:	Ericsson

Title:	New Solution for KI#2, 3, and 4 on Application AIoT Discovery and Monitoring Services

Spec:	3GPP TR 23.700-26 v0.1.0

Agenda item:	9.8

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

As described in Key Issue #2 (TR 23.700-26 clause 6.1), the study aspects on Exposing the value-added information of AIoT devices to the consumer include:

- How to enrich the processing (e.g. filtering, categorization, consolidation) of AIoT service requests and the collected Ambient IoT device data;

- How to utilize the exposure from other working group (e.g. SA2) and provide the value-added AIoT devices information to the consumer (e.g. VAL server) based on open issue #1.

In Key Issue #3 (TR 23.700-26 clause 6.3), the study aspects on provisioning and monitoring AIoT device presence include:

- How to provision information about AIoT devices requiring monitoring in the application enablement layer.

- How concurrent AIoT device monitoring operations can be managed in the application enablement layer considering existing CN AIoT exposure capabilities.

- Whether and how enhanced AIoT device monitoring information, such as status, location or trajectory of Ambient IoT devices, can be provided by the application enablement layer.

In Key Issue #4 (TR 23.700-26 clause 6.4), the study aspects on AIoT Data Management include:

- How to align the inventory result data according to the desired application semantics and formats.

- How to configure, manage and expose the conversion of inventory result data according to the vertical application requirements, such as ERP(enterprise resource planning system), MES(Management Execution System) or WMS(warehousing management system).

The existing services on AIoT inventory and AIoT demand cannot satisfy the application requirements on the aspects listed in the above key issues. For example, based on current exposure results from 5G core network as specified in 3GPP TS 23.369, only the AIoT devices ID will be provided for the inventory request, and cannot identify the AIoT devices in service granularity and indicate when one type of device will reach the pre-defined threshold to perform the increase or decrease.

Thus, this pCR proposes new services on AIoT discovery and monitoring based on the application enablement architecture in S6-254221 to support AIoT applications.

2. Reason for Change

This pCR proposes new services to address the study aspects in Key Issues #2, #3, and #4, on AIoT discovery and monitoring to support AIoT applications.

3. Conclusions

This paper proposes solution on new services on AIoT discovery and monitoring to support AIoT applications.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-26 v0.1.0.

* * * First Change * * * *

## 7.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |
| Sol #2 |  |  |  |  |
| Sol #Y |  | X | X | X |

* * * Next Change * * * *

## 7.Y	Services on Application AIoT Discovery and Monitoring

### 7.Y.1	Solution Description

The existing services on AIoT inventory and AIoT demand cannot satisfy the application requirements on the aspects listed in the above key issues. For example, based on current exposure results from 5G core network as specified in 3GPP TS 23.369 [3], only the AIoT devices ID will be provided for the inventory request, and cannot identify the AIoT devices in service granularity and indicate when one type of device will reach the pre-defined threshold to perform the increase or decrease.

The following clauses specify AIoT discovery and monitoring services for supporting AIoT applications.

#### 7.Y.1.1	Procedure for Application AIoT Discovery

Figure 7.Y.1.1-1: Application AIoT Discovery

- A consumer (e.g., VAL server) sends application AIoT discovery request to AIOTE server. The request information includes VAL service ID, Area of interest (e.g., physical location, logical location (e.g., warehouse, factory, supper market)), Application AIoT device information (e.g., list of application layer objects (e.g., goods in a warehouse) which correspond to a list of application AIoT devices), Number of AIoT devices, Requirements on categorization (e.g., location-based, object type-based), Requirements on discovery (e.g., count number of objects in specific location, count number of specific objects), and Time interval.

- The AIOTE server authenticates and authorizes the request.

- If authorized, the AIOTE server performs categorization of the AIoT devices in the list provided in step 1 according to the requirements on categorization and determines the AIoT inventory request to 5GC NF (i.e. NEF/AIOTF) for each of the categories. Based on the list of AIoT devices in a category, the AIOTE determines the Filtering Information to be used in the inventory request.

- The AIOTE server sends the AIoT inventory requests according to the determination in step 3 to the 5GC NF (NEF or AIOTF), for obtaining the AIoT device IDs and location of each AIoT device for each of the category.

- (Optional) The AIOTE server may collect AIoT device information from the 3rd party AIoT server.

- The AIOTE server maps the received AIoT device IDs into objects for each category and processes the received information according to the requirements on discovery, e.g., count the number of objects in specific location, count the number of specific object type.

- The AIOTE server sends application AIoT discovery response to the consumer, with the required discovery information, e.g., number of objects in specific location (e.g., different area of a warehouse), count the number of specific object type (e.g., specific goods in the warehouse).

#### 7.Y.1.2	Procedure for Application AIoT Monitoring

Figure 7.Y.1.2-1: Application AIoT Monitoring

- A consumer (e.g., VAL server) sends application AIoT monitoring request to AIOTE server. The request information include VAL service ID, Area of interest (e.g., physical location, logical location (e.g., warehouse, factory, supper market)), Application AIoT device information (e.g., list of application layer objects (e.g., goods in a warehouse) which correspond to a list of application AIoT devices), Number of AIoT devices, Requirements on categorization (e.g., location-based, object type-based), Requirements on monitoring (e.g., monitor the number of objects in specific location (e.g., number of goods in a warehouse), the expired time of the objects (e.g., the expired time of the goods in a supermarket)), Time interval for monitored data processing, and Reporting requirements.

Reporting requirements could be Periodic reporting (frequency of the reporting), or Event-triggered reporting (report when a threshold is exceeded or change occur).

For threshold-based report, Threshold(s) (e.g., number(s) of objects in specific locations, time threshold(s) for specific objects) is included in the request.

For flexible report (i.e., change occur and report), the event could be, for example, number of specific type object change compared with previous check (e.g., increase or decrease in given location (e.g., Goods with the given category are in or out the given location)) or status of object change (e.g., expired time is approached/reached (e.g., Goods are close to be expired with given time range per goods category)). Frequency for check the changes of object (e.g., number, status (may align with the time interval for monitored data processing)) are included in the request.

- The AIOTE server authenticates and authorizes the request.

- The AIOTE server responds to the application AIoT monitoring request, with subscription ID.

- If authorized, the AIOTE server performs categorization of the AIoT devices in the list provided in step 1 according to the requirements on categorization, determines the operations according to the requirements on monitoring in step 1 (e.g., AIoT inventory for monitoring the number changes of objects, AIoT command-read for monitoring the status of object), and schedules periodic AIoT inventory and/or read command.

- Based on the determinations in step 4, the AIOTE server performs corresponding operations for monitoring.

5a. The AIOTE server may send AIoT inventory request(s) to 5GC NF (NEF/AIoTF) to obtain the AIoT device IDs in given location.

5b. The AIOTE server may send AIoT command request(s) to 5GC NF (NEF/AIoTF) to read the AIoT data and location information.

- (Optional) The AIOTE server may collect AIoT device information from the 3rd party AIoT server.

- Based on the received information in steps 5 and 6, and requirements in step 1, the AIOTE server processes the data, and determines whether need to send notification to the consumer after each round of monitored data processing, e.g., report periodically, report the changes of object number, report the changes of object status (e.g., expired time approached/reached).

- The AIOTE server sends application AIoT monitoring notification to the consumer, with the required monitoring information.

#### 7.Y.1.3	Information Flows

Editor’s Note:	The information flows of the solution is FFS.

### 7.Y.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 7.Y.3	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

# *** End of Changes ***