3GPP TSG-SA WG6 Meeting #69	S6-254225

Wuhan, P.R.China, 13th – 17th October 2025	revision of S6-25xxxx

Source:	Ericsson

Title:	New Solution for KI#1 on New Service on Support Energy-Related Data Collection

Spec:	3GPP TR 23.700-44 v0.1.0

Agenda item:	9.9

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

As described in Key Issue #1 (TR 23.700-44 clause 5.1), the study aspects on enhance application enablement layer architecture and services to support energy saving include:

- Whether enhancements to existing application enablement layer services or a new application enabler service is required to support energy savings operations in the application enablement layer.

According to the use cases and comment functions identified in S6-254224, service on energy-related data collection is needed for supporting energy saving. This pCR proposes new Energy Saving Enablement (ESE) service to support energy-related data collection.

2. Reason for Change

This pCR proposes a new solution for Key Issue #1 by introducing a new Energy Saving Enablement (ESE) service to support energy-related data collection.

3. Conclusions

This paper proposes solution on new Energy Saving Enablement (ESE) service to support energy-related data collection.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.1.0.

* * * First Change * * * *

## 6.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI#5 | KI#6 |
| --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |  |  |
| Sol #2 |  |  |  |  |  |  |
| Sol #X1 | X |  |  |  |  |  |

* * * Next Change * * * *

## 6.X1	Solution #X1: New Service on Support Energy-Related Data Collection

### 6.X1.1	Solution Description

According to the use cases and comment functions identified in Solution #X, service on energy-related data collection is needed for supporting energy saving.

The following clauses specify the procedure and information flow of ESE (Energy Saving Enablement) service on energy-related data collection for support energy saving.

#### 6.X1.1.1	Procedure

Figure 6.X1.1.1-1: Energy-related data collection

- A consumer (e.g., VAL server) sends energy-related data collection request to ESE server. The request includes information as described in Table 6.X1.1.2.2-1.

- The EAME server authenticates and authorizes the request. If authorized, the ESE server determines the data source (e.g. EIF, ESE client, and/or other network entities (e.g. OAM, 3rd party energy entity)) for collecting the energy-related data. It is assumed that ESE server has a registry with the data sources from which it can collect energy-related data and what kind of data they can provide.

- The ESE sends energy-related data collection response to the consumer. The response includes information as described in Table 6.X1.1.2.3-1.

4.	The ESE server collects energy-related data from the data source.

4a.	The ESE server may interact with 5GC NF for energy-related data collection, by using the EIF service introduced in 3GPP TS 23.501 [3].

4b.	The ESE server may collect energy-related data from other network entities (e.g. other SEAL server/client, OAM, and/or 3rd party energy entity).

4c	The ESE server may collect energy-related data from EAME client(s).

5.	The ESE server aggregates the collected energy-related data. Aggregation includes combining or performing a statistical operation on the received data, and elimination of data duplications. If necessary, steps 4-5 are repeated to complete all the required data collection.

6.	The ESE server notifies the consumer the energy-related data collection result. The notification includes information as described in Table 6.X1.1.2.4-1.

#### 6.X1.1.2	Information Flows

##### 6.X1.1.2.1	General

The following information flows are specified for energy-related data collection.

##### 6.X1.1.2.2	Energy-related data collection request

Table 6.X1.1.2.2-1 shows the request sent by an ESE service consumer to an ESE server for energy-related data collection procedure.

Table 6.X1.1.2.2-1: Energy-related data collection request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| VAL service ID | M | The identity of the VAL service for which the request applies. |
| Target entity ID(s) | M | The identity(ies) of the target data collection entity(ies) (e.g. VAL UE ID(s), SEAL server ID(s)) |
| Data collection requirements | M | Identifies the requirement for energy-related data collection. |
| >Energy source type | O (NOTE 1) | Identifies the requirement on energy source to be collected (e.g. non-renewable energy, renewable energy). |
| >Energy consumption | O (NOTE 1) | Identifies the requirement on energy consumption to be collected (e.g. energy consumption per application, energy consumption per task operation, per energy source type). |
| >Energy status | O (NOTE 1) | Identifies the requirement on energy status to be collected (e.g. total remaining energy, remaining energy per energy source). |
| >Data volume | O (NOTE 1) | Identifies the requirement on collection of data volume of the application (e.g. uplink, downlink, or both, start and end time, or time interval). |
| >Time of Interest | M | Indicate the interested time for the data collection. |
| Area of Interest | O | The geographical or service area for which the request applies. |
| Reporting requirements | O | It describes the requirements for reporting. This requirement may include e.g. the type of reporting (periodic or one-time report), the reporting periodicity in case of periodic. |
| Data processing requirements | O (NOTE 2) | Indicates the requirements on data processing, as defined in Table 6.X1.1.2.2-2. |
| Additional info | O | Identifies addition requirement of the data to be collected (e.g. time stamp of the data, quality of the data). |
| NOTE 1: At least one of the IEs shall be provided. NOTE 2: The IE shall be provided if requires service on energy-related data combination/aggregation. |  |  |

Table 6.X1.1.2.2-2: Data processing requirements

| Information element | Status | Description |
| --- | --- | --- |
| Processing type | M | Identifies the processing type, e.g. raw-data, aggregation. |
| Aggregation requirements | O | Identifies the requirements for aggregation based on the energy-related data. |
| >Average | O (NOTE) | Indicates that calculate the average value of the energy-related data. |
| >Minimum | O (NOTE) | Indicates that calculate the minimum value of the energy-related data. |
| >Maximum | O (NOTE) | Indicates that calculate the maximum value of the energy-related data. |
| Granularity | M | Identifies the granularity for the processing, e.g. per entity level, or per application level, or per application operation level, per energy source type. |
| NOTE: At least one of the IEs shall be provided if the Processing type is calculation. |  |  |

##### 6.X1.1.2.3	Energy-related data collection response

Table 6.X1.1.2.3-1 shows the response sent by the ESE server to the consumer for energy-related data collection procedure.

Table 6.X1.1.2.3-1: Energy-related data collection response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the request (positive or negative acknowledgement). |
| Subscription ID | O | Identifier of the subscription. It shall be provided if the Result is positive acknowledgement. |

##### 6.X1.1.2.4	Energy-related data collection notify

Table 6.X1.1.2.4-1 shows the notification sent by the ESE server to the consumer for energy-related data collection procedure.

Table 6.X1.1.2.4-1: Energy-related data collection notify

| Information element | Status | Description |
| --- | --- | --- |
| Success status | M (NOTE 1) | Indicates that data collection was successful. |
| >Data collection result | O (NOTE 2) | Provides outputs for data collection result, it may also include the additional information of the collected data based on request. |
| >Aggregated result | O (NOTE 3) | Provides outputs for aggregated result, it may also include the additional information of the collected data and processing information based on request. |
| Failure response | O (NOTE 1) | Indicates that data collection was failed. |
| >Cause | M | Reason for the failure. |
| NOTE 1: One of the information elements shall be provided in the output. NOTE 2: The IE shall be provided if no processing requirements in the Energy-related data collection request. NOTE 3: The IE shall be provided if processing requirements are included in the Energy-related data collection request. |  |  |

### 6.X1.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.X1.3	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

# *** End of Changes ***