3GPP TSG-SA WG6 Meeting #69	S6-254227

Wuhan, P.R.China, 13th – 17th October 2025	revision of S6-25xxxx

Source:	Ericsson

Title:	New Solution for KI#2 on Energy Monitoring and Exposing in Application Enablement Layer

Spec:	3GPP TR 23.700-44 v0.1.0

Agenda item:	9.9

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

This pCR proposes a solution for the key issue #2.

2. Reason for Change

A solution to address the key issue #2 on monitoring and exposure of energy consumption information within the application enablement layer is proposed in this pCR, based on the application enablement architecture ESE (Energy Saving Enablement) introduced in S6-254224.

3. Conclusions

A solution for energy monitoring is proposed to address the key issue #2.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.1.0.

* * * First Change * * * *

## 6.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI#5 | KI#6 |
| --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |  |  |
| Sol #2 |  |  |  |  |  |  |
| Sol #x |  | X |  |  |  |  |

* * * Next Change * * * *

## 6.x	Solution #x: Monitoring and exposing energy consumption within the application enablement layer

This solution corresponds to the key issue #2 described in TR 23.700-44, on monitoring and exposure of energy consumption information within the application enablement layer.

### 6.x.1	Solution Description

According to key issue #2, there is a need for a service that can monitor and expose the energy information. This solution provides an energy saving monitoring service, that can be used by the consumer (e.g. VAL server, SEAL server) to receive information regarding energy consumption per application/UE, which in turn can be useful for energy savings. The solution includes the energy monitoring notification for periodic, and event-based reporting. The event for monitoring includes; when the budget threshold value for energy consumption is reached and when there is an anomaly detection in the energy consumption.

#### 6.x.1.1	Energy monitoring procedure

Figure 6.x.1.1-1: Energy monitoring subscription and notification

1.	The consumer (e.g. VAL server, SEAL server) sends an Energy Monitoring Subscription request to the ESE Server with an energy monitoring profile that may contain energy reporting requirements.

2.	The ESE Server checks if the consumer is authorized for the energy monitoring subscription for the requested target VAL UEs.

3.	The ESE Server obtains the energy reporting requirements for energy monitoring profile requested by the consumer and apply for subscription for energy information with data source (e.g., EIF, SEAL server/client).

4.	The ESE Server sends to the consumer with an energy monitoring subscription response to indicate the subscription status.

5	The ESE Server receives notifications about the energy information from the data source for the corresponding energy monitoring profile.

6.	The ESE Server aggregates the energy consumption related information from the data source and determines whether to send a notification to the consumer based on the consumer monitoring reporting requirements. The reporting can be periodic, or event triggered.

7.	The ESE Server sends an energy monitoring notification to the consumer by including the subscription identifier and the aggregated energy information for the list of VAL UEs.

The consumer may adjust the network requirements (e.g, QoS requirements) to the VAL client for energy saving, based on the energy monitoring data provided by the ESE server.

#### 6.x.1.2	Energy monitoring Information elements

##### 6.x.1.2.1	Energy monitoring subscription request

Table 6.x.1.2.1-1 shows request sent by the consumer (e.g. VAL server, SEAL server) to the ESE Server for the energy monitoring subscription.

Table 6.x.1.2.1-1: Energy monitoring subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Consumer ID | M | The identifier of the consumer (e.g. VAL server, SEAL server). |
| Target VAL UE ID(s) | M | List of VAL UE(s) for the energy monitoring subscription applies. |
| Target VAL service ID | O | The identifier of the VAL service for which the energy monitoring applies. |
| Target Slice ID | O | The slice ID (S-NSSAI) for which the energy monitoring applies. |
| Energy Monitoring profile | M | Represents the energy monitoring profile. |
| > Data Source | O (NOTE 1) | List of data source (e.g., EIF, SEAL server/client). |
| > Energy Consumption Category | O (NOTE 1) | Represent the category(s) of energy consumption (e.g, AIML, Data Delivery, SliceOAM). |
| Reporting requirements | O | It describes the requirements for energy consumption data reporting. This requirement may include e.g. the type and frequency of reporting (periodic or event triggered), the reporting periodicity in case of periodic, and reporting thresholds in case of event triggered. |
| > Reporting periodicity | O | It describes the Frequency of reporting is periodic. |
| > Reporting Event Triggered | O | It describes the event triggered reporting. |
| >> Reporting energy threshold event | O (NOTE 2) | Indicates the reporting threshold with the corresponding threshold reaching direction (e.g, corresponding energy source type exceeds the limit, slices accumulated energy consumption excess, slice average energy consumption excess, average energy consumption (eg, per hour) excess, maximum renewable energy excess, slice renewable energy ratio excess). |
| >> Reporting energy anomaly event | O (NOTE 2) | Indicates the anomaly events for energy consumption or usage (e.g, spike in energy consumptions/usage, high-consumption for a longer interval, black-out). |
| Time validity | O | The time validity of the subscription request. |
| NOTE 1: At least one of the IEs shall be present. NOTE 2: Only one of the IEs shall be present. |  |  |

##### 6.x.1.2.2	Energy monitoring subscription response

Table 6.x.1.2.2-1 shows response sent by the ESE Server to the consumer (e.g. VAL server, SEAL server) for the energy monitoring subscription procedure.

Table 6.x.1.2.2-1: Energy monitoring subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status | M | It indicates the subscription result. |
| Subscription ID | O | Represents the identifier of the subscription. This IE shall be present if the subscription status is success. |

##### 6.x.1.2.3	Energy monitoring notification

Table 6.x.1.2.3-1 shows the notification sent by the ESE Server to the consumer (e.g. VAL server, SEAL server) for the energy monitoring procedure.

Table 6.x.1.2.3-1: Energy monitoring notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Represents the identifier of the subscription. This IE shall be present if the subscription status is success. |
| List of VAL UE IDs | O | List of VAL UEs whose energy monitoring data is provided. This list can contain either the same number of VAL UEs provided in the subscription request or a less number. |
| Slice ID | O | The network slice whose energy monitoring data is provided. |
| Energy monitoring result | M | Energy monitoring result is an aggregation of energy consumption related data collected from the data source. |

### 6.x.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.x.3	Solution evaluation

Editor’s Note:	The solution evaluation of the solution is FFS.

* * * End of Changes * * * *