3GPP TSG-SA WG6 Meeting #69	S6-254238

Wuhan, China, 13th – 17th October 2025	(revision of S6-25xxxx)

Source:	Huawei, Hisilicon

Title:	Solution of monitoring AIoT device presence

Spec:	3GPP TR 23.700-26 v0.1.0

Agenda item:	9.8

Document for:	Approval

Contact:	Cuili Ge – gecuili@huawei.com

1. Introduction

This solution aims to address the issues identified in Key Issue 3 and provide a method to montior AIoT devices presence to minimize the service impact.

2. Reason for Change

Monitoring AIoT devices is a crucial method to ensure the security, reliability, and efficient operation of the entire AIoT system. By tracking the status or information of devices in real time, potential issues can be promptly identified or risks can be prevented, enabling quick responses and minimizing the impact on upper-layer application services.

3. Conclusions

From the perspective of system stability and controllability, it is necessary to add services for monitoring AIoT devices in the application enablement layer.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-26 v0.1.0.

* * * First Change * * * *

7.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #x |  |  | Y |  |

* * * Next Change * * * *

7.x	Solution #x: monitoring AIoT device presence

7.x.1	Solution description

This solution address key issue#3 to introduce a method to montior AIoT devices presence.

To achieve precise AIoT device existence monitoring, the core reliance lies in defining the scope of "existence" rather than a single definition. This solution proposes to have following presence related event/status:

- (1)	AIoT Device Connection Status: Although the SA2 specifies an inventory service for discovering the identities of all or part of the AIoT devices within a specific range, it is not real-time, meaning it cannot provide real-time information on whether an AIoT device is "currently" online. Between two inventory services, a device may disconnect due to reasons such as moving out of the specified area, poor signal, or battery depletion. Overall, the inventory service confirms the reachability of the device at the cellular network level but does not concern itself with whether the applications on the device are running normally. It cannot distinguish between an AIoT device being "online but with an application crash" and "everything is normal," which is a very necessary judgment basis for the application side. Therefore, the application enablement layer must confirm the connection status of AIoT devices based on this through application layer heartbeats.

The heartbeat messages from the application layer enablement monitoring of AIoT devices can include the following processes:

- Heartbeat Initiation: AIoT devices periodically trigger the generation of application-layer heartbeat messages, which include device identifiers, timestamps, and other information.

- Data Transmission: The heartbeat messages are forwarded through base stations and the core network to the application enablement layer.

- Heartbeat Processing: The application enablement layer parses and verifies the heartbeat information, updating the last online time of the device.

- Response Return: The application enablement layer returns a heartbeat response, confirming successful reception.

- Status Monitoring: The application enablement layer records the "last heartbeat timestamp" for each AIoT device and periodically checks the device status based on a preset "heartbeat timeout" (such as twice the heartbeat interval). Devices that fail to report a heartbeat within the timeout period are marked as offline, and an offline alert is triggered.

- (2)	AIoT Mobility: To meet the dynamic management needs of AIoT devices in scenarios such as warehouses, logistics hubs, and power plants, relying solely on the online status of devices is no longer sufficient to meet actual application requirements. Customers need to have real-time awareness of changes in the physical location of devices. This necessitates that the 5G core network supports positioning capabilities in AIoT service operations and ensures that the application enablement layer can convert raw positioning data into location identifiers that align with application semantics, thereby better monitoring the presence of AIoT devices.

- Positioning Capabilities: The application enablement layer must open standardized positioning interfaces. By integrating on-site engineering parameter information and performing geographic registration based on electronic maps or spatial models, it can convert raw positioning data such as location coordinates, Tracking Areas (TAI), and Time Difference of Arrival (TDOA) into location identifiers with application semantics (e.g., "Warehouse A - Loading Area 3"), thereby providing third-party applications with intuitive physical locations of AIoT devices.

- Location Update Reporting: To enable customers to better monitor real-time location changes of AIoT devices, the application enablement layer needs to support services that query the current location information of AIoT devices. Additionally, when the location information of an AIoT device changes, it must immediately invoke the communication interface with third-party applications and update the message about the location change of the AIoT device.

- Location Trajectories: The movement trajectory of AIoT devices is a dynamic extension of static location information, which can intuitively reveal the behavior patterns and anomalies of AIoT devices. By analyzing trajectories, it is possible to identify and warn of abnormal device events, optimize device movement paths, and more. Furthermore, in many scenarios, third-party applications do not require real-time location information and can generate movement trajectories from historically reported static location information. When it is necessary to view the movement of AIoT devices, trajectory information can be provided to assist users in monitoring.

- (3)	AIoT Device application Status: Typically, third-party platforms are unable to comprehend the raw information received from AIoT devices. Therefore, it is necessary for the application enablement layer to uniformly process this raw data. By pre-setting certain judgment criteria based on actual application scenarios, the system can promptly push notifications to third-party applications when status changes or anomalies are detected. For instance, in a warehousing scenario, when the location of a transport vehicle changes, real-time location information is immediately pushed to third-party applications, informing them that "xx has moved from Area A to Area B." If the transport vehicle moves outside the designated area and the application enablement layer does not receive any reported location data within a certain period, it can determine, based on historical data, that the vehicle is no longer within the coverage area. Consequently, it informs the third-party application that "xx has left the warehouse or is lost."

7.x.2	Architecture Impacts

This solution does not involve any impact on the architecture.

7.x.3	Corresponding APIs

A new Presence monitor service is introduced.

7.x.4	Solution evaluation