3GPP TSG-SA WG6 Meeting #69	S6-254347

Wuhan, China 13th – 17th Oktober 2025	(revision of S6-254xxx)

Source:	Netherlands Police

Title:	Pseudo-CR on

Spec:	3GPP TR 23700-37-030

Agenda item:	9.1

Document for:	Approval

Contact:	keesdotverweijatpolitiedotnl

1. Introduction

This pCR introduces a key issue on the different aspects of group membership of the authorized MC for discreet monitoring. The text originates from TR 23.784, but has been enhanced as required.

2. Reason for Change

This key issue should be addressed in the study.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23700-37-020 v 0.3.0.

* * * First Change * * * *

## 5.x	Key issue x:	Group membership of authorized MC user for discreet monitoring

The target of discreet monitoring may be a member of an MC service group and be able to take part in group communications. The authorized MC user will expect to receive all MC service group communication transmissions involving the target MC user however the authorized MC user might not be defined as an MC service group member of the MC service group in which the target MC user is communicating.

Issues:

-	Whether the authorized MC user should be a pre-defined member of an MC service group in order to receive group communications in which the target MC user is involved.

-	Whether the authorized MC user should be temporarily made an MC service group member in order to receive group communications in which the target MC user is involved.

-	Whether any restrictions on requiring MC service group membership are different when the MC service group is defined in a different primary MC system to that of the authorized MC user.

-	If the authorized MC user affiliates to the MC service group in order to receive the media in group communications, the affiliation of the authorized MC user to the MC service group may be visible to the target MC user and to other members of the MC service group; and the stage 1 requirement is that discreet monitoring takes place without the knowledge of the target MC user.

-	How the authorized MC user is able to recover the content of communications within the group that are protected by end-to-end encryption.

* * * End of Changes * * * *