3GPP TSG-SA WG6 Meeting #69	S6-254357

Wuhan, China 13th – 17th Oktober 2025	(revision of S6-254xxx)

Source:	Netherlands Police

Title:	Pseudo-CR on Key Issue DM Limitations due to regulatory constraints and operator security policies

Spec:	3GPP TR 23700-37-030

Agenda item:	9.1

Document for:	Approval

Contact:	keesdotverweijatpolitiedotnl

1. Introduction

This pCR adds a key issue about how to handle limitations due to regulatory constraints and operator security policies in Discreet monitoring. The text originates from TR 23.784, but has been enhanced as required.

2. Reason for Change

This key issue should be addressed in the study.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23700-37-020 v 0.3.0.

* * * First Change * * * *

## 5.x	Key Issue x:	Limitations on discreet monitoring due to regulatory constraints and operator security policies

An authorized MC user may request discreet monitoring either on a target MC user or a target MC group. Regulatory constraints and operator security policies can require not to provide media and metadata of transmissions from MC service users who are communicating with the discreet listening target MC service user, and who are not themselves targets of discreet listening.

Issues:

-	How only the communication transmissions coming from the target MC user involved in a private communication can be part of discreet monitoring.

-	How only the communication transmissions coming from one target MC user involved in a group communication can be part of discreet monitoring.

-	In a group regrouping scenario of a target MC group: How only the communication transmissions coming from members affiliated to the target MC group can be part of discreet monitoring.

-	How any other communication transmissions are made unavailable to the authorized MC user, as they are not part of discreet monitoring requests.

-	Whether any of the communication related signalling should be restricted.

* * * End of Changes * * * *