3GPP TSG-SA WG6 Meeting #69	S6-254516

Wuhan, China 13th – 17th Oct 2025	(revision of S6-25xxxx)

Source:	CATT

Title:	New Solution on support of energy saving for EDGE services

Spec:	3GPP TR 23.700-44 v0.1.0

Agenda item:	9.9

Document for:	Approval

Contact:	Liping Wu <wuliping@cictmobile.com>

1. Introduction

The contribution proposes a new solution to support the energy saving for EDGE services.

2. Reason for Change

To proposes a new solution to support the energy saving for EDGE services.

3. Proposal

It is proposed to agree the following changes for 3GPP TR 23.700-44 v0.1.0.

* * * First Change * * *

6.x	Solution #x: Support of energy saving for EDGE services

6.x.1	Solution description

This solution addresses the KI#1 and KI#3.

The following are the procedures which describe how the EAS registered the energy related information to the EES (clause 6.x.1.1) and how the EAS to be discovered considering the energy related information (clause 6.x.1.2).

6.x.1.1	Procedure of EAS registration on energy related information

The following procedure illustrates the high-level procedure of the EAS registration to the EES about the energy related information (e.g., the energy consumption information), which is updated based on the clause of 8.4.3.2.2 of 3GPP TS 23.558 [23558].

The enhancements for the existing procedure are all highlighted in bold italic.

8.4.3.2.2 EAS registration Pre-conditions: 1. The EAS has been configured with an EASID; 2. The EAS has been configured with the address (e.g. URI) of the EES; and 3. Both the EAS and EES have the necessary credentials to enable communications. Figure 8.4.3.2.2-1: EAS Registration procedure 1. The EAS determines that registration to the EES is needed (e.g. the EAS is instantiated and started up). 2. The EAS sends an EAS registration request to the EES. The request shall include the EAS profile and may include proposed expiration time for the registration and its energy related information (e.g., energy consumption information). NOTE: The EAS may get the energy related information from the 3rd parties or from OAM as described in 3GPP TS 28.552 [5] and 3GPP TS 28.554 [6]. 3. The EES performs an authorization check to verify whether the EAS has the authorization to register on the EES. 4. Upon successful authorization, the EES stores the EAS Profile for later use (e.g. for serving EAS discovery requests received from EECs, etc.) and replies to the EAS with an EAS registration response. If the request includes bundle ID, the EES stores the received information. The EES may provide an expiration time to indicate to the EAS when the registration will automatically expire. To maintain the registration, the EAS shall send a registration update request prior to the expiration time. If a registration update request is not received prior to the expiration time, the EES shall treat the EAS as implicitly de-registered.

The enhancements for the existing information flow are all highlighted in bold italic as follows.

8.2.4 EAS Profile An EAS Profile includes information about an EAS used to describe services and service characteristics offered. NOTE: Information elements in the EAS Profile are provided by the ASP. Table 8.2.4-1: EAS Profile Information element; Status; Description. EASID; M; The identifier of the EAS. EAS Endpoint; M; Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to ACs so that ACs can establish contact with the EAS.. List of EAS bundle information; O; List of EAS bundles to which the EAS belongs and related bundling requirements.. > Bundle ID (NOTE 3); O; Bundle ID as described in clause 7.2.10.. > List of EAS IDs (NOTE 2, NOTE 3); O; List of the EAS IDs of the EASs to be invoked by the EAS for an EAS driven association of EASs.. > Bundle type; M; Type of the EAS bundle as described in clause 7.2.10. > Main EASID; O; Indicate which EAS in a bundle takes the main EAS service role.. > EAS bundle requirements; O; Requirements associated with the EAS bundle as described in clause 8.2.10.. ACID(s); O; Identifies the AC(s) that can be served by the EAS. EAS Provider Identifier; O; The identifier of the ASP that provides the EAS.. Allowed MNO information (NOTE 4); O; Information of the allowed operator (e.g. MNO name, PLMN ID) from which its subscriber can consume the EAS services. EAS Type; O; The category or type of EAS (e.g. V2X, UAV, application enabler). EAS description; O; Human-readable description of the EAS. EAS Schedule; O; The availability schedule of the EAS (e.g. time windows). EAS Geographical Service Area; O; The geographical service area that the EAS serves. ACs in UEs that are located outside that area shall not be served.. EAS Topological Service Area; O; The EAS serves UEs that are connected to the Core Network from one of the cells included in this service area. ACs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1.. EAS Service KPIs; O; Service characteristics provided by EAS, detailed in Table 8.2.5-1. EAS service permission level; O; Level of service permissions e.g. trial, gold-class supported by the EAS. EAS Feature(s); O; Service features e.g. single vs. multi-player gaming service supported by the EAS. EAS content synchronization support; O; Indicates if the EAS supports content synchronization between EASs.. EAS Service continuity support; O; Indicates if the EAS supports service continuity or not. This IE indicates which ACR scenarios are supported by the EAS, also indicates the EAS ability (e.g. EAS bundle information) of handling bundled EAS coordinate ACR.. EAS Transport layer service continuity support; O; This IE indicates the EAS service continuity support for seamless transport layer (e.g. TCP/TLS/QUIC) relocation. General context holding time duration (NOTE 1); O; The time duration that the EAS holds the context before the AC connects to the EAS in case of ACR for service continuity planning. It is an indication of the time the EAS holds the application context for a UE to move to its service area after receiving an ACR notification from the EES following an ACR request from the EEC.. List of EAS DNAI(s); O; DNAI(s) associated with the EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN where the EAS resides.. List of N6 Traffic Routing requirements; O; The N6 traffic routing information and/or routing profile ID corresponding to each EAS DNAI.. EAS Availability Reporting Period; O; The availability reporting period (i.e. heartbeat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration.. EAS Status; O; The status of the EAS (e.g. enabled, disabled, overload warning etc.). List of associated devices; O; List of associated devices (e.g. haptic device, joy stick) required along with UE in order to provide service to the user. It includes device type.. Energy related information; O; The energy related information (e.g., energy consumption information) for the EAS.. NOTE 1: Since the EASID of the EAS identifies the type of the application (e.g. SA6Video, SA6Game etc) as described in clause 7.2.4, "General context holding time duration" determined by EAS can depend on the EASID (type of the application). NOTE 2: This IE may be provided when only bundle ID is provided, and the bundle type indicates the proxy bundle. NOTE 3: At least one of the IEs shall be present if EAS bundle information is provided. NOTE 4: For edge node sharing scenario, in order to restrict the access to the subscriber of the partner operator, this IE should only include MNO information of the leading operator..

6.x.1.2	Procedure of EAS discovery based on energy related information

Figure 6.x.1.2-1 illustrates the high-level procedure of EAS discovery considering the energy related information.

NOTE 1:	The following procedure also applies for the EAS discovery subscription/subscription update procedures.

Pre-condition:

-	It’s assumed the EMS (Energy Management Server) is a new enabler in the application enabled layer which collects the energy related information (e.g., energy consumption information) per different granularity (e.g., node-level, service-level) for different services.

Figure 6.x.1.2-1: Procedure of EAS discovery based on energy related information

- Similar with step 1 of clause 8.5.2.2 in 3GPP TS 23.558[23558], the EEC decides to send an EAS discovery request to the EES. The enhancement for this solution is the EAS discovery request not only includes the parameters as defined in the Table 8.5.3.2-1 of 3GPP TS 23.558[23558], but includes the EAS energy related information in the EAS discovery filters and the energy saving indicator which indicate the EES to consider the energy consumption information when selecting the EAS.

- Similar with step 1 of clause 8.5.2.2 in 3GPP TS 23.558[23558], the EES checks if the EEC is authorized to discover the requested EAS(s).

If the request is authorized, the EES will consider selecting an EAS or a group of EASs that meet the service requirements (e.g., energy saving indicator, EAS discovery filters).

-	If the requested EAS has provided its energy related information (e.g., energy consumption information) to the EES as described in clause 6.x.1.1, the EES will select a proper EAS or a group of EASs that meet the energy related service requirements (e.g., the energy consumption information threshold).

-	If the requested EAS is not previously registered on EES, or the registration information has expired, the EES may ask the OAM or the EMS to obtain the energy related information for the requested EAS as described in step 3a and step 3b respectively.

3a.	The EES may obtain the energy related information for the requested EAS from the OAM as described in clause 6.7.3.1 of 3GPP TS 28.554 [6].

3b. The EES may obtain the energy related information for the requested EAS from the EMS.

Editor’s note：How the EMS obtains and calculates the energy related information (energy consumption information) is FFS.

- Upon receiving the energy related information for the requested EAS from different resources (e.g., target EAS, OAM or EMS), if the energy saving indicator is included in step 1, the EES may select a proper EAS or a group of EASs that meet the energy related service requirements (e.g., the energy consumption information threshold).

- Similar with step 3 of clause 8.5.2.2 in 3GPP TS 23.558[23558], the EES sends an EAS discovery response to the EEC, including the information about the discovered EASs.

6.x.2	Architecture Impacts

If the EMS is not utilized via EES, this solution has no impact on the existing architecture.

If the EMS is used via EES, the architecture, functionalities and relevant procedures for the EDGE service may need some enhancements to support the new application enabler (i.e. EMS) and new functionalities for the energy saving.

6.x.3	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * Next Change * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[5]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[6]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[23558]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

* * * End of Change * * * *