3GPP TSG-SA WG6 Meeting #69	S6-254746

Wuhan, China 13th – 17th October 2025	(revision of S6-254615,S6-254314)

Source:	Ericsson

Title:	SEALDD enabled Multicast/Broadcast data delivery service

Spec:	3GPP TR 23.700-52 v0.3.0

Agenda item:	9.7

Document for:	Approval

Contact:	Ashish S Sharma (ashish.sanjay.sharma@ericsson.com)

1. Introduction

This pCR proposes a new solution to KI#1

2. Reason for Change

The solution enables VAL server to use the SEALDD server for the multicast/broadcast downlink data delivery. The SEALDD server uses the NRM services to create and manage the MBS sessions and performs the downlink data delivery for the MBS sessions. Hence, in this CR is aligned with the common understanding established and agreed related to SEAL specification, i.e., the NRM is the entity to request/manage resources and interacts with the 3GPP network for this matter, and SEADD is for data delivery.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-52 v0.3.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications".

[x]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services".

[y]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

* * * Next Change * * * *

## 6.x	Solution #x: SEALDD enabled Multicast/Broadcast data delivery service

### 6.x.1	Solution description

The solution enables the VAL server to use SEALDD server to deliver Multicast and Broadcast data to VAL clients.

#### 6.x.1.1	General

The SEALDD server provides the Multicast and Broadcast service (MBS) data delivery service to the VAL server using a 5G System to send a single copy of the data to several clients. The SEALDD server abstracts the underlying complexity of managing the MBS sessions and MBS data delivery and provides an exposure to the VAL server for MBS data delivery. The VAL server can request the MBS data delivery services by providing the media service requirements like QoS, location etc. to the SEALDD server. The SEALDD server uses the SEAL NRM services to create and manage the MBS sessions and use the SEALDD user plane to perform the data delivery.

The essence of the solution utilizes the NRM server for resource management for the MBS session management (like join notification, MBS session announcement messages etc), whereas SEALDD server is the entity to deliver the multicast and broadcast media over SEALDD-UU.

Editor's NOTE: The support for SEALDD enabled broadcast media delivery over eMBMS is FFS.

Figure 6.x.1.1 -1 shows the high-level functional model for this service.

Figure 6.x.1.1-1: SEALDD enabled Multicast and Broadcast data delivery service

* * * Next Change * * * *

#### 6.x.1.2	SEALDD enabled Multicast and Broadcast data delivery service

##### 6.x.1.2.1	SEALDD enabled Multicast and Broadcast data delivery create procedure

Figure 6.x.1.2.1-1 illustrates the high-level procedure of creating a SEALDD enabled Multicast and Broadcast data delivery service for the VAL server.

Pre-conditions:

- The VAL server and VAL client has established the application layer connection and provisioned the VAL client with application layer information for the MBS application or services.

Figure 6.x.1.2.1-1: SEALDD enabled Multicast and Broadcast data delivery create procedure

- The VAL server sends the SEALDD MulticastBroadcast service create request to send downlink data to the VAL UEs, VAL group ID. The request includes VAL server ID, VAL service ID, list of VAL UEs, delivery mode (i.e., multicast or broadcast), service area, QoS requirements, and optionally start and end time of the delivery

- The SEALDD server allocates the SSM address, port, SEALDD client MBS data delivery ID and invokes the NRM server to send a NRM MulticastBroadcast service create request including the requirements defined in the previous step 1.

- The NRM interacts with the 3GPP system based on the procedures in step 2 of clause 14.3.4A.2.2 3GPP TS 23.434[y] to create an eMBMS or an MBS session, respectively. Upon successful creation of an MBS session, the NRM server receives a MBS session ID.

- Same as step 3 defined in clause 14.3.4A.2.2 TS 23.434[y].

- Same as step 4 defined in clause 14.3.4A.2.2 TS 23.434[y].

- Same as step 5 defined in clause 14.3.4A.2.2 TS 23.434[y].

7a-7b.	The VAL client sends a service request to the SEALDD client for the Multicast broadcast session service as per VAL service provisioning information. The SEALDD client communicates with the NRM client about the SSM address, MBS Session ID to receive multicast or broadcast packets. Based on the service request, the SEALDD client establishes the SEALDD-UU connection as per clause 9.2.2.2 SEALDD regular connection establishment. The SEALDD server may include the allocated SEALDD client MBS data delivery ID in the regular connection establishment response for the SEALDD client.

NOTE: The interaction between the VAL client, SEALDD client and NRM client is implementation-specific and out of scope of 3GPP.

8a.	In case of broadcast delivery type, the NRM client listens to the MBS session ID for the broadcast data, same as step 6b defined in clause 14.3.4A.2.2 TS 23.434[y].

8b.	In case of multicast delivery type, the NRM client joins the multicast session with 5GSystem using the MBS session ID same as step 6a defined in clause 14.3.4A.2.2 TS 23.434[y].

8c.	Same as step 6a defined in clause 14.3.4A.2.2 TS 23.434[y] and clause 14.3.4A.2.2 TS 23.434[y].

- Same as step 8 defined in clause 14.3.4A.2.2 TS 23.434[y]

- The NRM server sends the MulticastBroadcast service create response to the SEALDD server with the received MBS session ID as per the clause 14.3.4A.2.2 TS 23.434[y]. On receiving the NRM response message, the SEALDD server allocates the endpoint information of the SEALDD server side to receive the data from the VAL server and SEALDD MBS Session ID to manage the MBS sessions.

- The SEALDD server sends the SEALDD MulticastBroadcast service create response to the VAL server with the allocated endpoint information to send data and SEALDD MBS session ID for managing the MBS sessions.

12.	The VAL server sends the DL data to the SEALDD server over the allocated ingress connection information. The SEALDD server delivers the DL data to the SEALDD clients over the SEALDD-UU session. The SEALDD clients forward them to the VAL clients. The SEALDD server may determine to switch to the unicast session to deliver the MBS data and back to MBS sessions.

* * * Next Change * * * *

##### 6.x.1.2.2	SEALDD enabled Multicast and Broadcast data delivery update procedure

Figure 6.x.1.2.2-1 illustrates the high-level procedure of updating a SEALDD enabled Multicast and Broadcast data delivery service for the VAL server.

Precondition:

-	A SEALDD server enabled multicast or broadcast session is created and media is delivered.

Figure 6.x.1.2.2-1: SEALDD enabled Multicast and Broadcast data delivery update procedure

1.	The VAL server sends a SEALDD MulticastBroadcast service update request to the SEALDD server. The request includes among others the parameters to be updated related to the MBS session under consideration, e.g., the service requirement or service area, list of VAL UEs, SEALDD MBS session ID.

2.	The SEALDD server sends an NRM Multicast resource update request towards the NRM server as per step 2 defined in clause 14.3.4A.3.2 TS 23.434[y].

3.	The NRM server sends an NRM MulticastBroadcast service update request towards the 5GS (to the MB-SMF either directly or via the NEF) as per the step 3, step 4 and step 5 of clause 14.3.4A.3.2 TS 23.434[y].

4.	Same as step step 7 of clause 14.3.4A.3.2 TS 23.434[y].

5.	The NRM client stores and process the updated received information. If required, the NRM client interacts with the SEALDD client to configure as per the updated information.

NOTE: It is implementation specific how the NRM client and SEALDD client communicate with each other for in the VAL UE and is out of scope.

6.	Same as step 5 defined in clause 14.3.4A.3.2 TS 23.434[y].

7.	Same as step 7 defined in clause 14.3.4A.3.2 TS 23.434[y]. For the updated MBS sessions (like new MBS sessions, update of service areas, etc.), the SEALDD server associates it with the allocated ingress address towards the VAL server.

8.	The SEALDD server sends a SEALDD MulticastBroadcast service update response to the VAL server indicating the success of the update with the updated parameters of the MBS session.

9.	The VAL server continues sending data delivery using the allocated address and SEALDD server delivers the data over the updated MBS session to the users.

* * * Next Change * * * *

##### 6.x.1.2.3	SEALDD enabled Multicast and Broadcast data delivery deletion procedure

Figure 6.x.1.2.3-1 illustrates the high-level procedure of deletion a SEALDD enabled Multicast and Broadcast data delivery service for the VAL server.

Precondition:

-	A SEALDD server enabled multicast or broadcast session upon the request of the VAL server, and media is delivered.

-	There is no ongoing VAL group communication over the MBS session.

Figure 6.x.1.2.2-1: SEALDD enabled Multicast and Broadcast data delivery deletion procedure

1.	The VAL server decides to delete an MBS session if it is no longer needed.

2.	The VAL server sends the SEALDD MulticastBroadcast service delete request to delete the MBS sessions.

3.	The SEALDD server invokes NRM service as per step 2 defined in 14.3.4A.4.2 TS 23.434[y]..

4.	Same as step 3 defined in 14.3.4A.4.2 TS 23.434[y].

5b.	In case of broadcast MBS session, the NRM client stops monitoring the MBS session under consideration as per step 4a defined in 14.3.4A.4.2 TS 23.434[y].

5b.	In case of multicast delivery type, the NRM client performs UE session leave procedure as per step 4b 4a defined in 14.3.4A.4.2 TS 23.434[y].

NOTE: The communication between NRM client and SEALDD client related to session deletion is implementation specific and out of scope of 3GPP.

6.	Same as step 5 defined in 14.3.4A.4.2 TS 23.434[y].

7.	Same as step 6 defined in 14.3.4A.4.2 TS 23.434[y].

7.	Same as step 7 defined in 14.3.4A.4.2 TS 23.434[y].

8.	The SEALDD server sends a SEALDD MulticastBroadcast service delete response to the VAL server indicating the result of the MBS deletion and MBS session ID.

* * * Next Change * * * *

### 6.x.2	Solution evaluation

The solution enables the VAL server to use the SEALDD layer as the user plane delivery for the broadcast/multicast data to send a single copy to several recipients. The SEALDD server hides the complexity of managing MBS sessions for the VAL server. In MBS session update, there is no change in the user plane data delivery between the SEALDD server and the VAL server. The SEALDD server handles the underlying updates in the MBS session without affecting the VAL server data delivery endpoint, which enables the VAL server to continue sending the data delivery using the allocated information. The SEALDD server utilizes the NRM server for Multicast broadcast resource management based on the existing procedures. This enables the vendor interoperability with flexible deployment scenarios like colocation of the NRM server and SEALDD server. Furthermore, the solution is aligned with the agreed concepts within SEAL specifications, i.e. the NRM server is responsible for resource management and the SEALDD server is responsible for data delivery.

* * * End of Changes* * * *