3GPP TSG-SA WG6 Meeting #70	S6-255026

Dallas, United States 17th – 21st November 2025

Source:	InterDigital

Title:	Solution #4 update and evaluation

Spec:	3GPP TR 23.700-26 v0.2.0

Agenda item:	9.8

Document for:	Approval

Contact:	Michel Roy, <michel.roy@interdigital.com>

1. Introduction

This contribution proposes an update and evaluation for solution #4.

2. Reason for Change

In SA6#69, complementary architectures were proposed in Solution #1 and Solution #2.

This contribution provides an update to solution #4 (Support of AIoT device presence information subscription) to align with the proposed architectures and provides the corresponding evaluation.

3. Conclusions

NA

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-26 v0.2.0.

* * * First Change * * * *

#### 7.4.1.2 	AIoT presence information subscription

Figure 7.4.1.2-1 describes a procedure for AIoT presence information subscription.

Figure 7.4.1.2-1: AIoT Presence Information Subscription

1.	The requestor (e.g., VAL server or AIOTEC) sends an AIoT presence information subscription request to the AIOTES. The request includes the requestor identifier, information about AIoT device(s) [device identifier, area information] and information about the presence reporting configuration [notification endpoint, frequency, schedule, location indication, change only indication].

2.	Upon receiving the request from the requestor, the AIOTES validates if the requestor is authorized for the request. If the requestor is authorized, the AIOTES allocates a presence reporting subscription identifier, and stores the associated AIoT device information and presence reporting configuration.

The AIOTES initiates the presence monitoring by sending the Nnef_AIoT_Inventory Request as described in 3GPP TS 23.369 [3]. Based on the AIoT devices information and reporting configuration, the AIOTES includes the AIOTES identifier, the AIOTES notification endpoint, the AIoT device identifiers and may include Area Information, Location information indication and number of AIoT devices as input parameters for Nnef_AIoT_Inventory Request. If the reporting configuration includes schedule, the AIOTES sends the Nnef_AIoT_Inventory Request according to the time schedule (e.g., delays the request based on reporting time schedule).

The AIOTES configures the reporting based on the presence reporting configuration. If the reporting configuration includes frequency, the AIOTES schedules periodic reporting of the monitored presence to the notification endpoint according to frequency. If the reporting configuration includes change only indication, the AIOTES sends the notification only if a change in the presence information has happened since the previous notification.

If the AIOTES sends the Nnef_AIoT_Inventory Request and the request is successful, the AIOTES stores the returned transaction identifier.

3.	The AIOTES sends an AIoT Information Provisioning response to the requestor. If the request was authorized, the response includes an indication of success for the request, the presence reporting session identifier and an indication that the presence monitoring has started. Otherwise, the response includes an indication of failure and may include a failure cause.

* * * Next Change * * * *

#### 7.4.1.3 	AIoT presence information notification

Figure 7.4.1.3-1 describes a procedure for AIoT presence information notification.

Figure 7.4.1.3-1: AIoT Presence Information Notification

1.	The AIOTES detects a presence reporting event. The presence reporting event may be one of receiving a Nnef_AIoT_Notify message or detecting a reporting event based on the reporting configuration.

If the AIOTES receives a Nnef_AIoT_Notify message as described in 3GPP TS 23.369 [3], the AIOTES identifies retrieves the subscription information using the AF transaction identifier. The AIOTES stores or updates the received AIoT device presence information locally and prepares the information to be included in the AIoT Presence Information notification. If the change only indication is present in the reporting configuration, the AIOTES determines, for each device, if the received presence information has changed and the AIoT Presence Information notification is sent only if the AIOTES has determined a change. If the Nnef_AIoT_Notify message includes a last report indication, the AIOTES may send a Nnef_AIoT_Inventory Request as described in step 2 of clause 7.4.1.2 to continue receiving inventory notifications.

If the AIOTES detects a reporting event which is the periodic reporting frequency timer expiration, the AIOTES prepares the information to be included in the AIoT Presence Information notification based on the AIoT device presence information locally available.

If the AIOTES detects a reporting event which is a scheduled reporting time event, if the reporting event is to start reporting, the AIOTES starts inventory command by sending the Nnef_AIoT_Inventory Request as described in step 2 of clause 7.x.1.2. If the detected reporting event is to stop reporting, the AIOTES sends a last notification to the requestor to indicate that reporting is stopped and stops sending notifications to the requestor.

2.	The AIOTES sends an AIoT presence information notification to the endpoint configured in the presence reporting configuration.

The notification includes a presence report including the AIoT device identifiers, the presence status of the AIoT devices and may include the location information (e.g., as indicated in Nnef_AIoT_Notify), a duration since the last presence status change. The presence report includes a presence reporting status (e.g., started or stopped) if the detected event scheduled reporting time event.

* * * Next Change * * * *

#### 7.4.3.2	AIoT Presence Information Subscription response

Table 7.4.3.2-1 describes the AIoT Presence Information Subscription response.

Table 7.4.3.2-1: AIoT Presence Information Subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | The indication that the request was successful. |
| > Subscription identifier | M | The identifier for the subscription. |
| Failure response | O (NOTE) | The indication that the request failed. |
| > Cause | O | The failure cause |
| NOTE: Only one of these information elements shall be provided. |  |  |

* * * Next Change * * * *

### 7.4.2	Architecture Impacts

This solution proposes that a new functional entity, as an AIoT Enablement Server (AIOTES) which is a SEAL Server, is supported in the application enablement layer as described in Solution #1 and Solution #2.

This solution also proposes that a new functional entity, as an AIoT Enablement Client (AIOTEC), which is a SEAL Client, is supported in the application enablement layer as described in Solution #1.

* * * Next Change * * * *

### 7.4.4	Solution evaluation

This solution addresses Key Issue #3 on provisioning information in the application enablement layer and enables presence monitoring of AIoT devices. It provides procedures for enabling a consumer (e.g., VAL server or AIOTEC) of monitoring AIoT device presence via a subscription mechanism.

The solution uses the capabilities of inventory command as described in 3GPP TS 23.369 [3] for obtaining the AIoT device presence and allows monitoring AIoT devices via their identifiers and/or area information. The monitoring reporting allows fine grained control of the monitoring via a reporting configuration.

The solution is compatible with architectures described in Solution #1 and Solution #2.

* * * End of Change * * * *