3GPP TSG-SA WG6 Meeting #70	S6-255028

Dallas, United States 17th – 21st November 2025

Source:	InterDigital

Title:	New Solution on AIoT information provisioning

Spec:	3GPP TR 23.700-26 v0.2.0

Agenda item:	9.8

Document for:	Approval

Contact:	Michel Roy, <michel.roy@interdigital.com>

1. Introduction

This contribution proposes a new solution on AIoT service provisioning.

2. Reason for Change

This solution address key issue#2, and #4 to introduce a solution for AIoT information provisioning and management of such information.

3. Conclusions

NA

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-26 v0.2.0.

* * * First Change * * * *

## 7.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 | x |  |  |  |
| Sol #2 | x |  |  |  |
| Sol #3 | x | x | x | x |
| Sol #4 |  |  | x |  |
| Sol #5 |  | x | x | x |
| Sol #6 |  | x |  | x |
| Sol #7 |  | x | x | x |
| Sol #8 |  | x |  |  |
| Sol #9 |  | x |  |  |
| Sol #10 |  | x |  |  |
| Sol #11 |  |  | x |  |
| Sol #12 |  | x | x |  |
| Sol #13 |  | x |  | x |
| Sol #y |  | x |  | x |
| Sol #x |  |  |  |  |

* * * Next Change * * * *

## 7.y		Solution #y: Solution for AIoT information provisioning

### 7.y.1	Solution description

#### 7.y.1.1 	Introduction

This solution address key issue#2, and #4 to introduce a solution for AIoT information provisioning for the application enablement layer AIoT service.

An AIoT service consumer needs to provision information related to AIoT devices, such as AIoT device identifiers or supported operations, before consuming AIoT enablement services.

The provisioned AIoT device information is stored by the AIoT enablement server (AIOTES) and is associated with an identifier that can be used to manage the provisioned AIoT devices or for requesting operations such as inventory or commands.

This solution defines procedures for:

- Provisioning AIoT device information in clause 7.y.1.2;

- Updating provisioned AIoT device information in clause 7.y.1.3

- Deleting provisioned AIoT device information in clause 7.y.1.4

#### 7.y.1.2 	AIoT information provisioning

The figure 7.y.1.2-1 describes the procedure for AIoT information provisioning.

Figure 7.y.1.2-1: AIoT information provisioning

1.	The requestor (e.g., VAL server or AIOTEC) sends an AIoT information provisioning request to the AIOTES. The request includes the requestor identifier and a list of AIoT device profiles as described in clause 7.y.3.1.

2.	Upon receiving the request from the requestor, the AIOTES validates if the requestor is authorized for the request. If the requestor is authorized, the AIOTES allocates an AIoT device group identifier for the provisioned devices and stores the associated AIoT device information. The AIoT device group identifier is used by the requestor when managing the provisioned devices group or when performing tasks such as requesting an inventory operation or a command for the group.

3.	The AIOTES sends an AIoT information provisioning response to the requestor as described in clause 7.y.3.2. If the request was authorized, the response includes an indication of success for the request and the AIoT device group identifier. Otherwise, the response includes an indication of failure and may include a failure cause.

#### 7.y.1.3 	AIoT information provisioning update

The figure 7.y.1.3-1 describes the procedure for AIoT information provisioning update.

Figure 7.y.1.3-1: AIoT information provisioning update

1.	The requestor (e.g., VAL server or AIOTEC) sends an AIoT information provisioning update request to the AIOTES. The request includes the requestor identifier, an AIoT device group identifier and a list of AIoT device profiles, including an indication whether the device profile must be updated, added or removed as described in clause 7.y.3.3.

2.	Upon receiving the request from the requestor, the AIOTES validates if the requestor is authorized for the request. If the requestor is authorized, the AIOTES updates the AIoT device profiles information which is associated with the AIoT device group. The update results in updating pre-existing AIoT device profiles (e.g., identified via their device identifier or VAL device identifier), adding new AIoT device profiles or removing existing device profiles.

3.	The AIOTES sends an AIoT information provisioning response to the requestor as described in clause 7.y.3.4. If the request was authorized, the response includes an indication of success for the request. Otherwise, the response includes an indication of failure and may include a failure cause.

#### 7.y.1.4 	AIoT information provisioning delete

The figure 7.y.1.4-1 describes the procedure for AIoT information provisioning delete.

Figure 7.y.1.3-1: AIoT information provisioning delete

1.	The requestor (e.g., VAL server or AIOTEC) sends an AIoT information provisioning delete request to the AIOTES. The request includes the requestor identifier, an AIoT device group identifier as described in clause 7.y.3.5.

2.	Upon receiving the request from the requestor, the AIOTES validates if the requestor is authorized for the request. If the requestor is authorized, the AIOTES deletes all the AIoT device profiles information which is associated with the AIoT device group.

3.	The AIOTES sends an AIoT information provisioning response to the requestor as described in clause 7.y.3.6. If the request was authorized, the response includes an indication of success for the request. Otherwise, the response includes an indication of failure and may include a failure cause.

### 7.y.2	Architecture Impacts

This solution proposes that a new functional entity, as an AIoT Enablement Server (AIOTES) which is a SEAL Server, is supported in the application enablement layer as described in Solution #1 and Solution #2.

This solution also proposes that a new functional entity, as an AIoT Enablement Client (AIOTEC), which is a SEAL Client, is supported in the application enablement layer as described in Solution #1.

### 7.y.3	Corresponding APIs

#### 7.y.3.1	AIoT Information Provisioning request

Table 7.y.3.1-1 describes the AIoT information provisioning request.

Table 7.y.3.1-1: AIoT Information Provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| List of AIoT device profiles | M | The list of AIoT device profile to be provisioned as described in Table 7.y.3.1-2. |

Table 7.y.3.1-2: AIoT device profile

| Information element | Status | Description |
| --- | --- | --- |
| Device identifier | M | The identifier of an AIoT device which is used for interactions with the network as described in 3GPP TS 23.369 [2]. |
| Device owner identifier | M | The identifier of the owner of the AIoT device. |
| VAL device identifier | O | The device identifier is used for interactions with the VAL layer. This device identifier is known by the device owner and is used for interactions with the VAL layer if present, otherwise the Device identifier is used. |
| Device type | M | The type of the AIoT device (e.g., smart-tag, sensor, etc.). |
| Device capabilities | M | The capabilities of the AIoT device. |
| > Supported operations | M | The operation types supported by the AIoT device (e.g., Inventory, Command) as described in 3GPP TS 23.369 [2]. For Command operation, information about the supported command parameters and the command response size is included. |
| > Maximum rate | O | The maximum rate at which operations can be sent to the AIoT device. This information indicates the maximum rate for requesting operations to the AIoT device and may indicate the energy replenishment time between operations. |
| > Maximum transmission size | O | The maximum amount of data that the AIoT device is capable of transmitting in one operation. This information indicates whether multiple operations may be required to obtain information from the AIoT device. |
| Area information | O | The area information associated with the AIoT device. |
| Device status | M | The status (e.g., enabled, disabled) of the AIoT device. |

#### 7.y.3.2	AIoT Information Provisioning response

Table 7.y.3.2-1 describes the AIoT information provisioning response.

Table 7.y.3.2-1: AIoT Information Provisioning response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | The indication that the request was successful. |
| > Device group identifier | M | The identifier for the provisioned device group. |
| Failure response | O (NOTE) | The indication that the request failed. |
| > Cause | O | The failure cause |
| NOTE: Only one of these information elements shall be provided. |  |  |

#### 7.y.3.3	AIoT Information Provisioning Update request

Table 7.y.3.3-1 describes the AIoT information provisioning update request.

Table 7.y.3.3-1: AIoT Information Provisioning Update request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Device group identifier | M | The identifier for the device group. |
| List of AIoT device profiles | O (NOTE) | The list of AIoT device profile as described in Table 7.y.3.1-2. Pre-existing device identifiers or VAL device identifiers are updated, and non-existing device identifiers are added. |
| List of removed AIoT device profiles identifiers. | O (NOTE) | The list of AIoT device profile identifiers to remove, including device identifiers or VAL device identifiers. |
| NOTE: At least one of these information elements shall be provided. |  |  |

#### 7.y.3.4	AIoT Information Provisioning Update response

Table 7.y.3.4-1 describes the AIoT information provisioning update response.

Table 7.y.3.4-1: AIoT Information Provisioning Update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | The indication that the request was successful. |
| Failure response | O (NOTE) | The indication that the request failed. |
| > Cause | O | The failure cause |
| NOTE: Only one of these information elements shall be provided. |  |  |

#### 7.y.3.5	AIoT Information Provisioning Delete request

Table 7.y.3.5-1 describes the AIoT information provisioning delete request.

Table 7.y.3.5-1: AIoT Information Provisioning Delete request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Device group identifier | M | The identifier for the device group. |

#### 7.y.3.6	AIoT Information Provisioning Delete response

Table 7.y.3.6-1 describes the AIoT information provisioning delete response.

Table 7.y.3.6-1: AIoT Information Provisioning Delete response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | The indication that the request was successful. |
| Failure response | O (NOTE) | The indication that the request failed. |
| > Cause | O | The failure cause |
| NOTE: Only one of these information elements shall be provided. |  |  |

### 7.y.4	Solution evaluation

This solution addresses Key Issue #2 on Exposing the value-added information of AIoT devices and Key Issue #4 on AIoT Data Management. The solution allows the VAL layer to provision AIoT device information (e.g., AIoT device profiles) with the AIoT enablement service to enable AIoT operations with the network and includes supplemental information that is required for interacting with the network.

The provisioned information includes AIoT device profile(s) for a group of AIoT devices and enable the AIoT enablement service to perform operations (e.g., Inventory, Command) based on a AIoT device group identifier. The solution proposes procedures for provisioning, updating, and removing the AIoT device information.

The solution uses the capabilities of inventory and command operations as described in 3GPP TS 23.369 [3].

The solution is compatible with architectures described in Solution #1 and Solution #2.

* * * End of Changes * * * *