3GPP TSG-SA WG6 Meeting #70	S6-255063

Dallas, Texas, USA 17th – 21st November 2025	(revision of S6-254081)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0490 | rev | 1 | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Call forwarding for Ad hoc Group calls involving multiple MCPTT systems (procedures) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Kontron Transportation France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph6-MC |  |  |  |  |  | Date: |  |  | 2025-11-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Stage1 has agreed CR0083 against TS 22.179 on call forwarding for ad hoc group call (clause 6.20.1): [R-6.20.1-001] The MCPTT Service shall provide a mechanism for an authorized MCPTT User, receiving an Ad hoc Group call, based on its MCPTT user Identity or its associated functional alias to configure the forwarding of an incoming Ad hoc Group call, to another MCPTT user in the following situations: - Always - If the receiving MCPTT User is not reachable (e.g., out of coverage) - If the incoming Ad hoc Group call is a call with manual commencement mode and the receiving MCPTT User does not answer within a configured period of time - If the receiving MCPTT User rejects an incoming call (in manual commencement mode) [R-6.20.1-002] The MCPTT Service shall provide a mechanism to inform the initiator of the Ad hoc Group call that the call has been successfully forwarded to another MCPTT user by providing the MCPTT user Identity and/or associated functional alias of the forwarding target. This CR addresses the requirements from above. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the procedures to specify call forwarding for ad hoc group calls involving multiple MCPTT systems |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 1 requirement to provide call forwarding for ad hoc group call is not fulfilled |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 10.19.3.2.8, (new) 10.19.3.2.8.1, (new) 10.19.3.2.8.2, (new) 10.19.3.2.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

##### 10.19.3.2.8	Call forwarding for ad hoc group calls

###### 10.19.3.2.8.1	General

Call forwarding of MCPTT ad hoc group calls involving multiple MCPTT systems allows users to configure forwarding of incoming ad hoc group call requests (with or without Floor control) to another MCPTT user in a partner MC system under the same conditions as defined for the single MCPTT system scenario in clause 10.19.3.1.7 (i.e., immediate, no answer and based on manual input).

An authorized user can update the call forwarding related parameters stored in the user profile as described in clause 10.19.3.1.7.2 (single MCPTT system).

For handling of multiple ad hoc group call forwardings the same rules are applied as described in clause 10.19.3.1.7.6 (single MCPTT system).

###### 10.19.3.2.8.2	Call forwarding of an ad hoc group call towards a partner MCPTT system

Figure 10.19.3.2.8.2-1 below illustrates the ad hoc group call setup procedure initiated by an authorized user when call forwarding of the ad hoc group call is applied and a target MCPTT user is in a partner MCPTT system.

NOTE 1:	The procedure shows manual commencement mode, but the changes are also applicable when using automatic commencement mode.

NOTE 2:	The procedure focusses on the interaction between the MCPTT servers, which is independent of the condition of the call forwarding. Therefore, the procedure is generically applicable for all types of MCPTT ad hoc group call forwarding.

Pre-conditions:

1.	The authorized user at MCPTT client 1 wants to invite MCPTT users at MCPTT client 2, and MCPTT client 3 for the ad hoc group call.

2.	MCPTT client 2 is authorized to use ad hoc group call forwarding and has immediate call forwarding enabled towards MCPTT client 4 in the partner MCPTT system.

Figure 10.19.3.2.8.2-1: Call forwarding of an ad hoc group call towards a partner MCPTT system

1.	The user at MCPTT client 1 would like to initiate an ad hoc group call. The MCPTT client 1 initiates the ad hoc group call by sending the ad hoc group call request containing either a list of participants or a critria or an ad hoc group ID from an ad hoc group emergency alert to the MCPTT server 1.

2.	If the ad hoc group call is supported, the MCPTT server 1 verifies whether the user at MCPTT client 1 is authorized to initiate an ad hoc group call.

3.	The MCPTT server 1 sends the ad hoc group call request return message to MCPTT client 1.

4.	The MCPTT server 1 determines the list of participants when the ad hoc group call is determined by criteria. The MCPTT server 1 checks if for any of the MCPTT clients call forwarding is enabled. The MCPTT server 1 detects that MCPTT client 2 has immediate call forwarding configured and enabled and checks the redirection counter of MCPTT client 2. The redirection counter is below the limit.

5.	The MCPTT server 1 increments the redirection counter and determines the new target MCPTT ID.

6a-6b.	The MCPTT server 1 sends the ad hoc group call requests towards the MCPTT clients of the invited users based on step 1 and as determined in step 5. The request towards MCPTT client 4 is sent via MCPTT server 2.

7a-7b.	The receiving MCPTT clients notify their corresponding MCPTT user about the incoming ad hoc group call request with the information of the MCPTT group ID of the ad hoc group.

8a-8b.	The receiving MCPTT clients may accept or reject the ad hoc group call requests and send ad hoc group call responses to the MCPTT servers.

9.	The MCPTT server 1 sends the ad hoc group call response to MCPTT client 1 through the signalling path to inform about successful call establishment.

10.	The MCPTT server 1 may notify the initiating MCPTT user of all MCPTT users joining or leaving the MCPTT ad hoc group call.

11.	MCPTT client 1, MCPTT client 2, and MCPTT client 4 establish media plane and floor control resources.

NOTE 3:	If the ad hoc group call was initiated using criteria, the criteria is still applied to MCPTT client 2 and determines the participation of MCPTT client 4 to the ad hoc group call.

###### 10.19.3.2.8.3	Call forwarding of an ad hoc group call occurring in a partner MCPTT system

Figure 10.19.3.2.8.3-1 below illustrates the ad hoc group call setup procedure when a target MCPTT user is in a partner MCPTT system and call forwarding initiated by an authorized user occurs in the partner MCPTT system.

NOTE 1:	The procedure shows manual commencement mode, but the changes are also applicable when using automatic commencement mode.

NOTE 2:	The procedure focusses on the interaction between the MCPTT servers, which is independent of the condition of the call forwarding. Therefore, the procedure is generically applicable for all types of MCPTT private call forwarding.

Pre-conditions:

1.	The authorized user at MCPTT client 1 wants to invite MCPTT users at MCPTT client 2, and MCPTT client 3 for the ad hoc group call.

2.	MCPTT client 3 in the partner MCPTT system is authorized to use ad hoc group call forwarding and has immediate call forwarding enabled towards MCPTT client 4 also in the partner MCPTT system.

Figure 10.19.3.2.8.3-1: Immediate call forwarding of an ad hoc group call

1.	The user at MCPTT client 1 would like to initiate an ad hoc group call. The MCPTT client 1 initiates the ad hoc group call by sending the ad hoc group call request containing either the list of participants or criteria or an ad hoc group ID from an ad hoc group emergency alert to the MCPTT server 1.

2.	If the ad hoc group call is supported, the MCPTT server 1 verifies whether the user at MCPTT client 1 is authorized to initiate an ad hoc group call.

3.	The MCPTT server 1 sends the ad hoc group call request return message to MCPTT client 1.

4a-4b.	The MCPTT server 1 sends the ad hoc group call requests towards the MCPTT clients of the invited users based on step 1 and as determined in step 5. The request towards MCPTT client 3 is sent via MCPTT server 2.

5.	If the MCPTT ad hoc group call request contains a functional alias instead of an MCPTT ID as called party, MCPTT server 2 checks whether MCPTT client 3 is allowed to receive an ad hoc group call from MCPTT client 1 using the functional alias.

6.	The MCPTT server 2 checks if any of the invited MCPTT clients received in step 4 have call forwarding enabled. The MCPTT server 1 detects that MCPTT client 3 has immediate call forwarding configured to MCPPT client 4.

7.	The MCPTT server 2 sends ad hoc group call forwarding request to MCPTT server 1.

8.	MCPTT server 1 checks whether the redirection counter is below the limit and increments the counter.

9.	MCPTT server 1 sends the ad hoc group call forwarding response to MCPTT server 2.

10.	The MCPTT server 1 sends an ad hoc group call request to MCPTT client 4.

11a-11b.	The receiving MCPTT clients notify their corresponding MCPTT user about the incoming ad hoc group call request with the information of the MCPTT group ID of the ad hoc group.

12a-12b.	The receiving MCPTT clients may accept or reject the ad hoc group call requests and send ad hoc group call responses to the MCPTT servers.

13.	The MCPTT server 1 sends the ad hoc group call response to MCPTT client 1 through the signalling path to inform about successful call establishment.

14.	The MCPTT server 1 may notify the initiating MCPTT user of all MCPTT users joining or leaving the MCPTT ad hoc group call.

15.	MCPTT client 1, MCPTT client 2, and MCPTT client 4 establish media plane and floor control resources.

NOTE 3:	If the ad hoc group call was initiated using criteria, the criteria is still applied to MCPTT client 3 and determines the participation of MCPTT client 4 to the ad hoc group call.

* * * End of Changes * * *