3GPP TSG-SA WG6 Meeting #70	S6-255112

Dallas, United States 17th – 21st November 2025	(revision of S6-25xxxx)

Source:	Telefónica, Apple, Fogus, UMA

Title:	TR 23.947 SEAL Framework overview

Spec:	3GPP TR 23.947

Agenda item:	9.14

Document for:	Approval

Contact:	w_featherstone@apple.com

1. Introduction

Add SEAL framework overview.

2. Reason for Change

Complete clause 4.4.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.947, version 0.2.0.

* * * First Change * * * *

## 4.4	Service Enabler Architecture Layer (SEAL)

The Service Enabler Architecture Layer (SEAL), specified in 3GPP TS 23.434 [6], defines a common service framework designed to support vertical and mission-critical applications operating over 3GPP networks. SEAL provides a standardized set of service enablers, such as group management, configuration, location, and network resource management, that allow applications to interact with network capabilities in a consistent, secure, and efficient way. Its goal is to simplify the development and deployment of vertical applications (VAL), ensuring interoperability and seamless integration across networks and operators. By introducing a horizontal service layer that bridges the application and network domains, SEAL abstracts the complexity of network functions and exposes their capabilities through unified APIs, enabling applications to focus solely on service logic while relying on SEAL for connectivity, security, and continuity management.

The SEAL architecture introduces four primary functional entities that cooperate to deliver these capabilities across the system: the VAL Client, the SEAL Client, the VAL Server, and the SEAL Server. Each plays a distinct role in bridging the application and service-enabler layers, ensuring transparent operation of SEAL-enabled services from device to network.

-	The Vertical Application Layer Client (VAL Client), which represents the end-user application component running on a device such as a UE, vehicle terminal, or IoT gateway. The VAL Client contains the service logic specific to a given vertical application, for example, mission-critical push-to-talk, industrial automation, or vehicular communication, and interacts locally with the SEAL Client to access common service enablers provided by the network. From the perspective of the application developer, the VAL Client remains independent from the underlying network details; it does not directly communicate with SEAL network functions but relies instead on the SEAL Client’s APIs to request network-related services. This separation ensures that vertical applications can be developed and deployed consistently across networks and operators, while still leveraging the optimized performance and reliability that SEAL provides.

-	The SEAL Client acts as the intermediary between the VAL Client and the SEAL system. It is a middleware function implemented on the same device or platform as the VAL Client, exposing standardized local APIs through which the VAL Client can invoke SEAL services. When the VAL Client requires a capability such as joining a communication group, obtaining a user’s location, or synchronizing configuration parameters, it makes a call to the SEAL Client, which then handles all the necessary signalling, security, and registration procedures toward the network. Internally, the SEAL Client communicates with the SEAL Server through the SEAL-2 reference point, following standardized procedures defined in 3GPP TS 23.434 [6]. This architecture ensures that the VAL Client experiences a simplified and consistent API model, while the SEAL Client abstracts all the complexities related to mobility, authorization, and network interaction. Moreover, the SEAL Client can manage service continuity by maintaining session state and coordinating with the SEAL Server when the device moves between coverage areas or network domains.

-	On the network side, the Vertical Application Layer Server (VAL Server) represents the application service logic deployed within the operator or third-party network domain. It is the counterpart of the VAL Client and provides the vertical-specific functionality, such as data aggregation, content delivery, analytics, or group coordination, on top of the common SEAL infrastructure. The VAL Server typically interacts with the SEAL Server through the SEAL-4 interface to access SEAL enablers, for example, to retrieve group membership information, user location data, or configuration parameters. By using the SEAL APIs exposed via the SEAL Server, the VAL Server can dynamically adapt its service logic to network conditions and user context. This allows, for instance, a vehicular application to select optimal communication groups or a public-safety system to update operational parameters in real time. The VAL Server may also register its service capabilities with the SEAL Server so that SEAL Clients and other network entities can discover and interact with it when needed.

-	At the core of the architecture is the SEAL Server, the main control-plane function of the SEAL framework. It is deployed in the operator’s network and serves as the authoritative entity responsible for managing SEAL services, enforcing access policies, and coordinating the various Service Enabler Functions (SEFs) defined within SEAL. The SEAL Server authenticates and authorizes SEAL Clients and VAL Servers, maintains context for registered devices and applications, and provides access to enablers such as group management, configuration, and location. When a SEAL Client registers, the SEAL Server establishes a control relationship with it and may also inform the VAL Server of the client’s presence or capabilities. The SEAL Server can also expose its APIs to external entities through the Common API Framework (CAPIF), allowing standardized discovery, onboarding, and authorization for SEAL-based services across multiple domains. Internally, the SEAL Server interfaces with 3GPP core functions (e.g., UDM, NEF, PCF) to obtain network and user information, ensuring that SEAL enablers operate in harmony with the underlying 3GPP system.

Figure 4.4-1: SEAL Architecture, as specified in 3GPP TS 23.434 [6]

The typical operational sequence in SEAL illustrates how vertical applications and network functions cooperate to provide network-assisted services in a standardized manner. While the exact sequence may vary depending on deployment or the specific enabler used, the process generally involves the following main phases:

1.	In the initial system setup phase, the network’s SEAL infrastructure is provisioned. The SEAL Server registers its available SEFs, such as the Group Management, Configuration, and Location enablers. These enablers are made discoverable to authorized clients and vertical applications through the SEAL framework, often integrated with the CAPIF for API exposure and onboarding. Similarly, VAL Servers that wish to utilize SEAL capabilities register with the SEAL Server, establishing a trust and policy relationship. Through this registration, the SEAL Server maintains a global awareness of which VAL applications and services exist in the system, what enablers they can access, and which domains or user groups they serve. This preparatory phase effectively bootstraps the control plane for SEAL-enabled communication, ensuring all service entities are authorized and discoverable.

2.	Once the system is ready, the device registration phase begins when a user device hosting a VAL Client and SEAL Client powers on or activates a SEAL-enabled application. The SEAL Client must register with the appropriate SEAL Server before any service invocation can occur. The SEAL Client may be pre-configured with the address of its serving SEAL Server, or it can discover it dynamically via network signalling or through the CAPIF discovery mechanism. Upon establishing contact, the SEAL Client initiates a registration procedure, providing identification, credentials, and potentially VAL application information to the SEAL Server. The SEAL Server authenticates the client and establishes a logical control association. During this process, the SEAL Server creates and stores the SEAL Client context, containing information such as supported enablers, current serving network, and associated VAL application identifiers. Upon successful registration, the SEAL Client becomes an active participant in the SEAL domain, capable of invoking SEAL services on behalf of its local VAL applications.

3.	Following registration, the service discovery and invocation phase begins. When the VAL Client requires a specific service, for example, to join a communication group, retrieve configuration parameters, or access a location service, it issues a local API request to the SEAL Client. The SEAL Client translates this request into a standardized SEAL service invocation toward the SEAL Server via the SEAL-2 interface. The SEAL Server receives the request, identifies the relevant SEAL enabler (e.g., Group Management Enabler, Configuration Management Enabler, or Location Management Enabler), and either processes it locally or routes it to another SEAL domain if inter-domain interaction is required. In many cases, the SEAL Server may also coordinate with a VAL Server, informing it of the request or retrieving application-specific data required to complete the operation. For instance, a VAL Client joining a mission-critical group may trigger the SEAL Server to validate the group through the Group Management Enabler, update the corresponding VAL Server, and return the necessary group identifiers and configuration back to the SEAL Client. The SEAL Client then provides this information to the VAL Client, completing the discovery and setup process transparently.

4.	Once service discovery is complete, the service utilization phase commences. The VAL Client begins exchanging application data with the VAL Server using the established context and parameters provided by SEAL. Depending on the type of service, the data path may be direct (VAL Client ↔ VAL Server) or optimized through SEAL-assisted routing within the network. The SEAL Client continues to play a supervisory role, monitoring connectivity, handling periodic signalling with the SEAL Server, and enforcing any policies related to session continuity or QoS. The SEAL Server, in turn, can dynamically provide updates to configuration or group membership through its enablers, ensuring that applications adapt to network or operational changes in real time. Throughout this process, SEAL also ensures that all security and authorization aspects remain consistent, for instance by validating CAPIF tokens before each service invocation or by enforcing domain-specific access control.

5.	Mobility handling is an intrinsic feature of SEAL, ensuring that services remain available as users or devices move across network areas or service domains. When a device hosting a SEAL Client changes its point of attachment, for example, moving between cells, edge zones, or even operator domains, the SEAL Client and SEAL Server cooperate to maintain service continuity. The SEAL Client can detect mobility either through lower-layer events from the 3GPP system or through monitoring of the active SEAL session. Upon detecting such a change, it informs its serving SEAL Server, which evaluates whether the device should remain within the same SEAL domain or be reassigned to another SEAL Server closer to the new location. If a relocation is required, the SEAL Server transfers the active SEAL context, including the state of active enablers such as group management or configuration data, to the target SEAL Server. The new SEAL Server initializes the corresponding enabler instances and updates its internal associations to reflect the client’s new serving area. Once the transfer is complete, the SEAL Client is notified and updates its connection parameters transparently. During this process, any associated VAL Servers can also be informed to synchronize application-specific state if necessary. From the application’s perspective, the transition occurs without service interruption: the VAL Client continues to operate normally, while SEAL ensures that all control and service contexts are seamlessly maintained throughout the mobility event.

* * * End of Changes * * * *