3GPP TSG-SA WG6 Meeting #70	S6-255133

Dallas, USA, 17th – 21th Nov 2025	(revision of S6-25xxxx)

Source:	CATT

Title:	Update Sol#10 to fix the EN

Spec:	3GPP TR 23.700-44 v0.2.0

Agenda item:	9.9

Document for:	Approval

Contact:	Liping Wu <wuliping@cictmobile.com>

1. Introduction

The contribution proposes to update the Sol#10 to fix the EN and provide the solution evaluation.

2. Reason for Change

The contribution mainly proposes to add some more steps and complementation for the Sol#10 to fix the following EN and also provide the solution evaluation.

Editor’s Note:	How the ADAES generates the analysis for energy related information and which information is FFS.

The main updates for the solution are:

-	Indicate the energy information is for energy consumption information;

-	In the energy information analysis subscription request, add the analytics type to indicate the request for statistics or predictions and the preferred confidence level;

-	add new step 6 and step 7 to let A-ADRF to provide energy consumption historical analytics/information to ADAES if there is.

3. Proposal

It is proposed to agree the following changes for 3GPP TR 23.700-44 v0.2.0.

* * * First Change * * * *

6.10	Solution #10: Support of energy saving for location services

6.10.1	Solution description

This solution addresses the KI#6.

The following are the procedures to introduce how LMS subscribes the energy related information analysis from ADAES (clause 6.10.1.1) and how LMS utilizes the obtained energy related information analysis to reduce the energy consumption for the location services (clause 6.10.1.2).

6.10.1.1	Procedure of LMS subscription for the energy related information analysis

Figure 6.10.1.1-1 illustrates the high-level procedure of the LMS subscription for the energy related information analysis.

The LMS may subscribe to the ADAES for the analysis of energy related information.

Pre-condition:

-	The A-ADRF may have stored the historical energy consumption analytics for the corresponding VAL UEs or VAL service.

Figure 6.10.1.1-1: Procedure of LMS subscription for the energy related information analysis

- The consumer (i.e. LMS) may subscribe the energy information analysis related to location services from ADAES, including the analysis ID (e.g., energy consumption information analytics), the analytics type (e.g., statistics or predictions), the service ID, the target area, the energy type (e.g., energy consumption), the energy information granularity (e.g., per VAL UE per location service), the filtering information (e.g., maximum energy consumption per VAL UE), the target UE ID, the trigger conditions, the time duration, preferred confidence level, etc.

- The ADAES check if the LMS is authorized to request the energy information analysis. If the request is authorized, the ADAES responds to the LMS.

- The ADAES sends the energy information analysis response to the LMS.

- To obtain the analysis for the energy information, the ADAES may subscribe the energy related information (e.g., energy consumption information) from 3GPP Core network as described in clause 5.51.2.4 of 3GPP TS 23.501 [3], with the parameters received in service request in step 1.

- When the trigger condition is met, the 3GPP Core network provides the energy related information (e.g., energy consumption information) to the ADAES per requested granularity (e.g., per VAL UE per location service).

- The ADAE server optionally requests the energy consumption historical analytics/information from A-ADRF for the corresponding VAL UEs or VAL service (e.g. location service).

- Based on the request, the A-ADRF checks the local storage and sends the energy consumption historical analytics/information to ADAES if there is.

- Upon receiving the energy related information from step 5~7, the ADAES utilizes and generates the analytics for energy related information per VAL UE per location service, including the energy consumption information statistic (e.g., maximum/average/minimum energy consumption information over a certain period of time, etc.), historical energy information, energy consumption information predication (e.g., the time point of peak energy consumption), confidence level, etc.

- The ADAES notifies the energy information analysis to the LMS as service requested, including the analysis results generated in step 8.

* * * Next Change * * * *

6.10.3	Solution evaluation

This solution addresses the KI#6 which introduces how LMS in SEAL subscribes the energy consumption information analytics from ADAES and how LMS reduces the energy consumption for the location services per VAL UE to support the energy saving via utilizing the obtained energy consumption information analytics.

This solution has no impact on the existing architecture. The existing ADAE functionalities can be enhanced to support the energy consumption information analytics request from the consumers. The existing LM functionalities can be enhanced to adjust the location reporting configuration for the location management client to save the energy.

NOTE:	The information flows for the enhanced ADAE functionalities/procedures and the enhanced LM functionalities/procedures will be considered and specified in the normative phase.

* * * End of Change * * * *