3GPP SA WG6 Meeting #69	S6-255142

# Nov. 17 – 21st, 2025; Dallas, US. 					(Revision of S6-254137)

Source: 	China Mobile

Title: 	KI#3 Solution: Enhance SEALDD to support satellite selection in data delivery

Document for: 	Approval

Agenda Item: 	9.12

Work Item / Spec.:	FS_5GSAT_Ph4_APP / TR 23.700-02 v0.1.0

Abstract of the contribution: This document proposes a solution to address the KI#3 (TR 23.700-02), i.e., support satellite selection in data delivery.

## 1	Introduction

This pCR proposes a solution to address the KI#3, i.e., support satellite selection in data delivery. The solution targets at enhancing the SEALDD layer with the dynamic discovery and selection of SEALDD server based on the application satellite coverage availability information (or ASCAI). In the proposed solution, SEALDD servers are deployed on satellites and SEALDD clients on VAL UEs are on the ground. The enhanced SEALDD data transmission and service continuity leverage the satellite access characteristics.

## 2	Reason for Change

For communication service over the satellite access, operators may deploy some components of the SEALDD enabler on-board satellite(s) to strive for certain objectives, e.g., large to global range of service coverage, low transmission latency, etc. The target of low latency implies the utilization of the low-earth-orbit (LEO) satellite constellation. However, thanks to the lower altitude of LEOs, and also depending on the minimum elevation angles, their coverage range on Earth is limited. This will result in the shorter connection time for on-ground users via LEO-based satellite access.

For example, assuming a LEO satellite orbits at the 500 km altitude, which makes the orbital period roughly 95 minutes. With a reasonable choice of the minimum elevation angle (e.g., 30 degree), a specific spot on Earth could access the satellite approximately for 7 minutes once within the coverage of the satellite. Accordingly, in some scenarios, incomplete data transmission via a LEO satellite might be experienced before the satellite moves aways and no longer resides within users’ connection range. Therefore, when the LEO satellite-assisted enabling layer performs data transmission, it is necessary to consider how to ensure uninterrupted data transmission and strive for service continuity.

The solution addresses the service continuity requirement by proposing an enhanced SEALDD architecture to handle the switching of serving satellites along with the context transfer of the SEALDD components (i.e., servers) deployed on-board the satellites.

## 3	Proposal

It is proposed to agree and capture the solution for the inclusion in the TR 23.700-02 v0.1.0.

# * * * * First change * * * *

## 5	Solutions

## 5.1	Mapping of solutions to key issues

Table 5.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI#6 |
| --- | --- | --- | --- | --- | --- | --- |
| …… |  |  |  |  |  |  |
| Sol #X |  |  | X |  |  |  |

# * * * * next change * * * *

## 5.x	Solution #x: Enhance SEALDD to support satellite selection in data delivery

### 5.x.1	Architecture

### This solution is based on the SEALDD architecture as in 3GPP TS 23.433 [3].

### 5.x.2	Solution description

Because the KI description states that operators deploy some components of the SEALDD architecture on satellites to ensure reliable data transmission via satellite communications, the solution proposes to deploy the SEALDD servers on-board satellites to achieve the large to global range of service coverage. Low-Earth-Orbit (or LEO) satellites are utilized to reduce the transmission latency. However, thanks to the lower altitude of LEOs, and also depending on the minimum elevation angles, the coverage range of LEO on Earth is limited. This will result in the shorter connection time for on-ground users via LEO-based satellite access.

For example, assuming a LEO satellite orbits at the 500 km altitude, which makes the orbital period be roughly 95 minutes. With a reasonable choice of the minimum elevation angle (e.g., 30 degree), a specific spot on Earth could access the satellite approximately for 7 minutes once within the coverage of the satellite. Accordingly, in some scenarios, incomplete data transmission via a LEO satellite might be experienced before the satellite moves aways and no longer resides within users’ connection range. Therefore, when the LEO satellite-assisted enabling layer performs data transmission, it is necessary to consider how to ensure uninterrupted data transmission and strive for service continuity.

With SEALDD servers on-board LEO satellites, the solution enhances the SEALDD enabled data transmission service with the dynamic discovery and selection of (target) SEALDD server based on the application satellite coverage availability information (i.e., ASCAI). Once the target SEALDD server (on-board another satellite) is selected, the enhanced procedure handles the switching of serving satellites along with the context transfer of the SEALDD servers deployed on-board the satellites.

In the solution, SEALDD clients on VAL UEs are on the ground.

### 5.x.2.1	Procedures

### 5.x.2.1.1	SEALDD enabled transmission with satellite selection support in data delivery

Figure 5.x.2.1.1-1 illustrates the procedure for the source SEALDD (or S-SEALDD) server to discover and select a target SEALDD (or T-SEALDD) server based on Application Satellite Coverage Availability Information or ASCAI, and then handle the switching of serving satellites along with the context transfer between the SEALDD servers.

Pre-condition:

-	SEALDD servers are deployed on-board satellites and SEALDD clients are on the ground.

-	The VAL server discovers and selects the S-SEALDD server by CAPIF functions.

-	The S-SEALDD server has gotten the ASCAI by leveraging the procedures like in the TS 23.434[4] clause 21.2.2.

-	The existence and impact of the Inter-Satellite Links (or ISLs) that provide the transmission connectivity between satellites are out of the scope of the solution.

Figure 5.x.2.1.1-1: SEALDD enabled transmission with satellite selection support in data delivery

- The VAL server sends a Sdd_RegularTransmission request to the SEALDD server#1 (S-SEALDD), as specified in 3GPP Ts 23.433[3] clause 9.2.2.2. The request includes the identifiers of the application traffic (e.g. VAL service ID, VAL server ID), and optionally, the duration of the data connection. The S-SEALDD server is selected with the consideration of ASCAI.

- Upon receiving the request, the SEALDD server#1 performs an authorization check. If authorization is successful, the SEALDD server sends a response to the VAL server.

- The regular data transmission connection is established according to TS 23.433[3] clause 9.2.2.2. The S-SEALDD server transmits data received from VAL server to SEALDD client.

- The VAL server uses the ASCAI and the duration of the data connection to determine whether the current connection can maintain the service continuity for data transmission, before the serving satellite (i.e., Sat#1 for SEALDD server#1) moves away from the VAL UE’s connection range. The current S-SEALDD server (on Sat#1) will continue the data connection if no service interruption is expected. If the duration of the data connection will lead to service interruption due to the moving away of the Sat#1, the VAL server triggers the decision process of the service continuity for the data transmission connection.

NOTE 1:	When the ASCAI is utilized to discover a list of the target SEALDD servers, the VAL UE’s information (e.g., location data) may be considered for the discovery. The SEAL location management server can provide the VAL UE information.

- The S-SEALDD server (SEALDD server#1) performs candidate SEALDD servers discovery by using EEL, as specified in TS 23.433[3] clause 9.4. The discovery can utilize various discovery filter (or query filters), with the consideration of ASCAI in the query filter.

NOTE 2:	The discovered candidate SEALDD servers must be on different serving satellites from the S-SEALDD server’s serving satellite (i.e., Sat#1), since SEALDD servers on-board the same satellite will have the same ASCAI that cannot address the data connection interruption issue.

- Once the available target SEALDD servers are discovered, the S-SEALDD server can select the T-SEALDD server (i.e., SEALDD server #2) based on the coverage range, the coverage duration, or local configuration.

- The SEALDD context transfer procedure is performed between the S-SEALDD server and the T-SEALDD server, as specified in TS 23.433[3] clause 9.6.2.1. Both SEALDD servers on-board different satellites interact via the reference point SEALDD-E to transfer data content and exchange information for SEALDD service provisioning, control, reporting etc., as specified in TS 23.433 clause 7.4.5.

After the SEALDD context transfer procedure, the SEALDD client on VAL UE connects to the new T-SEALDD server (i.e., SEALDD server#2) to continue the data transmission connection.

# * * * * next change * * * *

### 5.x.2.2	Information flows

The following information flows are modified based on those from the 3GPP TS 23.433 [3]. All new changes are in bold.

### 9.2.3.1 SEALDD enabled regular transmission request Table 9.2.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the regular application transmission service. Table 9.2.3.1-1: SEALDD enabled Regular transmission request Information element; Status; Description. VAL server ID; M; Identity of the VAL server. VAL service ID; O; Identity of the VAL service. Identity; O; Identifier of specific UE or VAL user. SEALDD-S connection information; M; Address information (e.g., IP address and/or port, URL) of the VAL server to receive the traffic from the SEALDD server. QoS information; O; QoS information provided by VAL server. Duration of data connection; O; The total time a data transmission connection expects to last. VAL server’s total bandwidth limit; O (See NOTE); The total bandwidth limit of VAL server, including UL/DL. VAL users’ bandwidth limit; O (See NOTE); The bandwidth limits (i.e. minimum bandwidth requirement and maximum bandwidth limit) for VAL users, including UL/DL. NOTE: These IEs are used for the SEALDD enabled bandwidth control for different VAL users..

### 5.x.3	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

# * * * * End of changes * * * *