3GPP TSG-SA WG6 Meeting #70	S6-255153

Dallas, United States 17th – 21st November 2025	(revision of S6-254287)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0078 | rev | 1 | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SM data source subscription APIs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_Ph2-App |  |  |  |  |  | Date: |  |  | 2025-11-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | APIs related to SM data source subscription are not specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the APIs related to the SM data source subscription operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.7.1, 9.7.x(new), 9.7.x.1(new), 9.7.x.2(new), 9.7.x.3(new), 9.7.x.4(new), 9.7.x.5(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 9.7.1	General

Table 9.7.1-1 illustrates the SEAL APIs for spatial map management.

Table 9.7.1-1: List of SEAL APIs for spatial map management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_SmManagement | Create | VAL server, SEAL SM client | Request/Response |
|  | Get | VAL server, SEAL SM client |  |
|  | Update | VAL server, SEAL SM client |  |
|  | Delete | VAL server, SEAL SM client |  |
|  | Subscribe | VAL server, SEAL SM client | Subscribe/Notify |
|  | Notify | VAL server, SEAL SM client |  |
|  | Unsubscribe | VAL server, SEAL SM client |  |
| SS_SmDiscovery | Request | SEAL SM client, VAL server | Request/Response |
| SS_SmLocalization | Request | VAL server, SEAL SM client | Request/Response |
| SS_SmDataSourceRegistration | Request | SEAL SM client | Request/Response |
|  | Update | SEAL SM client |  |
|  | Deregister | SEAL SM client |  |
| SS_SmDataSourceDiscovery | Request | VAL server | Request/Response |
|  | Notify | SEAL SM client | Notify |
| SS_SmDataSourceSubscription | Subscribe | VAL server | Subscribe/Notify |
|  | Notify | VAL server |  |
|  | Update | VAL server |  |
|  | Unsubscribe | VAL server |  |
| SS_SmSmasRegistration | Request | VAL server | Request/Response |
|  | Update | VAL server |  |
|  | Deregister | VAL server |  |

# * * * * Next change * * * *

### 9.7.x	SS_SmDataSourceSubscription API

#### 9.7.x.1	General

Service description: This API enables the VAL server to subscribe to the SEAL SM server to get the updates related to changes in SM data source profiles.

#### 9.7.x.2	Subscribe operation

API operation name: SS_SmDataSource_Subscribe

Description: Subscription to get the updates related to SM data source profiles.

Known Consumers: VAL server.

Inputs: Refer Table 9.5.6.3.1-1

Outputs: Refer Table 9.5.6.3.2-1

See clause 9.5.6.2.1 for the details of usage of this API operation.

#### 9.7.x.3	Notify operation

API operation name: SS_SmDataSource_Notify

Description: Notifies the subscribers of the changes to the SM data source profiles.

Known Consumers: VAL server.

Inputs: Refer Table 9.5.6.3.3-1

Outputs: None

See clause 9.5.6.2.2 for the details of usage of this API operation.

#### 9.7.x.4	UpdateSubscription operation

API operation name: SS_ SmDataSource _UpdateSubscription

Description: The consumer updates an existing subscription for SM data source profiles.

Inputs: Refer Table 9.5.6.3.a-1.

Outputs: Refer Table 9.5.6.3.b-1.

See clause 9.5.6.2.x for details of usage of this operation.

#### 9.7.x.5	Unsubscribe operation

API operation name: SS_ SmDataSource_Unsubscribe

Description: The consumer cancels an existing subscription for SM data source profiles.

Inputs: See clause 9.5.6.3.c-1.

Outputs: See clause 9.5.6.3.d-1.

See clause 9.5.6.2.y for details of usage of this operation.

# * * * * End change * * * *