3GPP TSG-SA WG6 Meeting #70	S6-255157

Dallas, United States 17th – 21st November 2025	(revision of S6-254307)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0679 | rev | 3 | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MC service UE remote management – Disable procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, UK Home Office, BDBOS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC_Ph2-MC |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 3GPP TS 22.280 has requirements related to remote management of MC service UE and which are not fulfilled in stage 2. Below are the requirements [R-6.13.4-001] The MCX Service shall support suspending or disabling of access from an MCX UE or an MCX User to the MCX Service. [R-6.13.4-003] The MCX Service shall provide a mechanism to temporarily disable an MCX UE remotely by the MCX Service Administrator or an authorized MCX User. Also paper S6-252045 discussed in details about the need for having a standardized solution to address the stage 1 requirements. This paper provides the solution for disabling the MC service UE temporarily by an authorized MC service user. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause 10.x to add the procedures for remote management (disable procedure) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This is an important feature which has market need and also clear Stage 1 requirements. If not approved, stage 1 requirements are not met which leads to incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.x(new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 10.X	Disabling and Re-enabling of MC service UE

### 10.X.1	General

Remote management of MC service UE provides a mechanims for the MC service administrator or authorized MC service user to  disable the MC service UE temporarily from accessing the MC services or to re-enable the MC service UE which is temporarily disabled. When the MC service UE is temporarily disabled, the MC service client logs off from the MC services and enters into the temporarily disabled state. When the MC service UE is in temporarily disabled state, the MC service server shall allow only the MC service UE re-enable operation. This subsclause describes the generic procedures for disabling and re-enabling of the MC service UE by authorized MC service user.

### 10.X.2	Information flows

#### 10.X.2.1	MC service UE disable request (MC service client – MC service server)

Table 10.X.2.1-1 describes the information flow MC service UE disable request from MC service client to an MC service server and from MC service server to the target MC service client.

Table 10.X.2.1-1: MC service UE disable request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The identity of the MC service user requesting to disable the MC service UE. |
| MC service ID | M | The identity of the MC service user whose MC service UE to be disabled |
| MC service UE identifier | M | Unique identifier to identity the target MC service UE that needs to be disabled. |

#### 10.X.2.2	MC service UE disable response (MC service server – MC service client)

Table 10.X.2.2-1 describes the information flow MC service UE disable response from MC service server to requesting MC service client and from the target MC service client to the MC service server

Table 10.X.2.2-1: MC service UE disable response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Indicate the MC service ID of the target user whose device is being disabled. |
| Success result | O (see NOTE 1) | Indicates MC service UE disable request is successful and also indicates whether the disable request is delivered to the target MC service UE or not. |
| Failure result | O (see NOTE 1) | Indicates that MC service UE disable request is failure and the reason. |
| NOTE 1: Only one of these IE shall be present. |  |  |

* * * Next Change * * * *

### 10.X.3	Disabling MC service UE

Figure 10.X.3-1 below illustrates the procedure of disabling the MC service UE by authorized MC service user or MC service administrator.

Figure 10.X.3-1: Disabling MC service UE by the authorized MC service user

1.	MC service client 1 of the authorized user retrieves the list of MC service UEs currently logged into by the target MC service user whose MC service UE has to be disabled by using the procedure specified in clause 10.Y and identifies the MC service UE and the corresponding binding information of the MC service UE that needs to be disabled.

2.	MC service client 1 sends the request to the MC service server to disable the MC service UE of MC service client 2. This request shall include the MC service identity of the requesting MC service user, the MC service identity of the target user whose MC service UE has to be disabled and a unique identifier to identify the MC service UE which has to be disabled.

3.	MC service server checks whether MC service client 1 is authorized to perform the MC service UE disable operation.

4.	MC service server stores the information that the MC service UE identified by the identifier provided in step 2 is disabled. MC service server shall not allow the MC service client residing on the disabled UE to access the MC services. If the MC service UE disable request is not delivered to the target MC service client due to connectivity issues, the MC service server shall attempt to re-send the request on periodic basis or when the MC service client becomes online.

5.	MC service server sends the MC service UE disable request to the particular MC service UE based on the UE identifier received in step 2.

6.	MC service client 2 acknowledges the receipt of MC service UE disable request by sending the MC service UE disable response to the MC service server.

7.	MC service client 2 logs off from the MC services automatically without user intervention and enters into the temporarily disabled state activating the access token which is scoped only for the re-enablement operation.

Editor’s Note: Handling of MC service UE disable request by the MC service client to enter into temporarily disabled state is in SA3’s scope.

8.	MC service server sends the MC UE disable response to the MC service client which requested to disable the MC service UE.