3GPP TSG-SA6 Meeting #70	S6-255161

# Dallas, USA, 17th - 21st Nov 2025 	                              		       (revision of S6-25xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0713 | rev | - | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Small corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Airbus |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2025-11-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Quality improvements for TS 23.280. Missing words, wrong reference numbers, duplicate references. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lower TS quality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.2.9.1, 7.5.2.6, 9.2.2.1.4, 9.2.2.1.6, 10.1.5.6.1.2, 10.7.3.11.2, 10.16.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.179: "Mission Critical Push to Talk (MCPTT); Stage 1".

[3]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[4]	3GPP TS 22.281: "Mission Critical Video services".

[5]	3GPP TS 22.282: "Mission Critical Data services".

[6]	3GPP TS 23.002: "Network Architecture".

[7]	3GPP TS 23.179: "Functional architecture and information flows to support mission critical communication services; Stage 2"

[8]	3GPP TS 23.203: "Policy and charging control architecture".

[9]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[10]	3GPP TS 23.237: "IP Multimedia Subsystem (IMS) Service Continuity; Stage 2".

[11]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[12]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[13]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[14]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[15]	3GPP TS 23.335: "User Data Convergence (UDC); Technical realization and information flows".

[16]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[17]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[18]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[19]	3GPP TS 29.283: "Diameter Data Management Applications".

[20]	Void

[21]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[22]	IETF RFC 5245 (April 2010): "Interactive Connectivity Establishment (ICE): A Protocol for Network Address Translator (NAT) Traversal for Offer/Answer Protocols".

[23]	GSMA PRD IR.92 v10.0: "IMS Profile for Voice and SMS".

[24]	GSMA PRD IR.88 v15.0: "LTE and EPC Roaming Guidelines".

[25]	3GPP TS 33.180: "Security of the mission critical service".

[26]	IETF RFC 6733 (October 2012): "Diameter Base Protocol".

[27]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

[28]	3GPP TS 22.011: "Service accessibility".

[29]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[30]	3GPP TS 25.305: "Stage 2 functional specification of User Equipment (UE) positioning in UTRAN".

[31]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[32]	IETF RFC 2865 (June 2000): "Remote Authentication Dial In User Service (RADIUS)".

[33]	IETF RFC 3162 (August 2001): "RADIUS and IPv6".

[34]	3GPP TS 25.501: "System architecture for the 5G System (5GS)".

[35]	IETF RFC 1541 (October 1993): "Dynamic host configuration protocol".

[36]	IETF RFC 8415 (November 2018): "Dynamic host configuration protocol for IPv6 (DHCPv6)".

[37]	IETF RFC 9330:"Low Latency, Low Loss, Scalable Throughput (L4S) Internet Service: Architecture".

[38]	Void

[39]	3GPP TS 23.380: "IMS Restoration Procedures".

[40]	IETF RFC 7866 (May 2016): "Session Recording Protocol"

[41]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

# * * * * 2nd change * * * *

#### 5.2.9.1	General

Migration provides means for the MC service user to obtain MC services from a partner MC system. As the MC service client receives one or multiple user profiles for the MC service user from its primary MC system (as described in clause 10.1.4.3). Each user profile contains a list of partner MC systems that the user is permitted to migrate to, along with necessary access information to facilitate service authentication, hence, facilitate migration to the partner MC system.

MC service interconnection needs to be provided between MC systems that wish to provide migration of their MC service users.

Migration of MC service users between any two MC systems can be on a bilateral or unilateral basis.

NOTE 1:	Whether migration is bilateral or unilateral between any two MC systems is left to business agreements between the two MC service providers, and is outside the scope of the present document.

Migration may be triggered by the primary MC system or may be requested by the MC service client due to its geographical changes.

Upon a successful service authorization for migration, i.e., once the MC service user is authorized to migrate to a partner MC system, its primary MC system marks the user as migrated, is informed of the target partner MC system, and records the corresponding MC service ID of the migrated user provided by the partner MC system. Further details are described in clause 10.6.3.

The functional alias of a migrated MC service user at a partner MC system, which is defined in the partner MC system, can be utilized as a target address in private communication as described in clause 10.16.3.

During migration of an MC service user to a partner MC system, media of the MC service user's calls may need to be routed to the MC service user's primary MC system e.g. for recording purposes.

NOTE 2:	Recording of migrated users is out of the scope of this version of the present document.

Based on the choice of PLMN and IMS utilization (as described in clause 5.2.9.2 and clause 5.2.9.3), if the migration results in having the HSS-IMS and HSS-EPS to be across two different administrative domains, as in clause 9.2.2.1.4 and clause 9.2.2.1.6, then IMS features such as, e.g., T-ADS (3GPP TS 23.237 [10]) related to service continuity between on-network and UE-to-network relay as in annex B, NPLI (3GPP TS 23.228 [9]) related to lawful interception, and P-CSCF restoration (3GPP TS 23.380 [39]) are not supported.

# * * * * 3rd change * * * *

#### 7.5.2.6	Reference point CSC-5 (between the MC service server and the configuration management server)

The CSC-5 reference point, which exists between the MC service server and the configuration management server, provides a means for the MC service server to obtain non-group MC service-related information (e.g. policy information). The CSC-5 reference point shall use HTTP-1 reference point and HTTP-2 reference point for transport and routing of non-subscription/notification related signalling. The CSC-5 reference point shall use SIP-2 and SIP-3 reference points for transport and routing of subscription/notification related signalling. The SIP-3 reference point is used when a MC service server and the configuration management server, are served by different SIP cores.

# * * * * 4th change * * * *

##### 9.2.2.1.4	MC service provider administers SIP core, separate from EPS

In this scenario, as illustrated in figure 9.2.2.1.4-1, the MC service provider administers the SIP core, and the MC services and SIP core are independent of the PLMN operator.

Figure 9.2.2.1.4-1: MC service provider provision of SIP core, separate domain from EPS

The MC service provider may require that all identities and other sensitive information at the application services layer are hidden from the EPS. The MC service provider need not hide the identities and signalling at the application services layer from the SIP core. However the MC service provider may require that identities and other sensitive information between SIP core and SIP client in the MC service UE are also hidden from the EPS.

All authentication and authorisation mechanisms, including security roots, at both application services layer and at SIP signalling plane may need to be hidden from, and not available to, the PLMN operator.

The HSS-IMS and HSS-EPS in this deployment scenario is across two different administrative domains, then IMS features such as, e.g., T-ADS (3GPP TS 23.237 [10]) related to service continuity between on-network and UE-to-network relay as in annex B, NPLI (3GPP TS 23.228 [9]) related to lawful interception, and P-CSCF restoration (3GPP TS 23.380 [39]) are not supported.

# * * * * 5th change * * * *

##### 9.2.2.1.6	PLMN operator administers SIP core with SIP identities administered by MC service provider

In this scenario, the PLMN operator administers the SIP core. However, the identities used by the SIP core (IMPI and IMPU) for MC service UEs served by the MC service provider are provided from the SIP database of the MC service provider.

Figure 9.2.2.1.6-1: MC service provider provides identities to PLMN operator SIP core

The MC service provider may require that all identities and signalling at the application services layer are hidden from the SIP core and EPS.

When required by the MC service provider, all authentication and authorisation mechanisms, including security roots, at the application services layer may need to be hidden from, and not available to, the PLMN operator.

The security roots (authentication keys) required for access to the signalling control plane are not available to the PLMN operator as these are held in the MC service provider's SIP database. However, derived parameters e.g. authentication vectors are provided to the SIP core to allow signalling control plane authentication to take place.

The HSS-IMS and HSS-EPS in this deployment scenario is across two different administrative domains, then IMS features such as, e.g., T-ADS (3GPP TS 23.237 [10]) related to service continuity between on-network and UE-to-network relay as in annex B, NPLI (3GPP TS 23.228 [9]) related to lawful interception, and P-CSCF restoration (3GPP TS 23.380 [39]) are not supported.

# * * * * 6th change * * * *

###### 10.1.5.6.1.2	Subscribe group dynamic data response

Table 10.1.5.6.1.2-1 describes the information flow subscribe group dynamic data response from the MC service server to the MC service client and from the MC service server to the group management server and from the MC service server to the location management server. This information flow from the MC service server to the MC service client is sent individually addressed on unicast or multicast.

Table 10.1.5.6.1.2-1: Subscribe group dynamic data response

| Information element | Status | Description |
| --- | --- | --- |
| MC service group ID | M | The MC service group ID for which dynamic data is requested. |
| Status | M | Success or failure of the request |

# * * * * 7th change * * * *

##### 10.7.3.11.2	FEC encoding by the BM-SC

In this procedure, depicted in figure 10.7.3.11.2-1, the MC service server asks the BM-SC to apply FEC to a set of medias, transported by a MBMS bearer, using the Setup FEC request.

This procedure can be applied when using pre-established MBMS bearers (10.7.3.1) or dynamic MBMS bearers (10.7.3.2).

Pre-condition:

1.	The MC service server has already activated a MBMS Bearer, with the MBMS Bearer request specified in 3GPP TS 23.468 [18].

Figure 10.7.3.11.2-1: Application of FEC by the BM-SC

1.	The MC service server decides to set up FEC for a set of MC service media flows. The request is done on the MB2-C reference point. It includes the following elements: the TMGI of the bearer transporting those media, the media descriptions (codecs, transport protocols, bitrates, destination ip addresses and ports), the identification of the FEC repair packet flow (IP destination and port), an upper bound to the additional latency resulting to FEC application.  The MC Service server may perform this request several times to protect separately different sets of media transported within the same MBMS bearer.

2.	If the BM-SC can satisfy the request, the Setup FEC response includes a modified list of media information and FEC information. The response also includes an identifier to the FEC process instance, which can be used to release the application of FEC for these media flows.

NOTE 1:	Source media packets may be modified by the application of FEC (e.g. addition of a footer of header), leading to a modification of the delivery protocol to be announced within the media information.

NOTE 2:	The Release FEC request is not shown on the figure.

3.	The MC service server announces the MBMS bearer to the MC service client with the MBMS bearer announcement procedure, including the modified list of medias information and FEC information within the SDP information.

4.	When the MC service server decides to transmit the MC service media flow for a group communication, the MC service server sends to the group a message identifying the MC service media flow and the TMGI of the MBMS bearer, such as the MapGroupToBearer message for MCPTT, specified in 3GPP TS 23.379 [16], or the MapGroupToBearer message for MCVideo, specified in 3GPP TS 23.281 [12].

5.	The MC service server sends the downlink media to the BM-SC on the MB2-U reference points and according to 3GPP TS 23.468 [18].

6. The BM-SC performs FEC encoding of the downlink media in accordance to the announced FEC algorithm and parameters and delivers it over MBMS.

7. The MC service client performs FEC decoding of the encoded media flows in accordance to the announced FEC information and delivers the decoded flows to the media player.

Editor's note: the need for a MC service server ability to turn off/on the production of repair packets on the BM-SC during the media transmission must be discussed, and may impact the procedure.

# * * * * 8th change * * * *

#### 10.16.5.2	Private call redirection

Table 10.16.5.2-1 describes the information flow of a private call redirection, which is sent from the MC service server to an MC service client initiating a private call towards a migrated MC service user.

Table 10.16.5.2-1: Private call redirection

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the MC service user initiating a private call, i.e., calling party. The MC service ID can either be MCPTT ID, MCVideo ID, or MCData ID. |
| MC service ID | M | The MC service ID of the target MC service user (i.e., called party), which the MC service user has obtained from its primary MC system before migration. The MC service ID can either be MCPTT ID, MCVideo ID, or MCData ID. |
| MC service ID | M | The MC service ID of the target MC service user, which the MC service user has obtained from its migrated MC system after Migration. The MC service ID can either be MCPTT ID, MCVideo ID, or MCData ID. |
| KMS URI (NOTE) | O | The KMS URI associated with the MC service ID of the migrated MC service user. |
| Redirection reason | O | The MC service server informs the calling party that the target MC service user has migrated. |
| NOTE: If the KMS URI is not configured in the MC service user profile and is not included in the private call redirection message, then the MC service client shall obtain KMS URI using the KMS lookup procedure defined in 3GPP TS 33.180 [25]. |  |  |

# * * * * End of changes * * * *