3GPP TSG-SA WG6 Meeting #70	S6-255172

Dallas, USA, 17th – 21st November 2025	(revision of S6-25xxxx)

Source:	Huawei, Hisilicon

Title:	Pseudo-CR on update to solution#11

Spec:	3GPP TR 23.700-26 0.2.0

Agenda item:	9.8

Document for:	Approval

Contact:	Cuili Ge – gecuili@huawei.com

1. Introduction

This pCR propose to update the solution#11 to capture that the AIoT enabler server could aggregate multiple requests and only sent one single request towards the 3GPP network.

2. Reason for Change

In solution #11 general solution description part, it mentions “the application enablement layer must coordinate and aggregate concurrent requests from multiple applications.”. However, this is not well reflected in the step description.

This pCR is proposed to update the step 4 to reflect this point.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-26 v0.2.0.

* * * First Change * * * *

7.11.3.1	Provision and monitor AIoT device presence

Figure 7.11.3.1-1 illustrates the procedure for the AIoT application enabler to interact with the 5GS for the support of the Ambient IoT App service that leverages the corresponding 5GC NF services and APIs as specified in the TS 23.369 [3]. The VAL server sends the ‘location information request’ indication to AIoTApp, along with the potential monitoring schedule. The AIoTApp sends the received request, or based on local configuration, to the 5GC. Once the requested AIoT device information (e.g., location information) is received from the 5GC, the AIoTApp enabler can do more enhanced information processing (e.g., presence verification, location tracking) before sending the information to the VAL server (i.e., AIoT service consumer).

Pre-condition:

-	The VAL server discovers and selects the AIoTApp enabler by CAPIF functions.

-	Multiple AIOTFs may be discovered and selected in the 5GS.

-	AIOTFs have determined to provide the AIoT device location information to AF based on operator policy.

Figure 7.11.3.1-1: Provision and monitor AIoT device presence

1.	The VAL server decides to use the AIoT application enabler (AIoTApp) to collect the AIoT device information and allocates address/port as AIoTApp-S Data transmission connection information for receiving the AIoT data packets from the AIoTApp enabler. The VAL server sends the AIoT_app_service request to the AIoTApp enabler. The type of AIoT service request is Inventory. The service request includes VAL server ID, VAL service ID, and the AIoT service related parameters (e.g., AF ID, [External Target Area information], [information about the target AIoT Device(s)], [Approximate number of AIoT Devices], [time interval], location information requested). The VAL server may also send AIoT device provisioning and monitoring parameters (e.g., device location tracking request, monitoring schedule (e.g., every hour for 24 hours), device-id/a group of device-IDs). The details of AIoT service related parameters are described in 3GPP TS 23.369 [3].

As specified in the clause 5.8 of 3GPP TS 23.369 [3], Information about the target AIoT Device(s) may include Filtering Information that might be constructed by one or multiple components (i.e. ID Type, PLMN Identifier, NID, third party identifier and Identification Information). These components can be used by the AIoTApp enabler to identify a particular AIoT device or a group of devices.

2.	Upon receiving the request, the AIoTApp enabler performs an authorization check. If authorization is successful, the AIoTApp enabler responds with the AIoT_app_service response.

3.	After the AIoTApp enabler receives from the VAL server both the AIoT service related parameters and AIoT device provisioning and monitoring parameters, depending on the local configuration, the AIoTApp enabler sends AIoT service requests to 5GC, with the indication ‘location information requested’.

The AIoTApp may send concurrently the requests for a group of AIoT device-IDs to multiple discovered and selected AIOTF NFs in 5GC.

4.	The AIoTApp sends Inventory service operations towards the (selected) AIOTF(s), as in 3GPP TS 23.369 clause 6.2.

There could be multiple AIOTF NFs selected from the step-3. for concurrent AIoT device monitoring operations.

If the geographical area parameters of multiple operations point to the same network target service area, the AIoT App enabler aggregates those AIoT device operation requests and initiate a single Inventory service API invocation (with the parameter being the network service area) towards the 3GPP network. If multiple operations under the same network target service area include different AIoT device IDs which are covered by the same filter(s), the AIoT App enabler further processes those device IDs using a masking mechanism and then sends a single Inventory service API invocation with the filter(s) towards the 3GPP network.

NOTE 2:	All the invoked service operations, together with the service parameters, conform to what have been specified in TS 23.369. E.g., depending on the trustiness of the AF (i.e., AIoTApp), AIoTApp sends either External Target Area information (to NEF as an untrusted AF) or Target Area information (to AIOTF as a trusted AF).

NOTE 3:	The AF (i.e., AIoTApp) can send the parameter ‘location information requested’ to AIOTF in 5GC to request the AIOTF to report the location information of AIoT devices.

The 5GS processes the AIoT service request(s) and collects the AIoT device information based on the provisioned service parameters, or local configuration. The AIoT device location information can be reported to the AF (i.e., AIoTApp) as described in the TS 23.369 [3]. (Multiple) AIOTF(s) send collected AIoT device information to AIoTApp (or via NEF). This information may include the location information of an AIoT device, depending on operator policy and the requested parameter from the AIoTApp.

5.	The AIoTApp enabler processes the received AIoT device information, e.g.

-	If the VAL server has requested the AIoTApp to monitor the presence of an AIoT device and the AIoTApp receives the device information for the AIoT device, then AIoTApp reports the presence to the VAL server.

-	If the VAL server has requested the AIoTApp to monitor the presence of an AIoT device and the AIoTApp does not receive the device information for the AIoT device from 5GC, then AIoTApp may expand the service request in more Target Areas that have been received in step-1 from the VAL server. The step 4 of the procedure will be repeated to monitor the presence of the AIoT device.

-	If the VAL server has requested the AIoTApp to track the historical location information of an AIoT device, then AIoTApp will record the received location information of the AIoT device. Based on the monitoring schedule received in step-1, the AIoTApp enabler may repeat the step 4 of the procedure to continue tracking the location of the device. When the monitoring schedule is fulfilled, the AIoTApp can send the tracked historical location information of the AIoT device to the VAL server.

6.	The AIoTApp enabler sends the processed AIoT device information via the AIoT_app_service notification to the VAL server.