3GPP TSG-SA WG6 Meeting #70	S6-255176

Dallas, United States 17th – 21st November 2025	(revision of S6-255xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.482 | CR | 0058 | rev | -- | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Registration and discovery of VAL client as AI/ML member |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI-20 |  |  |  |  |  | Date: |  |  | 2025-10-6 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the existing TS,there are description about VAL client as AI/ML member for some AI operation acccording to data operations (data preparation and processing) performed by VAL clients in section 6.14, the following description in 8.6.2: “8.6.2 Procedures for AI/ML task transfer Due to various reasons (e.g., changes of available resource, changes of available time), an AI/ML member (e.g., AIMLE Client, VAL Client) finds itself cannot finish the assigned AI/ML task during the performing process. Here AI/ML member means the participant of an AI/ML task. To save resource and time, the AI/ML member (source AI/ML member) decides to transfer the intermediate AI/ML information (e.g., the intermediate AI/ML operation status and results) to another AI/ML member (target AI/ML member) for further operations to complete the AI/ML task.” However, the description about how VAL client can be discovered and selected by AIMLE server is not clear. To solve the above issue,the following approach is suggested: - Although it is decribed in 5.2.1, the AIMLE client provides functionality to the VAL client(s) over AIML-C reference point ,it should be also possible that AIMLE client can be implemented by VAL client. Thus, VAL clients which intergrate AIMLE clients can supported all the functions required for being discovered and selected as AI/ML task member e.g., to make registration for been discovered by acting as AIMLE client. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding a note in 5.2.1 to capture the approach of AIMLE client been implemented as part of VAL client. - Correcting the un-exsting case about VAL server discove rthe AIMLE client, and Add some text about how to support VAL client discovery in 6.7. - There is no any interafce between VAL server and a seperated AIMLE client entity to support the potential AI/ML task operation requested by VAL server following AIMLE client selection. So rewording the description of 6.12 to make it linked to VAL client intergaration case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of the FL member registration is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4, 8.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************************Begin of the changes**********************************

5.2	Application enablement architecture

5.2.1	On-Network AIML Enablement (AIMLE) Functional Architecture

Figure 5.2.1-1: On-network AIMLE functional model

Figure 5.2.1-1 illustrates the on-network functional model of AIMLE. In the vertical application layer, the VAL client communicates with the VAL server over VAL-UU reference point. VAL-UU supports both unicast and multicast delivery modes. The AIMLE functional entities on the UE and the server are grouped into AIMLE client(s) and AIMLE server(s) respectively.

The AIMLE includes of a common set of services for comprehensive enablement of AIML functionality, including federated and distributed learning (e.g., FL client registration management, FL client discovery and selection), and reference points. The AIMLE services are offered to the vertical application layer (VAL).

The AIMLE client communicates with the AIMLE server(s) over the AIML-UU reference points. The AIMLE client provides functionality to the VAL client(s) over AIML-C reference point. The VAL server(s) communicate with the AIMLE server(s) over AIML-S reference points. The AIMLE servers communicate with the underlying 3GPP network systems using the respective 3GPP interfaces specified by the 3GPP network system. AIML-E reference point enables interactions between two AIMLE servers (e.g. central and edge AIMLE servers).

NOTE:	AIMLE client can be implemented as a separated software and provide APIs to VAL client over AIML-C as the above. It can also be implemented as part of VAL client.

The AIMLE server interacts with the ML repository which serves as repository for ML model and ML participants over AIML-R.

******************************Begin of the change**********************************

6.7	Support for AIMLE client discovery

This functionality enables the AIMLE server to discover available AIMLE clients for AI/ML operations, such as training or inference. The AIMLE server provides the functionality to select suitable AIMLE clients that fulfill the discovery criteria.

NOTE:	AIMLE client discovery can be also used to discover VAL clients, which integrate the AIMLE clients as part of the VAL client software.

******************************Begin of the change**********************************

6.12	Support AIMLE client selection subscription and notification

This functionality is related to the AIMLE client (being implemented as part of VAL client) selection subscription request and notification to enable VAL Servers to subscribe for monitoring AIML members who meet criteria for performing an AIMLE service, selecting AIMLE clients and receiving notification when there is an update on the selected and re-selected AIML client’s status when re-selection is performed according to AIML member selection criteria.

NOTE:	The AIMLE client selection subscription and notification to VAL server is used for the case where VAL server needs to find some VAL clients (integrating AIMLE client), which can be used by VAL server to request certain AI task operation.

******************************End of the change**********************************