3GPP TSG-SA WG6 Meeting #70	S6-255194

Dallas, United States 17th – 21st November 2025	(revision of S6-255xxx)

Source:	Nokia

Title:	Pseudo-CR on Solution to KI#3

Spec:	3GPP TR 23.700-43 (v0.2.0)

Agenda item:	9.11

Document for:	Approval

Contact:	Sapan Shah (Sapan.shah@nokia.com)

1. Introduction

This contribution provides solution to KI#3.

2. Reason for Change

In TR 23.700-43, KI#3 is agree with following open issue:

“This key issue will study approaches to managing entities (i.e., API Invoker or API Provider) without a valid CCF issued and signed certificate, specifically how such entities can be deleted from the CCF including associated resources to the entities like Security_Context associated to API Invokers and API resources published by APFs served by AEFs.”

In order to manage the entities, it is required to have valid certificates. And so, if the entities do not have valid certificate (either it lost or corrupted), the entity needs to obtain reissued certificate from the CCF. The certificate is issues to the API invoker while onboarding and to the API provider domain function while registering. The certificate life cycle management is within scope of SA3. It is proposed that the key issue is considered solved and coordination with SA3 is initiated to request them to study this key issue further.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-43 (v0.2.0).

* * * First Change * * * *

## 6.x	Solution #x: Solution for certificate unavailability

### 6.x.1	General

This solution relates to KI #3.

### 6.x.2	Architecture Impacts

This solution does not impacts CAPIF architecture.

### 6.x.3	Solution description

The KI#3  has following open issue: “This key issue will study approaches to managing entities (i.e., API Invoker or API Provider) without a valid CCF issued and signed certificate, specifically how such entities can be deleted from the CCF including associated resources to the entities like Security_Context associated to API Invokers and API resources published by APFs served by AEFs.”

In order to manage the entities, it is required to have valid certificates. And so, if the entities do not have valid certificate (either it is lost or corrupted), the entity needs to obtain reissued certificate from the CCF. The certificate is issues to the API invoker while onboarding and to the API provider domain function while registering. The certificate life cycle management is within scope of SA3.

#### 6.x.3.1	Information flows

The solution does not introduce any new information flow.

### 6.x.4	Solution evaluation

The solution is related to KI#3. It is studied that to manage the CAPIF entities within CCF a valid certificate is required. The cerfiticate is issued and managed by SA3, and so this solution proposes that the key issue should be further studied in SA3 scope.

* * * End of Change * * * *