3GPP TSG-SA WG6 Meeting #70	S6-255217

Dallas, United States 17th – 21st November 2025	(revision of S6-255xxx)

Source:	Lenovo

Title:	Architecture enhancements

Spec:	3GPP TR 23.700-83 v1.1.0

Agenda item:	9.4

Document for:	Approval

Contact:	Manos Pateromichelakis (epateromiche@lenovo.com)

1. Introduction

This paper provides enhancements to clause related to AIMLE architecture.

2. Reason for Change

This paper provides enhancements to clause related to AIMLE architecture, to remove the ENs and complete the clause.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v1.1.0.

* * * First Change * * * *

# 6	Enhanced architecture for enabling AI/ML services

## 6.1	General

## 6.2	Enhanced AI/ML enablement functional model

In this clause the architecture enhancements relate to the support for multi-operator services. There are two cases for the multi-operator service support:

- An AIMLE server connected to AIMLE clients over different PLMNs

- Each AIMLE server connected to AIMLE clients of single PLMN, whereas the AIMLE server interact each other via AIML-E.

In the first scenario, the AIML service (VAL service, ASP service) runs in a certain service area, which can be an edge or cloud service area or geographical area, where UEs connected to different PLMNs are present. Such service can be considered multi-operator service. VAL UEs are assumed to have AIMLE clients installed and active and connect to AIMLE server via different PLMN.

In Figure 6.2.1-1, the VAL UE#1 is connected to PLMN#1 and VAL UE#2 to PLMN#2. Both UEs are connected to the same AIMLE server. In this scenario, different VAL server may connect to UE#1 VAL client and different VAL server to UE#2; however, both VAL servers are connected to the same AIMLE server.

Figure 6.2.1-1 architecture for multi-operator support with common AIMLE server

In the second scenario, as depicted in Figure 6.2.1-2, the VAL UEs are assumed to have AIMLE clients installed and active and connect to different AIMLE servers provided by different MNOs or trusted 3rd parties of the MNOs. In Figure 6.2.1-2, each AIMLE server connects to each PLMN. The VAL UE 1 is connected to PLMN1 and AIMLE server 1 and VAL UE 2 is connected to PLMN 2 and AIMLE server 2.

Figure 6.2.1-2 architecture for multi-operator support with distributed AIMLE servers

NOTE: The cardinality of ML repositories will be further explored in normative phase.

### 6.2.2	AI/ML enablement functional model for roaming scenarios

This clause describes the AIMLAPP architecture for roaming UEs. This architecture is aligned with network roaming architecture, since it re-uses existing network roaming principles and describes the AIML service continuity while VAL UEs are roaming; hence how AIMLE spans across HPLMN and VPLMN is discussed. Such architecture applies to both roaming models (LBO and HR roaming architectures).

In Figure 6.2.2-1, the AIMLAPP architecture for enabling AI/ML services for roaming scenarios is showcased. In this architecture, AIML-UU is the interface between the AIMLE client and the AIMLE server, which can be over HPLMN or VPLMN which VAL UE is roaming. AIML-E supports the interaction among AIMLE servers deployed by different MNOs, whereas the MLR-E is the new interface supporting the interaction among repositories for aligning the information on the ML models and ML/FL members to allow the continuity of the AIML service while the VAL UE is roaming.

Figure 6.2.2-1 architecture for roaming support

## 6.3	Functional Elements

The functional elements have been defined in clause 5.2.3 of 3GPP TS 23.482. No additional functional element has been identified.

## 6.4	Reference Points

The reference points have been defined in clause 5.2.4 of 3GPP TS 23.482.

The additional or enhanced reference points are described below. The enhancements over existing reference points are shown in bold italics.

### 6.4.1	AIML-E

The interactions related to AIML enablement functions between AIMLE servers (e.g., central and edge AIMLE servers, or AIMLE servers of different PLMNs).

### 6.4.2	AIML-R

The interactions related to AIML enablement functions between the AIMLE server, and the ML repository are supported by AIML-R reference point. These interactions can be also between the ML repository and AIMLE servers connected to different PLMNs in multi-operator scenarios.

### 6.4.3	AIML-PC5

The interactions related to AIML enablement functions between AIMLE clients in off-network deployments, and in scenarios where split AIMLE services are provided using relay VAL UEs.

### 6.4.3	MLR-E

The interactions related to ML repository functions between ML repositories in distributed multi-operator deployments are supported by AIML-R reference point.

* * * Next Change * * * *