3GPP TSG-SA WG6 Meeting #70	S6-255223

Dallas, United States 17th – 21st November 2025	(revision of S6-255xxx)

Source:	China Mobile

Title:	Pseudo-CR on Solution of supporting virtual number in MMTel

Spec:	3GPP TR 23.700-57 v0.4.0

Agenda item:	9.6

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

This pCR provides the solution of Open issue#1 of KI#5 on Support of virtual number.

This pCR proposes this new solution for TS23.700-57.

The capabilities used in this solution may be specified by SA2 in FS_NG_RTC_Ph3 (i.e. SP-250855) study under WT#2 as:

WT#2: Study on IMS and DC capability exposure enhancements

-	WT#2.1: How to enhance IMS capability exposure framework to expose IMS capability for eMMTel services (voice/video) and related events besides Data Channel, supporting IMS session establishment, modification and release to enable eMMTel services that may require enhancing audio/video services with DC capabilities;

In support of IMS capability exposure for eMMTel services (voice/video) as depicted above, study how to enhance Rel-19 IMS event exposure framework and event definition to support IMS DC events supported by DCSF (e.g. application downloading event) and non-DC events, including subscriber specific and non-subscriber specific supported by IMS AS, (e.g. subscriber registration event, or event of all UEs calling a particular customer support number and that have DC capabilities for customization of customer support to UE capabilities);

2. Reason for Change

This pCR provides the solution of Open issue#1 of KI#5 on Support of virtual number.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-57 v0.4.0.

* * * First Change * * * *

6.X	Solution #X: Supporting virtual number in MMTel

6.X.1	Description

This solution resolves Key Issue #5: Support of virtual number.

The user or Application Server configures the virtual number and related use mode on MMTel Enabler Server. The MMTel Enabler Server subscribes the call events related to the real number and virtual number of the user or Application Server by using IMS DC capability APIs defined in 3GPP TS 23.228 [4] or NEF exposed IMS session management capabilitiesspecified in 3GPP TS 23.502 [3].

If the MMTel Enabler Server finds that the virtual number is needed to be used when a mmtel call event is notified, the MMTel Enabler Server invokes the IMS session modification APIs provided by IMS AS to request the IMS AS to modify the number(s) in the mmtel call session.

NOTE:	the virtual number can only be provided by the MMTel Enabler Server in the trusted domain of the mmtel service provider.

Editor's note:	whether the modification of caller or callee’s number can be supported by the IMS session management APIs depends on the SA2’s ongong NG_RTC_Ph3 study. .

6.x.2	Procedures

#### 6.x.2.1	Virtual number configuration

Figure 6.x.2.1-1 illustrates the procedure of a user or Application Server configuring the virtual number.

Pre-conditions:

1.	The MMTel Enabler Server is authorized to use IMS DC capability APIs as defined in 3GPP TS 23.228 [4] or authorized to use the NEF exposed IMS session management capabilities as specified in 3GPP TS 23.502 [3].

2.	The MMTel Enabler Client in the UE has connected to the serving network successfully. Both the MMTel Enabler Client and MMTel Enabler Server have been configured with the necessary credentials to enable authenticating one another.

Figure 6.x.2.1-1: Virtual number configuration

1a.	The MMTel Enabler Client in the UE sends a Set virtual number request to the MMTel Enabler Server; or

1b	The Application Server sends a Set virtual number request to the MMTel Enabler Server.

The request message in step 1a and 1b include information elements as specified in Table 6.x.2.1-1.

Table 6.x.2.1-1: Information elements in Set virtual number request

| Information element | Status | Description |
| --- | --- | --- |
| Originating ID (see NOTE) | M | The identifier of the UE or Application Server which requests the virtual number |
| Virtual number requirements template | O | A virtual number requirements template that contains the requirements of using virtual number(s) from originating UE or Application Server, e.g. how to use the virtual number, when the virtual number is intended to be used, etc. If this IE is absent, the MMTel Enabler Server will assign a virtual number to the Originating UE or Application Server based on the default service policy. |
| >required use mode of virtual number | O | The use mode of virtual number required by the UE or Application Server. |
| > Specific Callee ID(s) (see NOTE) | O | One or multiple identifier(s) of the terminating UE(s) or Application Server(s) which the virtual number is intended to be used. |
| > Virtual number valid time | O | The time period when the virtual number is intended to be used. |
| NOTE: The Originating ID and Specific Callee ID may be MSISDN based identifier. One of the Originating ID or Specific Callee ID may also be a private identifier in the Application domain. If Originating ID or Specific Callee ID is not MSISDN based identifier, the MMTel Enabler Server will translate it into a routable ID in the IMS domain and record the mapping relationship between the Originating ID/Terminating ID and the routable ID. |  |  |

2.	The MMTel Enabler Server verifies that the sender is authorized to request the virtual number.

3.	If the sender is authorized to request the virtual number, the MMTel Enabler Server assigns the virtual number(s) to the originating UE or Application Server and decides how to use the virtual number(s), i.e. use mode, based on the Virtual number requirements template. The MMTel Enabler Server stores the virtual number(s) and related use mode.

4a.	The MMTel Enabler Server sends a Set virtual number response to the MMTel Enabler Client in the UE; or

4b.	The MMTel Enabler Server sends a Set virtual number response to the Application Server.

The response message in step 1a and 1b include information elements as specified in Table 6.x.2.1-2.

Table 6.x.2.1-2: Information elements in Set virtual number response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indication if the Set virtual number is success or failure |
| Virtual number (see NOTE 1) | O | The virtual number(s) used by the originating UE or Application Server in the mmtel call session. |
| >Virtual number valid time | O | The time period when the virtual number is allowed to be used. |
| >Virtual number use mode | O | The use mode of virtual number. |
| Failure Cause (see NOTE 2) | O | The reason for failure |
| NOTE 1: This IE shall only be present when the Set virtual number is Success. NOTE 2: This IE shall only be present when the Set virtual number is Failure. |  |  |

5.	The MMTel Enabler Server Updates the subscriber’s data of the originating UE or Application Server to the IMS by invoking the IMS Services provided by SBI capable HSS specified in Annex AA.2.1 of 3GPP TS 23.228 [4].

NOTE:	The subscriber’s data of the originating UE or Application Server is needed to be updated to both the IMS serves the real number and virtual number of the originating UE or Application Server.

6.	The MMTel Enabler Server subscribes call event related to the real number and virtual number of the originating UE or Application Server by invoking the IMS AS Services specified in Annex AA.2.4 of 3GPP TS 23.228 [4].

Editor's note:	Whether the APIs used in step 5 and step 6 can be supported by IMS, including DCSF and HSS, depends on the ongoing SA2 study FS_NG_RTC_Ph3 and is FFS. Coordination with SA2 is needed.

6.x.2.2	Establishment of MMTel call session using virtual number

Figure 6.x.2.2-1 illustrates how to establish MMTel call session using virtual number.

Figure 6.x.2.2-1: Establishment of MMTel call session using virtual number

1.	A mmtel call session related to real number or virtual number goes to IMS AS. IMS AS notifies the MMTel Enabler Server (which acts as the Data Channel Application Server) either directly as specified in Annex AA.2.4 of 3GPP TS 23.228 [4], or indirectly via the Network Exposure Function (NEF) as defined in clause 5.2.6.39 of 3GPP TS 23.502 [3].

2.	The MMTel Enabler Server checks whether virtual number is needed to be used in this mmtel call session and decides how to use the virtual number.

3.	The MMTel Enabler Server invokes the IMS session modification APIs provided by IMS AS to request the IMS AS to modify the number(s) in the mmtel call session.

NOTE:	The IMS modifies the necessary SIP headers, e.g. From or To header, in the SIP request/response based on the IMS session modification request.

4.	The IMS AS to modify the number(s) in the mmtel call session and the mmtel call session is established by the IMS using the numbers provided by the MMTel Enabler Server.

NOTE:	The mmtel call session establishment procedures specified in 3GPP TS 23.228 [3] apply to step 4.

Editor's note:	Whether the APIs used in step 1 and step 3 can be supported by IMS, including DCSF and HSS, depends on the ongoing SA2 study FS_NG_RTC_Ph3 and is FFS. Coordination with SA2 is needed.

8.2.3	Impacts on Existing Nodes and Functionality

In this solution, the APIs and procedures use the basic capability exposured by IMS core which is under studied by SA2.