3GPP TSG-SA WG6 Meeting #70	S6-255224

Dallas, United States 17th – 21st November 2025	(revision of S6-255xxx)

Source:	China Mobile

Title:	Pseudo-CR on Solution on  Alignment 8.4.4 and 8.4.5 service flows with SA2 Rel-19 support for IMS Capability Exposure Framework

Spec:	3GPP TR 23.700-57 v0.4.0

Agenda item:	9.6

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

This pCR provides the solution of KI#1: Alignment MMTel service flows with SA2 Rel-19 support for IMS Capability Exposure Framework. This solution addresses the  procedures in clause 8.4.4 and 8.4.5 of 3GPP TS 23.392.

This pCR proposes this new solution for TS23.700-57.

2. Reason for Change

This pCR provides the solution of KI#1: Alignment MMTel service flows with SA2 Rel-19 support for IMS Capability Exposure Framework.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-57 v0.4.0.

* * * First Change * * * *

6.x	Solution #x: Alignment 8.4.4 and 8.4.5 service flows with SA2 Rel-19 support for IMS Capability Exposure Framework

6.x.1	Description

This solution resolves KI#1: Alignment MMTel service flows with SA2 Rel-19 support for IMS Capability Exposure Framework. This solution addresses the  procedures in clause 8.4.4 and 8.4.5 of 3GPP TS 23.392 [2] in Rel-20.

6.x.2	Procedures

6.x.2.1	Update to existing procedures in 8.4.4

The following changes are proposed to clause 8.4.4. (New text is highlighted in bold italics, and removed text is marked with a remove mark.)

Editor’s note: Whether this procedure needs to be further updated depends on whether more IMS session management capabilities will be provided by the IMS/Core Network. This is FFS.

8.4.4 Establishing an Application Call with Data Channel capability (A2P) 8.4.4.1 Procedure The MMTel Enabler Server may also act as trusted AF and interact with 5GC/IMS directly by using IMS DC capability APIs as defined in 3GPP TS 23.228 [3] or acts as untrused AF and interact with 5GC/IMS via NEF by using NEF exposed IMS session management capabilities as specified in 3GPP TS 23.502 [12]. Figure 8.4.4-1 illustrates the operation to establish an Application Call with Data Channel capability between the Application and the MMTel Enabler Server in order to add an A2P Data Channel to an existing IMS session of the user on UE1 by using NEF exposed IMS session management capabilities as specified in 3GPP TS 23.502 [12]in alignment with the A2P flow specified in 3GPP TS 23.228 [3]. Pre-conditions: 1. The Application can use DC (supports the media set information for DC) and is authorized to use the service API provided by the MMTel Enabler Server. 2. The MMTel Enabler Server is authorized to use IMS DC capability APIs as defined in 3GPP TS 23.228 [3]. 3. The MMTel Enabler Server is authorized to use the NEF exposed IMS session management capabilities as specified in 3GPP TS 23.502 [12]. 4. The MMTel Enabler Server is subscribed to the AEF (NEF) for IMS session events information e.g., media changes such as requested for the specific DC Application, for a list of users. NOTE: How the IMS session ID is provided by the NEF exposure of IMS events in 3GPP TS 23.502 [12] is out of scope of this specification. 5. The DC Application Server is subscribed to the MMTel Enabler Server for events related to the DC application. 6. An IMS MMTel session is ongoing for User (UE1) with another user who requests the DC Application to provide some DC media to the user on UE1. Figure 8.4.4-1: Establishing an Application Call with Data Channel capability 1. The DC Application server determines to add A2P application data channel(s) to an existing IMS session and sends request to MMTel Enabler Server. The request includes parameters such as the session ID, MDC2 endpoint, A2P type and Media operation set (e.g.Media Type, Media Parameter set) as specified in Annex AA.2.4.4.3 of 3GPP TS 23.228 [3]. 2. The MMTel Enabler Server requests an IMS session update to the NEF or IMS AS, providing the Session ID and MDC2 endpoint address Media operation set. The NEF propagates the request to the IMS core which sets the media operation. The NEF sends the response to the MMTel Enabler to the IMS session Update request. 3. The MMTel Enabler Server sends the response to the DC Application Server including the Session ID for the Add media request. 4. Once the IMS system completed the media preparation for the IMS session, based on the notification from the IMS, the NEF or IMS AS sends an IMS session management notification request to the MMTel Enabler Server with MDC2 resource information for the Session ID and corresponding Media resource information (e.g.remote media address for MDC2 interface). 5. The MMTel Enabler Server notifies the DC Application Server of the MDC2 resource information for the Session ID and corresponding Media resource information (e.g.remote media address for MDC2 interface). 6. The DC Application Server responds (acknowledges) the MMTel Enabler Server notification. 7. The MMTel Enabler Server acknowledges the NEF notification or IMS AS notification. Post-condition: The UE1 upon getting the updated IMS session information for the DC media proceeds to download the DC Application. 8.4.4.2 Information flows 8.4.4.2.1 Application Add Data Channel media to UE1 session (A2P) request Table 8.4.4.2.1-1: Application Add Data Channel media to UE1 session (A2P) request Information element; Status; Description. Data Channel APPID; M; The identifier of the DC application.. Session ID; M; The session identifier of the session on which MMTel Enabler Server must add the new media.. Media resource information; O; The media information to be passed via the Data Channel. It includes information described in 3GPP TS 23.502 [12] clause 5.2.6.40.. Application Profile requested; O; The DC application profile expected to be used in this call.. Notification address; M; The address where the call related notification, e.g. whether the call between UE A and UE B is established successfully or not, is sent to.. 8.4.4.2.2 Application Add Data Channel media to UE1 session (A2P) response Table 8.4.4.2.2-1: Application Add Data Channel media (A2P) response Information element; Status; Description. Session ID; M; The session identifier of the session on which MMTel Enabler Server must add the new media.. Result; M; Indicates the Notification of Application of DC media info (A2P) request was successful or failure.. > Cause; O; Indicates the cause of request failure.. 8.4.4.2.3 Notify Application of DC media info (A2P) request Table 8.4.4.2.3-1: Notify Application of DCH media info (A2P) request Information element; Status; Description. Session ID; M; The session identifier of the session on which MMTel Enabler Server must add the new media.. Notification address; M; The address where the call related notification, e.g. whether the call between UE A and UE B is established successfully or not, is sent to.. Media resource information; O; The media information to be passed via the Data Channel. It includes information described in 3GPP TS 23.502 [12] clause 5.2.6.40.. Application Profile requested; O; The DC application profile expected to be used in this call.. 8.4.4.2.4 Notify Application of DC media info (A2P) response Table 8.4.4.2.4-1: Notify Application of DC media info (A2P) response Information element; Status; Description. Session ID; M; The session identifier of the session on which MMTel Enabler Server must add the new media.. Result; M; Indicates the Notification of Application of DC media info (A2P) request was successful or failure.. > Cause; O; Indicates the cause of request failure..

6.x.2.2	Update to existing procedures in 8.4.5

The following changes are proposed to clause 8.4.5. (New text is highlighted in bold italics, and removed text is marked with a remove mark.)

Editor’s note: Whether this procedure needs to be further updated depends on whether more IMS session management capabilities will be provided by the IMS/Core Network. This is FFS.

8.4.5 Application calling service API with Data Channel capability 8.4.5.1 Procedure 8.4.5.1.1 Establishing an Application Call with Data Channel capability (P2A) The MMTel Enabler Server may also act as trusted AF and interact with 5GC/IMS directly by using IMS DC capability APIs as defined in 3GPP TS 23.228 [3] or acts as untrused AF and interact with 5GC/IMS via NEF by using NEF exposed IMS session management capabilities as specified in 3GPP TS 23.502 [12]. Figure 8.4.5.1.1-1 illustrates the operation to establish an Application Call with Data Channel capability between a user (on UE1) and a Data Chanel (DC) Application via the MMTel Enabler Server by using NEF exposed IMS session management capabilities as specified in 3GPP TS 23.502 [12]. Pre-conditions: 1. The Application can use DC (supports the media set information for DC) and is authorized to use the service API provided by the MMTel Enabler Server. 2. The MMTel Enabler Server is authorized to use IMS DC capability APIs as defined in 3GPP TS 23.228 [3]. 3. The MMTel Enabler Server is authorized to use the NEF exposed IMS session management capabilities as specified in 3GPP TS 23.502 [12][8]. 4. The MMTel Enabler Server is subscribed to the AEF (NEF) for IMS session events information for the specific DC Application (using the DCApplication Called party ID). sNOTE: How the IMS session ID is provided by the NEF exposure of IMS events in 3GPP TS 23.502 [12] is out of scope of this specification. 5. The DC Application Server is subscribed to the MMTel Enabler Server for events related to the DC application. 6. An IMS MMTel session is ongoing for User (UE). Figure 8.4.5.1.1-1: Establishing a Application Call with Data Channel capability (P2A) 1. The user (on UE1) sends a request, via the 5GC and IMS, to add DC media for a specific DC Application. 2. The P2A session establishment is handled via 5GC and IMS system. 3. NEF or IMS AS notifies the MMTel Enabler Server of the new IMS session event for the DC Application. 4. The MMTel Enabler Server notifies the DC Application Server of the new request for DC media. 5. The DC Application Server accepts the request and provides the MDC2 media resource requested. 6. The MMTel Enabler Server utilizes the IMS session update capability exposed by the NEF as specified in 3GPP TS 23.502 [12][8] or utilizes the IMS session update capability exposed by as defined in 3GPP TS 23.228 [3] to request addition of the DC media to the existing IMS MMTel Call Session. 7. The NEF further interacts with the IMS system to request the DC media addition to the ongoing MMTel session and IMS further handles this as specified in Annex AC of 3GPP TS 23.228 [3]. Post-condition: The Application data channel is activated. 8.4.5.2 Information flows 8.4.5.2.1 Notify DC Application of MDC2 media (P2A) request Table 8.4.5.2.1-1: Notify DC Application of MDC2 media (P2A) request Information element; Status; Description. Session ID; M; The session identifier of the session on which MMTel Enabler Server must add the new media.. Media resource information; O; The MDC2 media information requested by the UE1 to be sent over the Data Channel. It includes information described in 3GPP TS 23.502 [12] clause 5.2.6.40.. ##### 8.4.5.2.2 Notify DC Application of MDC2 media (P2A) response Table 8.4.5.2.2-1: Notify DC Application of MDC2 media (P2A) response Information element; Status; Description. Session ID; M; The session identifier of the session on which MMTel Enabler Server must add the new media.. Result; M; Indicates the Notification of Application of DC media info (A2P) request was successful or failure.. > Cause; O; Indicates the cause of request failure..

6.x.3	Impacts on Existing Nodes and Functionality

The MMTel Enabler Server may also act as trusted AF and interact with 5GC/IMS directly by using IMS DC capability APIs as defined in 3GPP TS 23.228 [3] or acts as untrused AF and interact with 5GC/IMS via NEF by using NEF exposed IMS session management capabilities as specified in 3GPP TS 23.502 [12].

* * * Next Change * * * *

7.2	Mapping of Solutions to Key Issues

Table 7.2-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 | #6 |
| #1 |  | X |  |  |  |  |
| #2 |  |  |  | X |  |  |
| #x | X |  |  |  |  |  |