3GPP TSG-SA WG6 Meeting #70	S6-255227

Dallas, United States 17th – 21st November 2025	(revision of S6-25xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0335 | rev | - | Current version: | 19.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Improvements on CAPIF framework description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL_Ph4-APP |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are two types API exposing functions supported in 3GPP API frame system: 1) Border AEF for interacting with external API invoker normally deployed in API gateway. 2) API service logic execution AEF for processing service logical requested by API invoker(s) normally supported by any network/service layer function entity which support exposed services(e.g.,locaion service, QoS gurantee for application traffic transmission). However,there are serveral issues existing in current TS which may cause confusion as the following: - The defintion and function element description of AEF does not cover API service logic execution. - No clear description for distinguishing two types of AEF in the archieture which will cause the cofusion about how to implement and deploy different features of AEF in the real expsoure system. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Adding the description of service execution AEF to the CAPIF reference architeture in 6.2 to make it distinguishing-able with entry point AEF. 2. Add the description in annex for the mapping between different 3GPP entities with the service execution AEF to guide the system implementention. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The incompleted description of API framework system will cause some confusion how to use to API framework in the exposure system. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1,3.2,6.2.0, 6.2.1, 6.2.2, 6.2.3, 6.3.4,6.3.x(new),6.4.12, anne X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /****************************** First changes **********************/

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

API: The means by which an API invoker can access the service.

API invoker: The entity which invokes the CAPIF or service APIs.

API invoker profile: The set of information associated to an API invoker that allows that API invoker to utilize CAPIF APIs and service APIs.

API exposing function:  The entity which provides the service communication entry point for the service APIs when invoked by third party.

Service execution API exposing function :The entity which performs the execution of service logical requested by a service API invoker.

API exposing function location: The location information (e.g. civic address, GPS coordinates, data center ID) where the API exposing function providing the service API is located.

API category: A name used to group service APIs by the service domain in which they are used (e.g., IoT, V2X). The CAPIF core function maintains a list of API category names.

API management function: The entity which enables the API provider to perform administration of the service APIs.

API publishing function: The entity that enables the API provider to publish the Service APIs information in order to enable the discovery of APIs by the API invoker.

Authorization function: The entity which issues access tokens to the API invokers after successfully authenticating the resource owner and obtaining authorization.

CAPIF administrator: An authorized user with special permissions for CAPIF operations.

Common API framework: A framework comprising common API aspects that are required to support service APIs.

Designated CAPIF core function: The CAPIF core function which is configured as the serving CAPIF core function for interconnection.

Network Slice Info: Network slice information of service API which include a list of network slice related identifiers, as described in clause 8.4 of 3GPP TS 23.435 [13]).

Northbound API: A service API exposed to higher-layer API invokers.

Onboarding: One time registration process that enables the API invoker to subsequently access the CAPIF and the service APIs.

Resource: The object or component of the API on which the operations are acted upon.

Resource owner: A UE user or an MNO subscriber capable of granting access to a protected resource related to the invoked API via resource owner function.

Resource owner function: The entity that enables the authorization for resource access and managing and revoking authorization for resource access.

Resource owner-aware northbound API access: An API invocation scenario where the API invoker needs an authorization from the resource owner.

Service API: The interface through which a component of the system exposes its services to API invokers by abstracting the services from the underlying mechanisms.

Serving Area Information: The location information for which the service APIs are being offered to.

CAPIF provider domain: A domain that contains an instance of CAPIF core function and may contain API provider domains and API invokers. The CAPIF provider could be a PLMN, SNPN or 3rd party. Throughout this document, PLMN trust domain is often used as the typical deployment of a CAPIF provider domain however SNPN trust domain or 3rd party trust domain are applicable as well.

PLMN trust domain: The entities protected by adequate security and controlled by the PLMN operator or a trusted 3rd party of the PLMN.

SNPN trust domain: The entities protected by adequate security and controlled by the SNPN operator or a trusted 3rd party of the SNPN. 3rd party trust domain: The entities protected by adequate security and controlled by the 3rd party.

Application management client: The application developers utilize the CAPIF APIs using an application management client as an API invoker to obtain the service APIs information to implement the application program. Such application programs are hosted on cloud, edge or on a UE.

Hosted applications: The hosted applications (which are programmed to utilize the service APIs) as an API invoker invoke the CAPIF APIs and service APIs as per the application business logic.

Channel Aggregator Platform: The Channel Aggregator aggregates the APIs from one or more southbound CAPIF providers with the intention to re-expose such APIs or expose value-added APIs developed using the APIs from the southbound CAPIF providers to the northbound side Application management client or Hosted applications. The Channel Aggregator Platform as API invoker invokes the CAPIF APIs and service APIs as per its business logic.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 32.240 [6] apply:

Offline charging

Online charging

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GS	5G System

AEF	API Exposing Function

AF	Application Function

AMF	API Management Function

APF	API Publishing Function

API	Application Program Interface

AS	Application Server

BM-SC	Broadcast Multicast Service Centre

CAPIF	Common API Framework

CDR	Charging Data Record

CRUD	Create, Read, Update, Delete

DDoS	Distributed Denial of Service

E-UTRA	Evolved Universal Terrestrial Radio Access

EPS	Evolved Packet System

ETSI	European Telecommunications Standards Institute

GS	Group Specification

IP	Internet Protocol

MBMS	Multimedia Broadcast and Multicast Service

MEC	Multi-access Edge Computing

NEF	Network Exposure Function

NGSI	Next Generation Service Interfaces

NR	New Radio

OMA	Open Mobile Alliance

OAM	Operations, Administration and Maintenance

OWSER	OMA Web Services

PC	Protocol Converter

PLMN	Public Land Mobile Network

REST	REpresentational State Transfer

RNAA	Resource owner-aware Northbound API Access

RO	Resource Owner

ROF	Resource Owner Function

RPC	Remote Procedure Call

SCEF	Service Capability Exposure Function

SCS	Service Capability Server

SNPN	Stand-alone Non-Public Network

UDDI	Universal Description, Discovery and Integration

URI	Uniform Resource Identifier

WSDL	Web Services Description Language

S-AEF	Service execution API exposing function

# *********************************Next change************************************

6.2.0	Functional model description for the CAPIF

Figure 6.2.0-1 shows the reference point based functional model for the CAPIF.

Figure 6.2.0-1: Functional model for the CAPIF

The CAPIF is hosted within the PLMN operator network (or even an SNPN). The API invoker is typically provided by a 3rd party application provider who has service agreement with PLMN operator. The API invoker may reside within the same trust domain as the PLMN operator network.

In a reference point based model, the API invoker within the PLMN trust domain interacts with the CAPIF via CAPIF-1 and CAPIF-2. The API invoker from outside the PLMN trust domain interacts with the CAPIF via CAPIF-1e and CAPIF-2e. The API exposing function, service execution API exposing function, the API publishing function and the API management function of the API provider domain (together known as API provider domain functions) within the PLMN trust domain interacts with the CAPIF core function via CAPIF-3, CAPIF-4 and CAPIF-5 respectively.

When API exposing function is deployed as the service communication entry point for the service APIs, the API exposing function may interact with service execution AEF (service execution API exposing function entities) over CAPIF-7.

In 3GPP exposure system, 3GPP service execution AEF respresent any 3GPP function entity (e.g., an OAM function entity, or Core network function entity, or an application enabler layer service entity ), as Annex I.

Figure 6.2.0-2: Functional model for interactions between API exposing functions

As illustrated in figure 6.2.0-2, the interactions between the API exposing functions within the PLMN trust domain is via CAPIF-7.

The CAPIF core function provides CAPIF APIs to the API invoker over CAPIF-1 and CAPIF-1e. The API exposing function provides the service APIs to the API invoker over CAPIF-2 and CAPIF-2e.

NOTE 1:	The communication between the API exposing function and the CAPIF core function, between the API publishing function and the CAPIF core function and between the API management function and the CAPIF core function over CAPIF-3, CAPIF-4 and CAPIF-5 respectively can be API based.

The detailed information of the APIs provided by the CAPIF core function is specified in clause 10.

The security aspects of CAPIF reference points are specified in 3GPP TS 33.122 [12].

Figure 6.2.0-3 illustrates the CAPIF functional model using service-based interfaces.

Figure 6.2.0-3: CAPIF functional model representation using service-based interfaces

Table 6.2.0-1 specifies the service-based interfaces supported by CAPIF.

Table 6.2.0-1: Service-based interfaces supported by CAPIF

| Service-based interface | Entity | APIs offered |
| --- | --- | --- |
| Cccf | CAPIF core function | Specified in subclause 10 |
| Caef | API exposing function | Specified in subclause 11 |

6.2.1	Functional model description to support 3rd party API providers

Figure 6.2.1-1 shows the functional model for the CAPIF to support 3rd party API providers.

Figure 6.2.1-1: Functional model for the CAPIF to support 3rd party API providers

The CAPIF core function in the PLMN trust domain supports service APIs from both the PLMN trust domain and the 3rd party trust domain having business relationship with PLMN. The API invokers may exist within the PLMN trust domain, or within the 3rd party trust domain or outside of both the PLMN trust domain and the 3rd party trust domain. The API provider domain 1 offers the service APIs from the PLMN operator. The API provider domain 2 offers the service APIs from the 3rd party. When the 3rd party API provider is a trusted 3rd party of the PLMN, the API provider domain 1 also offers the service APIs from the 3rd party.

The API invoker 2 within the PLMN trust domain interacts with the CAPIF core function via CAPIF-1, and invokes the service APIs in the PLMN trust domain via CAPIF-2 and invokes the service APIs in the 3rd party trust domain via CAPIF-2e. The API invoker 3 within the 3rd party trust domain interacts with the CAPIF core function via CAPIF-1e, and invokes the service APIs in the PLMN trust domain via CAPIF-2e and invokes the service APIs in 3rd party trust domain via CAPIF-2. The API invoker 1 from outside the PLMN trust domain and 3rd party trust domain, interacts with the CAPIF core function via CAPIF-1e and invokes the service APIs in the PLMN trust domain and the service APIs in the 3rd party trust domain via CAPIF-2e.

The API exposing function, the API publishing function and the API management function of the API provider domain 1 within the PLMN trust domain interacts with the CAPIF core function via CAPIF-3, CAPIF-4 and CAPIF-5 respectively. The API exposing function, the API publishing function and the API management function of the API provider domain 2 within the 3rd party trust domain interacts with the CAPIF core function in the PLMN trust domain via CAPIF-3e, CAPIF-4e and CAPIF-5e respectively. The API exposing function within the PLMN trust domain and the 3rd party trust domain provides the service APIs to the API invoker, offered by the respective trust domains.

The interactions between the API exposing functions within the PLMN trust domain is via CAPIF-7 (not shown in the figure 6.2.1-1 for simplicity). The API exposing function within the PLMN trust domain interacts with the API exposing function in the 3rd party trust domain via CAPIF-7e.

NOTE 1:	The communication between the API exposing function and the CAPIF core function, between the API publishing function and the CAPIF core function and between the API management function and the CAPIF core function over CAPIF-3/3e, CAPIF-4/4e and CAPIF-5/5e respectively can be API based.

The detailed information of the APIs provided by the CAPIF core function is specified in clause 10.

NOTE 2:	The security aspects of CAPIF reference points are under SA3 responsibility and out of scope of the present document.

6.2.2	Functional model description to support CAPIF interconnection

Figure 6.2.2-1 shows the architectural model for the CAPIF interconnection which allows API invokers of a CAPIF provider to utilize the service APIs from the 3rd party CAPIF provider.

Figure 6.2.2-1: High level functional architecture for CAPIF interconnection with multiple CAPIF provider domains

Figure 6.2.2-2 shows the architectural model for the CAPIF interconnection within the same CAPIF provider domain, which allows API invokers of CAPIF core function 1 to utilize the service APIs from CAPIF core function 2, where both CAPIF core function 1 and CAPIF core function 2 are hosted within the trust domain of the CAPIF provider A.

Figure 6.2.2-2: High level functional architecture for CAPIF interconnection within a CAPIF provider domain

The CAPIF provider A and CAPIF provider B host the CAPIF in their trust domains. A business relationship exists between the CAPIF providers.

The CAPIF providers in their respective trust domain hosts multiple CAPIF instances where each CAPIF instance consists of the CAPIF core function (local), the API provider domain and the API invokers. All interactions within the CAPIF instance is according to the functional model specified in clause 6.2.0.

When multiple CAPIF instances are deployed by a CAPIF provider there may be a hierarchy associated with the multiple CAPIF core function deployed which allows:

-	the designated CAPIF core function of the CAPIF provider A to interconnect with the designated CAPIF core function of the CAPIF provider B; and

-	within CAPIF provider A, one or more CAPIF core function interacts with the designated CAPIF core function 1.

The designated CAPIF core function of the CAPIF provider A provides the information about the CAPIF instances and service APIs deployed by the CAPIF provider A to the designated CAPIF core function of the CAPIF provider B and vice versa over CAPIF-6e reference point.

The CAPIF core function 2 of CAPIF provider A provides the information about the service APIs to the CAPIF core function 1 over CAPIF-6 reference point.

NOTE 1:	Void

The API invokers may exist within the trust domain of CAPIF provider A, or within the trust domain of CAPIF provider B or outside of the trust domains of both CAPIF provider A and CAPIF provider B. The API invoker of a CAPIF provider is onboarded with the CAPIF core function in the corresponding trust domain of the CAPIF provider.

NOTE 2:	For sake of simplicity, the service API interactions of API invokers of the CAPIF provider B are not shown. From each CAPIF provider's perspective the other CAPIF provider is a 3rd party.

One or more CAPIF core function can publish service APIs to the designated CAPIF core function over CAPIF-6 reference point and, also discover the service APIs from the designated CAPIF core function and vice versa over CAPIF-6 reference point.

The API invoker within the trust domain of CAPIF provider A interacts with the CAPIF core function of the CAPIF provider A via CAPIF-1 and discovers the service APIs of both CAPIF providers, and invokes the service APIs in the trust domain of CAPIF provider A via CAPIF-2 and invokes the service APIs in the trust domain of CAPIF provider B via CAPIF-2e. The API invoker from outside the trust domain of CAPIF providers, interacts with the CAPIF core function of th CAPIF provider A via CAPIF-1e and invokes the service APIs in the trust domain of the CAPIF providers via CAPIF-2e.

NOTE 3:	The communication between the CAPIF core function of the CAPIF providers over CAPIF-6 or CAPIF-6e can be API based.

The detailed information of the APIs provided by the CAPIF core function is specified in clause 10.

NOTE 4:	The security aspects of CAPIF reference points are under SA3 responsibility and out of scope of the present document.

NOTE 5:	All interactions among entities within the CAPIF provider domains (regardless if CAPIF is deployed in a PLMN, SNPN or 3rd party network) are ruled by the functional model in clause 6.2.0, the support of 3rd party API providers is as in clause 6.2.1, whereas the interconnection among CCFs is according to this clause.

NOTE 6:	RNAA for CAPIF interconnection is not specified in this release of the specification.

6.2.3	Functional model description to support RNAA

Figure 6.2.3-1 shows the architectural model for the RNAA which allows the resource owner to provide authorization to access their resources through Service API invocation.

Figure 6.2.3-1: High level functional architecture for CAPIF supporting RNAA

The authorization function is an internal entity of the CAPIF core function and performs API invoker authorization based on the API invoker authorization policies available in the CCF and the RO authorization information obtained via the ROF.

The resource owner function interacts with the authorization function in the CAPIF core function via CAPIF-8. The authorization function in the CAPIF core function interacts with the resource owner function to obtain the resource owner authorization. The CAPIF core function (authorization function) uses resource owner authorization information obtained via the ROF as part of the processing of the authorization requests received from API invokers.

The API exposing function (e.g., NEF, SCEF) interacts with the authorization function in the CAPIF core function via CAPIF-3. The API exposing function acts as an authorization enforcement point according to the authorization token received from the API invoker.

NOTE 1:	RNAA support is not dependant of the access network, i.e., the RNAA support is not restricted to 5G networks.

The API invoker interacts with authorization function in the CAPIF core function via CAPIF-1/CAPIF-1e.

NOTE 2:	In the current release, 3rd party API providers (i.e., API providers outside the PLMN trust domain) are not supported for RNAA.

NOTE 3:	The interaction between resource owner function and CCF over CAPIF-8 is not specified in the current release of the specification.

NOTE 4:	The authorization information from the resource owner used by CCF (described in 3GPP TS 33.122 [12]) is independent from the user consent information used from user subscription data at UDM/UDR (described in Annex V of 3GPP TS 33.501 [16]). In the current release of 3GPP specifications, no synergy between CCF and UDM is specified.

The security aspects of CAPIF supporting RNAA are specified in 3GPP TS 33.122 [12].

Editor's note:	The details for CAPIF-8 procedures are dependent on SA3 specification and the related NOTEs in this specification will be re-visited accordingly.

# /****************************** Next changes **********************/

6.3.4	API exposing function

The API exposing function is the provider of the service APIs and is also the service communication entry point of the service API to the API invokers. The API exposing function consists of the following capabilities:

-	Authenticating the API invoker based on the identity and other information required for authentication of the API invoker provided by the CAPIF core function;

-	Requesting the revocation of the API invoker authorization;

-	Enforcing the authorization provided by the CAPIF core function;

-	Supporting authorization revocation;

-	Logging the service API invocations at the CAPIF core function; and

-	Hiding the topology of the PLMN trust domain from API invokers, depending on configured policy. Routing the API invoking messages to/from service execution AEF, acting as API invoker of CAPIF-7/7e activiate the service execution function.

# /****************************** Next changes **********************/

6.3.x	Service execution API exposing function

The service execution function is responsible for executing service logical process as requested by service API consumer., e.g tracing a UE’s location and dermining whether it moves out of certain area if requested by a location  API invoker via API exposing function. Different service execution AEF supports different type of service API logical associated to the service API.

Service execution API exposing function enables the following:

-Functions required by CAPIF-7/7e for processing API messages from/to API exposing function from a API exposing function, if the dedicate API exposing function is deployed for the reasons of topology hiding, simplifying third party’s implemenion for invoking different service APIs from an API expsoure system. Or

-Performing API service logical as requested by API invokers and providing the desired output(e.g., location information) or activating desired operation (e.g., activating the QoS scheduling on application traffic, or tracing a UE).

- It may additional support the functions of API exposing functions to resolving the interaction with third party via CAPIF-2/2e if no dedicated entities are deployed for supporting the system level API exposing function.

# /****************************** Next changes **********************/

6.4.12	Reference point CAPIF-7 (between the API exposing functions)

The CAPIF-7 reference point, which exists between the API exposing functions and the service execution API exposing functions belonging to the same trust domain, is used for the forwarding or routing of the API invoker's service API invocation from one API exposing function to a service execution API exposing functions deployed in the PLMN trust domain.

The CAPIF-7 reference point supports all the functions required for MNO internal API invocation:

- Authentication and authorization mechanism for an API invocation over SBA interface as defined in 3GPP TS 33.501

The CAPIF-7 reference point supports invocation of service APIs originated by the API invoker using CAPIF-2.

NOTE 1:	The aspects related to the specific service API invocation in reference point CAPIF-7 are out of scope of the present document.

NOTE 2:	The security aspects of CAPIF-7 are the responsibility of SA3.

# /****************************** Next changes **********************/

Annex X(informative):3GPP exposure system

Annex X.1 Layered representation of 3GPP network exposure system

X.1.1 General

The figure H1.1-1 is to show how CAPIF system can be used for 3GPP system providers to expose their services supported by different domain.

FigureH.1-1: Layered representation of SEAL service layer in 3GPP network exposure system

NOTE1:	In figure H1.1-1, it assumes that enabler layer clients belong to the same enabler service provider as enabler layer servers. If enabler layer clients are deployed by different party, it should invoke enabler layer APIs via API exposing function and apply to the CAPIF-1/1e.

X.1.2	Functional Elements

X1.2.1	3GPP Exposure layer

3GPP Exposure layer acts entry points for exposing services APIs supported by 3GPP system, to a services APIs customer (the customer can be Application layer/or a third party's re-exposure system). This layer mainly includes API exposing function and CAPIF core function.

The APIs exposed by this layer including CAPIF-1 and CAPIF-2 APIs defined in clause 10 and 11, and also services APIs for requesting the services from service execution functions in different layer and different domains.

X1.2.2	Service execution AEFs at 3GPP service layer

3GPP service layer includes the legacy MNO services like IMS, SMS, and 3GPP application enabler layer services.

3GPP application enabler layer provides value added service based on network layer capabilities (including capabilities supported by Core network and OAM). it provides the following services based on generality level:

- Edge application enabler layer services as described in H1.2.2.1.

- SEAL service layer as described in H1.2.2.2.

- Vertical specific services layer as described in H1.2.2.3.

NOTE2:	In some organizations, this layer is also called value added service layer or service abstraction layer.

X1.2.2.1 Edge application enabler layer

The services and APIs provided by EEC/EES/ECS as defined in 3GPP TS 23.558[20] at this layer, enable an application on mobile UE to discover and select an edge application server (EAS) deployed at Edge site (at or close to 3GPP network border), and application server rediscovery and reselection due to UE mobility or overloaded.

The services and APIs provided by ECSP management system as defined in3GPP TS 28.538 [19] at this layer, enable an Entity within ECSP or third party to trigger EAS instantiation on Edge nodes.

X1.2.2.2 SEAL service layer

The services at this layer defined in 3GPP TS 23.434[3], 3GPP TS23.435[4], 3GPP TS23.436[5], 3GPP TS23.437[6], 3GPP TS23.438[7], 3GPP TS23.482[12] provide the value-added services to enable application layer to utilize 3GPP network and UE capabilities in an easy-to-use way. SEAL services and APIs are generic to any application type.

SEAL layer includes SEAL services server(s) and SEAL client(s).

- SEAL services server(s): it is implemented in network by MNO or 3rd part and are authorized to (1) provide APIs to application and SEAL client, and (2) invoke network APIs to get 3GPP network services.

- SEAL client(s): it is implemented in UE and can be provided by same organization as SEAL service server or other origination e.g., UE vendor/OS and are authorized to (1) provide APIs to application, (2) invoke APIs from SEAL service server to get Network services, or (3) trigger UE modem to initiate NAS procedures to invoke network capability so as to provide service result to application.

X1.2.2.3 Vertical specific services layer

A service at this layer provides certain application layer service functions, which is specific for certain vertical application.

In 3GPP, the application specific service includes 3GPP TS 23.255 [22] for UAV, or 3GPP TS 23.286 [21] for V2X.

X1.2.2.4 User communication service

It provides another type of service layer services, which mainly refers to the legacy user-orient operator applications, like IMS voice, SMS. These applications can also be exposed as the services to a third-party application.

The APIs specified in 3GPP includes 3GPP TS 23.228 [24], or 3GPP TS 23.239 [23]

X1.2.3	Service execution AEFs at 3GPP network connection layer

The smallest granularity services that a 3GPP network layer function entity or OAM function entity, or a 3GPP UE is able to support e.g., 3GPP network connectivity services, QoS control, 3GPP position, slice creation/updating/deletion.

Network functions and capabilities defined at this layer are designed generally to serve multiple applications/scenarios.

NOTE:	The principles of network layer design normally consider how to use one network capability to support multiple scenarios. So normally the capabilities defined at this layer is in a small granularity. These capabilities can be used as element capabilities and been further aggregated to a larger granularity added value service.

In this layer, it also includes the NAS procedure supported by both UE and network layer so as to provide service result to application by UE.

X.1.3	Relationship between different layer

APIs from 3GPP network connection layer and APIs from SEAL service server of application enabler layer can be provided by MNO and offered certain services towards application layer via a unified API framework.

3GPP service layer consumes the API services from 3GPP network connection layer and also terminal side APIs services.

APIs from SEAL client can be provided by MNO or 3rd party e.g., UE vendor and offer certain SEAL layer services towards application. When these API applies, the SEAL client may further invoke APIs from SEAL service server to get Network services, or SEAL client may request network layer services (e.g., PDU session) via UE modem.