3GPP TSG-SA WG6 Meeting #70	S6-255239

Dallas, United States 17th – 21st November 2025	(revision of S6-255xxx)

Source:	Lenovo

Title:	Solution on energy-aware FL member availability change

Spec:	3GPP TR 23.700-44 v0.2.0

Agenda item:	9.9

Document for:	Approval

Contact:	Manos Pateromichelakis (epateromiche@lenovo.com)

1. Introduction

This contribution provides a new solution to KI 4.

2. Reason for Change

This contribution provides a new solution to KI 4.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.2.0.

* * * First Change * * * *

## 6.x	Solution #x: Support Energy-aware FL member availability update

### 6.x.1	Solution Description

This solution addresses Key Issue #4.

Stage 1 has introduced the notion of ‘energy credit’ as per TR 22.882 and TS 22.261, which is a quantity of credit associated with the subscriber (end user, ASP) that can be used for credit control by the 5G system. Energy credit can be associated to the ability for the network operator to create a 'maximum energy credit' policy, after which services are gated. Such energy credit or budget may be for a particular application service (end to end) or VAL UE.

This solution describes the event notification procedure for the FL member availability, since the FL member may be a VAL user who has a maximum energy credit for supporting FL operations. The notification can be triggered based on a change at the availability or capabilities of the candidate or selected FL members. The enhancement in this procedure (shown in bold italics) is the addition of the energy credit/budget status event notification to the subscribers. This will allow the pro-active modification of the FL member in case the credit is low, or the consumption / usage of energy is high at the AIMLE or VAL server or EAS acting as candidate FL member.

Pre-conditions:

1.	The subscription procedure in clause 8.5.2 of 3GPP TS 23.482 is performed by the AIMLE server or VAL server (via AIMLE server).

2.	The FL member has registered its capabilities (including energy credit / budget status) or availability to the ML repository based on the support for FL registration capability.

Figure 8.5.3-1: Procedure for FL-related event notifications

1.	The ML repository detects a change related to the availability of an FL member and generates events to be consumed by the AIMLE server, based on the notification, or based on a received updated registration from FL member, who has already registered at the ML repository.

The energy aware trigger event can be based on the registration update request message for the respective FL member(s) or by monitoring the energy status at the AIMLE or VAL / EAS server side. As part of the detection, the ML repository may request an energy monitoring function (e.g. EIF or other energy monitoring function at OAM or SEAL ESE server) to provide the energy credit status and limit / remaining value for the respective candidate FL member.

2.	For the generated event, the ML repository retrieves the list of corresponding subscriptions.

3.	The ML repository sends FL-related event notification to all entities (e.g., AIMLE servers) that have subscribed for the FL-related event matching the criteria. If a notification reception information is available as part of the FL-related event subscription, then the notification reception information is used by the ML repository to send event notifications to the corresponding AIMLE servers.

In case on energy-related event, the information about the event related to the FL member energy status change can include:

- the FL member credit status (remaining energy credit) or energy usage information (e.g. percentage or qualitative value e.g. high, medium) for the respective EAS or VAL server or AIMLE acting as FL member.

- the actual or predicted absolute energy consumption or efficiency metric (bits / Joule) if the credit is not provided.

- optionally, the energy contribution on FL process which is the foreseenable impact of the high energy usage to the end-to-end energy performance of the FL process.

4.	The notified subscriber (s) sends an event notification acknowledgement to the ML repository.

The definition of the FL related events is provided in Table 8.5.4-1 of TS 23.482 (new event in bold italics).

Table 8.5.4-1: Definition of FL related events

| Events | Events Description |
| --- | --- |
| Availability changes of FL member | The event type relates to the availability change of a FL member. Such availability change can be an indication of availability or unavailability for a given service area, or for a given ML model ID/profile. |
| FL model information | Information such as accuracy, time schedule and latency for the FL training. Latency is considering the target latency, i.e., when the FL model training shall be completed. |
| FL member load information | The event type relates to the monitoring of the computational load for the requested FL member. Such load can be the estimated (based on measurements) or expected/predicted based on the tasks that the FL member undertakes. This event may be triggered on-demand or can be monitored periodically. |
| FL member energy status information. | Information on the energy credit or budget or usage for the FL member. Such energy information can be the estimated (based on energy credit monitoring) or expected/predicted based on the tasks that the FL member undertakes. This event may be triggered on-demand or can be monitored periodically. |

### 6.x.2	Architecture impacts

This clause will provide the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 6.x.3	Solution evaluation

This clause will provide an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * Next Change * * * *