3GPP TSG-SA WG6 Meeting #68	S6-255240

Dallas , US, 17th – 21rd Nov 2025	(revision of S6-25xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.438 | CR | 0035 | rev | - | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for permission management of digital asset usage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_Ph2-APP |  |  |  |  |  | Date: |  |  | 2025-11-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The procedure of permission management is included in the Rel-19, but is not included in Rel-19 normative work. As per Rel-19 exception (i.e. SP-241689) sheet and Rel-20 WID proposal, it is necessary to finish the permission management in Rel-20. This contribution provides a proposal for permission management of digital asset usage. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR is proposed to add the feature on permission management of digital asset usage. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | permission management is not supported in this specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2, 8.2.3.1, 8.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

6.3.2	DA server

The following capabilities are supported by the DA server:

-	Supports the functionalities for digital asset profile create, update, retrieve and delete operations.

-	Supports the functionalities for digital asset discovery.

-	Supports the authorization and verification functionalities for digital asset usage permission.

NOTE:	The DA server as specified in this specification supports the Base Avatar Repository (BAR) (related to avatar communications) as specified in 3GPP TS 23.228 [5].

* * * Next Change * * * *

8.2.3.1	Digital asset profile create request

Table 8.2.3.1-1 describes the information elements for a digital asset profile create request sent from a requestor (e.g. DA client) to a DA server.

Table 8.2.3.1-1: Digital asset profile create request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| Digital asset profile parameters | M | Parameters to create digital asset profile |

Table 8.2.3.1-2: Parameters for digital asset profile

| Information element | Status | Description |
| --- | --- | --- |
| Digital asset owner identifier(s) | M | A list of identifier(s) for the digital asset owner. |
| Digital asset type | M | The type of digital asset, e.g. avatar, wallet, etc. |
| List of service provider identifiers | O | List of service providers for the digital asset. |
| access control list | O | List of tuples indicating the requestors allowed to perform operations with this digital asset and the allowed operation types. |
| > allowed user list | O | List of users allowed to access this digital asset |
| >> allowed operations | O | List of operations allowed by this user. Possible operations are (one or more of) : Discovery, Get, Update, Delete |
| >> list of predictive models | O | For each allowed user, list of one or more predictive model for the user, in order to predict user's behaviour. |
| spatial conditions | O | List of spatial conditions (e.g. locations) where this digital asset profile is allowed to be accessed |
| > current location | O | Current location where the profile is being used |
| allowed application list | O | List of application IDs which are allowed to use this digital asset profile |
| associated accessories identifiers | O | List of digital asset identifiers which are accessories (e.g. Hat, watch, shoes) purchased for this digital asset profile |
| expiry time | O | Time until this digital asset profile is valid |
| Sharing information | O | List of parameter for digital asset sharing. |
| > Sharing Mode | O | Whether the digital asset is shareable. If yes, authorization mode is necessary. |
| > Authorization Mode | O | Whether owner verification is mandatory for authorization processing. If no, share policies is necessary |
| > Share policies | O | Set of characteristics of share policy(e.g., Conditions for usage rights authorization, Validity period ).This parameter provides the criteria for the DA to evaluate request approval, operation types (Download, Update, etc), and validity period, etc. |

Editor's Note: The association of a subscriber/user identifier for the digital asset is FFS.

Editor's Note: Additional parameters (communication parameters, experience requirements) are FFS,

* * * Next Change * * * *

8.X	Usage Permission management of Digital Asset

8.X.1	General

The configuration of the owner's sharing information for digital assets is defined within the profile, with the relevant process detailed in clause 8.2.2.

Based on the owner's configuration, there are two procedures for consumer to obtain usage permission, one is authorization with owner confirmation, the other one is DA server completes authorization based on the share policies predefined by the owner. The following clauses specify procedures, information flows, and APIs for apply and obtain the usage permission of digital asset when digital assets are shared in virtual environments.

The consumer with usage permission may perform operations on the digital assets. The corresponding procedures are described in clause 8.4.2.2 and 8.4.2.3.

- X.2	Procedure

8.X.2.1 Obtain the usage permission with owner verification

Figure 8.x.2.1-1 illustrates procedure for obtain the permission for digital asset operation piror to the service. DA server may authorize the consumer to operate the digital asset based on the owner consent.

Pre-condition:

a.	 The parameter configuration for Authorization Mode is set to ‘owner verification is mandatory’.

Figure 8.x.2.1-1: Obtain the usage permission with owner verification

- Requestor(e.g. Consmer VAL Server or DA client) sends DA request for the digital asset usage permission. The request includes information listed in Table 8.X.3.1-1.

2.	DA checks whether owner’s verification is mandatory for this request. If needed, the DA searches the owner information of the digital asset.

3.	DA sends an Owner consent request to the owner (the owner VAL server or DA client for the owner) for the owner’s consent. The request includes information listed in Table 8.X.3.3-1.

4.	The VAL Server serves the owner of digital asset gets owner’s consent.

NOTE:	The interactions between the the owner of digital asset and the VAL Server serves the owner of digital asset are implementation specific and out of scope of the present document.

5.	Owner VAL server or DA client sends the Owner consent response to DA to indicate agree or deny of the  autherization. The response includes information listed in Table 8.X.3.4-1.

6.	DA update the access control list of  the digital asset profile based on the owner’s consent.

7.	DA sends response to the Consmer VAL server or DA client. The response includes information listed in Table 8.X.3.2-1.

8.X.2.2	Obtain the usage permission based on share policies

When the digital asset owner delegates authority to the DA server, authorizing it to grant digital asset usage permission. Users can obtain the usage permission without owner verification. Instead, authorization based on pre-configured policies set by the owner.

NOTE: In this release, the security aspect of digital asset owner delegating the authority to grant permissions is implementation specific.

Figure 8.x.2.2-1 illustrates the procedure for obtain the usage permission based on share policies.

Pre-condition:

a.	 The parameter configuration for Authorization Mode is set ‘No Owner Confirmation Required’, and the Share Policy has already been configured.

Figure 8.x.2.2-1: Procedure for obtain the usage permission based on share policies

1.	Requestor(e.g. Consmer VAL Server or DA client)  sends a Apply for Usage Permission Request to DA server. The request includes information listed in Table 8.X.3.1-1.

2.	DA server performs the necessary process of authentication and authorization , and authorizes the usage permission based on owner-defined share policies. If  the requestor fulfill the conditions, the DA server update the access control list.

3.	The DA server sends a Apply for Usage Permission Response to the requestor indicating success or failure of the operation. The response includes information listed in Table 8.X.3.2-1.

8.X.3	Information flows

8.X.3.1	Usage permission request

Table 8.x.3.1-1 describes the information elements for the Digital Asset Usage Permission Request sent from a Requestor to a DA server.

Table 8.x.3.1-1: Digital asset  usage permission request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Requested asset identifier | O | The identifier of the requested asset. |
| Apply operation | M | List of operations applied by this requestor. Possible operations are (one or more of) : Download, Update |
| Validity period | M | The duration of the requested usage permissions. |

8.X.3.2	Usage permission response

Table 8.x.3.2-1 describes the information elements for the Digital Asset Usage Permission Response sent from a DA server to Requestor.

Table 8.X.3.2-1: Digital asset usage permission response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | A status for the request: success, failure. . |
| Failure code | O | If the status is failure, a failure code may be provided.The code indicates the reason of the failure |
| Permission scope | O | Permissible operation Scope, include allowed operation types (Download, Update), granted permission time limit, etc. |

8.X.3.3	Owner consent request

Table 8.x.3.3-1 describes the information elements for the Owner consent Request sent from a DA server to a owner.

Table 8.x.3.3-1: Owner consent request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| Requested asset identifier | M | The identifier of the requested asset. |
| Apply operation | M | List of operations applied by this requestor. Possible operations are (one or more of) : Download, Update, etc. |
| Validity period | M | The duration of the requested usage permissions. |

8.X.3.4	Owner consent response

Table 8.x.3.2-1 describes the information elements for the Owner consent Response sent from a owner to a DA server.

Table 8.X.3.4-1: Owner consent response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | A status for the request: success, failure. . |
| Failure code | O | If the status is failure, a failure code may be provided.The code indicates the reason of the failure |

8.X.4	APIs

8.X.4.1	SS_DAUsgPermissionManagement API

8.X.4.1.1	General

Service description: This API is used for the application and assignment of the digital asset usage permission on the DA server.

8.X.4.1.2	Usage permission request operation

API operation name: SS_DAUsgPermissionManagement_request

Description: The consumer requests to obtain the usage permission of the digital asset.

Known Consumers: VAL server, DA client

Inputs: See Table 8.X.3.1-1.

Outputs: See Table 8.X.3.2-1.

See clause 8.X.2.1 and 8.X.2.2 for details of usage of this operation.

8.X.4.1.3	Obtain owner consent operation

API operation name: SS_DAUsgPermissionManagement_consent

Description: The DA requests to obtain the owner’s consent for the authorization.

Known Consumers: VAL server, DA client

Inputs: See Table 8.X.3.3-1.

Outputs: See Table 8.X.3.4-1.

See clause 8.X.2.1 for details of usage of this operation.

* * * End of Change * * * *