3GPP TSG-SA WG6 Meeting #70	S6-255249

Dallas, USA, 17th – 21st November 2025	(revision of S6-25xxxx)

Source:	China Mobile

Title:	New solution of updating QoS according to renewable energy to improve user experience in EDGEAPP

Spec:	3GPP 23.700-44 v0.2.0

Agenda item:	9.9

Document for:	Approval

Contact:	Liutangqing (liutangqing@chinamobile.com)

lvhuazhang (lvhuazhang@chinamobile.com)

1. Introduction

This pCR proposes new solution for FS_EnergySys_Ph2_APP.

2. Reason for Change

This paper focus on the updating QoS according to renewable energy.

In SA2 R19 Energy SID, that EIF is introduced to notify the UE or QoS level energy consumption information to AF. After receiving the notification from EIF, if the energy consumption information indicates the renewable energy information, the AF can trigger a QoS update procedure that improving the user experience. This is because considering the renewable energy can be recycled and regenerated in nature, if the UE or QoS flow has renewable energy in energy consumption, from operator's policy, the operator can improve the QoS to a certain extent.

What we discussion here is, the energy consumption of RAN or UPF serving the UE or QoS flow has renewable energy.

3. Conclusions

None.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.2.0.

* * * First Change * * * *

6.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI#5 | KI#6 |
| --- | --- | --- | --- | --- | --- | --- |
| Sol #1 | X |  |  |  |  |  |
| Sol #2 | X |  | X |  |  |  |
| Sol #3 |  |  | X |  |  |  |
| Sol #4 |  |  |  |  |  |  |
| Sol #5 | X |  |  | X |  |  |
| Sol #6 |  |  |  |  |  |  |
| Sol #7 |  |  |  |  |  |  |
| Sol #8 |  |  |  |  | X |  |
| Sol #9 |  |  |  |  |  | X |
| Sol #10 |  |  |  |  |  |  |
| Sol x |  |  | X |  |  |  |

* * * Second Change, all new text * * * *

6.x	Solution #x: Updating QoS according to renewable energy to improve user experience in EDGEAPP

6.x.1	Solution Description

This paper proposes the solution to solve the problem listed in KI#3, and proposes to improve the user experience if UE or QoS flow consumes renewable energy.

6.x.1.1	General

According to the section 5.51.2.2.1 of TS 23.501[3], the Energy Information Function (EIF) collects the UE related Energy Consumption information including Node-level energy consumption information, Node-level data volume from OAM and data volume of the required granularities (i.e. S-NSSAI, PDU Session and/or Service Data Flow) from UPF (via SMF). The EIF can expose the energy consumption at the requested granularity over a certain time window (i.e. the total energy consumed from a start time to an end time) or periodically (i.e. every reporting period of e.g. 10 minutes). And NEF/AF can subscribe the energy consumption.

Energy can be classified into two major types: renewable energy and non-renewable energy. Renewable energy sources include solar energy, hydropower, wind energy, biomass energy, wave energy, tidal energy, ocean thermal energy, geothermal energy, etc. They can be recycled and regenerated in nature. It is an inexhaustible and endless energy source that regenerates automatically without human intervention.

6.x.1.2	Solution description

In order to save the energy consumption and reduce the carbon emission, the operators may consider using renewable energy or green energy to replace traditional energy. For example, some servers of operators can be powered by electricity converted from renewable energy to reduce the carbon emission. Considering the renewable energy can be recycled and regenerated in nature, if the UE has renewable energy in energy consumption, from operator's policy, the operator can improve the QoS to a certain extent.

In Edge Computing deployment, as described in section 8.6.6.2.3 of TS 23.558[8], the EAS can trigger the session with QoS update request. And if in the notification from EIF that renewable energy is consumed by UE or QoS flow, the EAS can act as AF to trigger another session with QoS update to 5GC to improve the QoS parameters.

6.x.2	Architecture impacts

In this solution, it doesn’t change the overall architecture of enabling edge applications defined in section 6.2 of TS 23.558[x]. But the following enhancements are added:

-	The EAS can act as AF to subscribe the energy consumption information of certain UE or certain QoS from 5GC. And if renewable energy is consumed by UE or QoS flow, the EAS can act as AF to trigger another session with QoS update to 5GC to improve the QoS parameters.

6.x.3	Procedure of QoS update related to renewable energy consumption

The procedure listed here are based on the procedure defined in section 8.6.6.2.3 of 3GPP TS 23.558 [8].

8.6.6.2.3 Update a session Figure 8.6.6.2.3-1 illustrates the session with QoS update request operation between the EAS and the EES. It is used to request a modification of the QoS of the data session between AC and EAS. Figure 8.6.6.2.3-1: Session with QoS API: update operation 0. The EAS act as AF to subscribe and receive the energy consumption information from 5GC as described in clause 4.29 of TS 23.502[7]. And if there is the existence of renewable energy in the energy consumption information of target UE or QoS flow, the EAS may act as AF to improve QoS parameters to allow the UE to have higher user experience. 1. The EAS requests a modification of the QoS of the data session between the AC and the EAS, e.g. by including a different QoS reference or different bandwidth value. The EAS shall include the Context ID. The EAS may also request a modification of the event monitoring by subscribing to new events and updating or removing subscriptions to existing events. The EAS may consider to update the QoS according to the energy related information received in step 1. There are the examples below: - Increase the requested maximum bitrate or aggregate bit rates of both UL/DL to improve the data rate of target UE. - Adjust the Requested Priority to give higher priority to target UE to increase the pre-emption capability. - Adjust the Requested Packet Error Rate of target UE to further reduce the possibility of packet loss. 2. The EES checks if there is a context for the Context ID available and whether the EAS is authorized for this operation. If yes and the UE IP address is known by the EES, the EES invokes the Policy Authorization Update service or AF Session with QoS Update service with the 3GPP Core Network (PCF or NEF, respectively) as described in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3], providing the updated information to the PCF or NEF, respectively. The EES may subscribe to additional events or unsubscribe to certain events, if necessary. 3. If the operation is successful, the EES responds with a Context ID and a Result. If the EAS is not authorized or any other failure happens during the operation, the EES provides a rejection response with cause information. NOTE: The EES will report the resource allocation outcome, e.g. the successful modification of the Service Data Flow(s) related to the data session, with a separate session with QoS notify operation (see 3GPP TS 23.503 [12], clause 6.1.3.18).

6.x.4	Solution evaluation

Editor’s note: This part is for further update.

* * * End of Changes * * * *